3GPP TSG-RAN WG2 Meeting #133	R2-2601243

# Gothenburg, Sweden, 9th – 13th February, 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2156 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UL Rate Control MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Qualcomm, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2026-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | - The eLCIDs for UL Rate Control MAC CE identification are specified in Table 6.2.1-1b and Table 6.2.1-2b, not in Table 6.2.1-1 and Table 6.2.1-2. - The word “available” in “available bit rate recommendation” is not needed for better consistency in the spec text. - The index i of Fi field starts from zero, which means the maximum number of QoS flows can be max(i)+1 instead of max(i). Therefore, if following existing specs to only reserve Fi field “if the number of QoS flows configured to support UL rate control is less than i”, UE will miss reserving one QoS flow. - The reference for PDU Session ID/QoS Flow Identifier are not correct. - QoS flow list for UL rate control and UL rate query are configured separately in TS38.331(e.g. ul-RateControlConfigList-r19 and ul-RateQueryConfigList-r19), the number of QoS flows UE considered for setting UL rate control MAC CE should be the number of QoS flows configured in the direction where the UL Rate Control MAC CE is sent, which shall be clearly specified in the specs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the eLCIDs for UL Rate Control MAC CE identification “are specified in Table 6.2.1-1 and Table 6.2.1-2” to “are specified in Table 6.2.1-1b and Table 6.2.1-2b” - Delete “available” in “available bit rate recommendation” - For Fi field description, update the ' is less than i' to ‘is less than i+1’ - Update the references of PDU Session ID and QoS Flow Identifier - Update Fi field description to emphasize UE reserves Fi field when there are less QoS flows configured to support UL rate control in the direction UL Rate Control MAC CE is sent. Impact Analysis Impacted 5G architecture options: NR standalone, MR-DC Impacted functionality: UL Rate Control MAC CE Inter-operability: If the network is implemented according to the CR and the UE is not, or vice versa, the UE and network may have different understanding for the MAC CE structure, and the behaviour between UE and network may be inconsistent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec is unclear, the UE and network may have different understanding for the MAC CE structure, and the behaviour between UE and network may be inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.86 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Update the CR based on meeting agreements |  |  |  |  |  |  |  |  |

==============Start of change==============

6.1.3.86	UL Rate Control MAC CE

The UL Rate Control MAC CE is identified by a MAC subheader with an eLCID as specified in Table 6.2.1-1b and Table 6.2.1-2b for bit rate recommendation from the serving gNB and bit rate query from the UE, respectively.

The UL Rate Control MAC CE has a variable size and consists of the following fields:

-	Fi: This field indicates the presence of bit rate for the i:th QoS flow. The index i is the ascending order of the value defined by PDU Session ID × 64 + QoS Flow Identifier, where PDU Session ID (specified in clause 5.6.1 in TS 23.501 [32]) and QoS Flow Identifier (specified in clause 5.7.1 in TS 23.501 [32]) are those of the QoS flows configured to support UL rate control (as specified in clause 5.18.40) in the direction where the UL Rate Control MAC CE is sent. The Fi field set to 1 indicates that a bit rate for the i:th QoS flow is included in the MAC CE. The Fi field set to 0 indicates that no bit rate information for the i:th QoS flow is included. The Fi field is reserved and set to 0, if the number of QoS flows configured to support UL rate control in the direction where the UL Rate Control MAC CE is sent is less than i+1;

-	Bit Rate: When the serving gNB sends bit rate recommendation(s) in the UL Rate Control MAC CE, this field indicates a recommended bit rate for the QoS flow indicated by the Fi field. When the UE sends bit rate query(s) in the UL Rate Control MAC CE, this field indicates a preferred bit rate for the QoS flow indicated by the Fi field. Bit rates are included in the ascending order according to the Fi field. The MAC entity shall use the bit rates specified in Table 6.1.3.86-1 to set the value of this field. Each Bit Rate field is indicated in kbits/s and has a length of 8 bits.

Figure 6.1.3.86-1: UL Rate Control MAC CE

==============End of change==============