3GPP TSG-RAN WG3 Meeting #133	R2-2601261

# Gothenburg, Sweden, 9th - 13th Feb. 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0456 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PEI subgrouping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core, NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2026-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | In the current description for PEI subgrouping, the term “otherwise” in the sentence “Otherwise, the subgroup ID based on UE_ID based subgrouping as specified in clause 7.3.2 is used in the cell.” is intended to correspond to the condition “if available for the UE” in the preceding sentence “The subgroup ID based on CN assigned subgrouping as specified in clause 7.3.1, if available for the UE, is used in the cell;”, rather than to the multiple parallel “if” conditions listed earlier. Therefore, the current indentation of this sentence is incorrect. The indentation was correct in versions prior to v17.8.0, but starting from v17.9.0 it was mistakenly changed as an editorial modification, which led to this misinterpretation. Similar issue exists for LP-WUS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For both PEI and LP-WUS, decrease the indent for the clause starting with “Otherwise” for the UE subgroup assignment, and update the case for “Otherwise”. Impact Analysis Impacted 5G architecture options: NR SA and MR-DC Impacted functionality: PEI subgrouping, LP-WUS subgrouping. Inter-operability: There is no interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behaviour is not clear when to use UE ID based subgrouping for PEI and LP-WUS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.0, 7.6.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

## 7.3	Subgrouping for PEI

### 7.3.0	General

If PEI and subgrouping are configured, UEs monitoring the same PO can be divided into one or more subgroups. With subgrouping, the UE monitors the associated PO if the corresponding bit for subgroup the UE belongs to is indicated as 1 by PEI corresponding to its PO, as specified in clause 10.4a in TS 38.213 [4].

The following parameters are used for the determination of subgroup ID:

-	subgroupsNumPerPO: total number of subgroups for both CN assigned subgrouping (if any) and UE_ID based subgrouping (if any) in a PO, which is broadcasted in system information;

-	subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information.

UE's subgroup can be either assigned by CN as specified in clause 7.3.1 or formed based on UE_ID as specified in clause 7.3.2:

-	If subgroupsNumForUEID is absent in subgroupConfig, the subgroup ID based on CN assigned subgrouping as specified in clause 7.3.1, if available for the UE, is used in the cell.

-	If both subgroupsNumPerPO and subgroupsNumForUEID are configured, and subgroupsNumForUEID has the same value as subgroupsNumPerPO, the subgroup ID based on UE_ID based subgrouping as specified in clause 7.3.2 is used in the cell.

-	If both subgroupsNumPerPO and subgroupsNumForUEID are configured, and subgroupsNumForUEID < subgroupsNumPerPO:

-	The subgroup ID based on CN assigned subgrouping as specified in clause 7.3.1, if available for the UE, is used in the cell;

-	If the subgroup ID based on CN assigned subgrouping, as specified in clause 7.3.1, is not available for the UE, the subgroup ID based on UE_ID based subgrouping as specified in clause 7.3.2 is used in the cell.

If a UE has no CN assigned subgroup ID or does not support CN assigned subgrouping, and there is no configuration for subgroupsNumForUEID, the UE monitors the associated PO according to clause 7.1.

Next change

## 7.6	Subgrouping for LP-WUS

### 7.6.0	General

With LP-WUS monitoring, UEs monitoring the same PO can be divided into one or more subgroups.

The following parameters are used for the determination of subgroup ID for LP-WUS:

-	lp-SubgroupsNumPerPO: total number of subgroups for both CN assigned subgrouping (if any) and UE_ID based subgrouping (if any) for LP-WUS in a PO, which is broadcasted in system information;

-	lp-SubgroupsNumForUEID: number of subgroups for UE_ID based subgrouping for LP-WUS in a PO, which is broadcasted in system information.

UE's subgroup for LP-WUS can be either assigned by CN as specified in clause 7.6.1 or formed based on UE_ID as specified in clause 7.6.2:

-	If lp-SubgroupsNumForUEID is absent in lp-SubgroupConfig, the subgroup ID based on CN assigned subgrouping for LP-WUS as specified in clause 7.6.1, if available for the UE, is used in the cell.

-	If both lp-SubgroupsNumPerPO and lp-SubgroupsNumForUEID are configured, and lp-SubgroupsNumForUEID has the same value as lp-SubgroupsNumPerPO, the subgroup ID based on UE_ID based subgrouping for LP-WUS as specified in clause 7.6.2 is used in the cell.

-	If both lp-SubgroupsNumPerPO and lp-SubgroupsNumForUEID are configured, and lp-SubgroupsNumForUEID < lp-SubgroupsNumPerPO:

-	The subgroup ID based on CN assigned subgrouping for LP-WUS as specified in clause 7.6.1, if available for the UE, is used in the cell;

-	If the subgroup ID based on CN assigned subgrouping for LP-WUS as specified in clause 7.6.1, is not available for the UE, the subgroup ID based on UE_ID based subgrouping for LP-WUS as specified in clause 7.6.2 is used in the cell.

If a UE has no CN assigned subgroup ID for LP-WUS or does not support CN assigned subgrouping for LP-WUS, and there is no configuration for lp-SubgroupsNumForUEID, the UE monitors the associated PO according to clause 7.1 or monitors PEI as specified in clause 7.2.

End of change