3GPP TSG RAN WG2 Meeting #133              	                	                          R2-2601255

Title:	[Draft] Reply LS on UE 1Tx-1Tx switching period capability for NR inter-band UL CA and SUL band combinations

Response to:	R2-2600023 (R4-2522413)

Release:	Release 16

Work Item:	NR_RF_FR1

Source:	Xiaomi (to be RAN2)

To:	RAN, RAN4

Cc:	RAN1

Contact person:	Ziyi Li

liziyi5@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R2-2601251/R2-2601252/R2-2601253/R2-2601254 (endorsed CR)

# Overall description

RAN2 thanks RAN4 for the LS on UE 1Tx-1Tx switching period capability for NR inter-band UL CA and SUL band combinations. RAN2 discussed the signaling design and how to support this feature as early implementation from Rel-16.

Based on consensus, RAN2 agrees to reuse uplinkTxSwitchingPeriod-r16 of ULTxSwitchingBandPair-r16 to indicate the support of non-zero switching period for 1Tx-1Tx switching period capability for NR inter-band UL CA and SUL band combinations.

The endorsed CR (with RAN2 consensus) can be found in the attachment R2-2601251/R2-2601252/R2-2601253/R2-2601254.

# Action

To RAN and RAN4: RAN2 respectfully asks RAN and RAN4 to take above replies into account in the final decision.

# Dates of the next TSG RAN WG2 meetings

TSG RAN2 Meeting #133bis	13th – 17th Apr, 2026	Malta, MT

TSG RAN2 Meeting #134		18th – 22nd May, 2026	China

# Annex – RAN2 agreements

RAN2 has consensus on below agreements:

  - RAN2 agrees to indicate the support of switching period of 1Tx-1Tx UL Tx switching for NR inter-band UL CA and SUL band combinations by reusing uplinkTxSwitchingPeriod-r16 of ULTxSwitchingBandPair-r16 since Rel-16.

  - 1T-2T NUL+SUL band UE may not report legacy BC in Rel-15 capability. It is up to network implementation how to handle such UE if it performs initial access on SUL band.

  - 1T-1T NUL+SUL UL Tx switching UE has the same behavior as 1T-2T NUL+SUL band UE during initial access.

  - UE supporting Rel-16 1T-1T inter-band UL CA/NUL+SUL only with non-zero UL switching time will only report BC UL Tx switching via Rel-16 capability, and will not report legacy BC in Rel-15 capability. This will be captured in TS 38.306, therefore no RAN1 impact from RAN2 perspective.

  - If the UE reports the ULTxSwitchingBandPair-r16 and ULTxSwitchingBandPair-r18 simultanesouly, a UE supporting UL 1Tx-1Tx switching shall also indicate the support of UL 1Tx-1Tx switching in ULTxSwitchingBandPair-r16 for the corresponding band pair. It is up to network either use Rel-16/18 capability.