3GPP TSG-RAN Meeting #133	R2-260xxxx

# Gothenburg, Sweden, 9 – 13 Feb, 2025

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1409 | rev | - | Current version: | 17.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on 1Tx-1Tx switching period for NR inter-band UL CA and SUL band combinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, Spreadtrum, UNISOC, Oppo, Vivo, Huawei, HiSilicon, China Telecom, Ericsson, MediaTek Inc., China Unicom, CMCC, CATT, ZTE Corporation, Sanechips, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1 |  |  |  |  |  | Date: |  |  | 2026/02/10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | In R4-2522413, RAN4 has agreed the RF requirements to allow 1Tx UE to support non-zero 1Tx-1Tx switching for NR inter-band UL CA and SUL band combinations since Rel-16 as optional feature with the following switching period: - The switching period values are: {35us, 140us, 210us}. However, according to RAN2 specification, the field description of Rel-16 UL Tx switching is limited to 1Tx-2Tx switching case, i.e., it requires UE to indicate support for 2-layer UL MIMO capabilities on one of the indicated two bands in each FeatureSet entry supporting UL 1Tx-2Tx switching, and only the band where UE supports 2-layer UL MIMO capability can work as carrier2. I.e., UE cannot report the support of non-zero 1Tx-1Tx switching for NR inter-band UL CA and SUL band combinations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To align with RAN4 specificiation, extend support in RAN2 specification how for UE to report 1Tx-1Tx switching capability based on reuse of existing Rel-16 Tx switching capability. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: UL Tx switching, inter-band UL CA, SUL Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the network can know that the UE only supports 1T-2T switching based on the UL MIMO layer capability, hence no inter-operability issues are foreseen. 2. If the UE is implemented according to the CR and the network is not, the network may not comprehend the Rel-16 1T-1T switching band combination if UE indicates support for 1-layer UL MIMO capabilities on the indicated two bands in each FeatureSet entry supporting UL 1Tx-1Tx switching. It is up to network implementation how to handle such scenario; however, network will not configure 1T-2T switching considering the UE does not support 2-layer UL MIMO as indicated via UL MIMO layer capability. Furthermore, it is up to network implementation how to handle configurations of 1T-1T switching during handover. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Switching period of 1Tx-1Tx for NR inter-band UL CA and SUL band combinations cannot be supported by Rel-16 1Tx UE, and thus misalignment betweeen RAN2 and RAN4 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 4.2.7.1	BandCombinationList parameters

<omit the unrelated part>

| supportedBandCombListPerBC-SL-RelayDiscovery-r17, supportedBandCombListPerBC-SL-NonRelayDiscovery-r17 Indicates, for a particular Uu band combination, the PC5 Relay discovery and non-Relay discovery band combination(s) on which the UE supports simultaneous transmission/reception of PC5 data (Relay discovery or non-Relay discovery) and Uu uplink/downlink respectively. The leading / leftmost bit (bit 0) corresponds to the first band combination included in supportedBandCombinationListSL-RelayDiscovery-r17/supportedBandCombinationListSL-NonRelayDiscovery-r17, the next bit corresponds to the second band combination included in supportedBandCombinationListSL-RelayDiscovery-r17/supportedBandCombinationListSL-NonRelayDiscovery-r17 and so on. with value 1 indicating simultaneous transmission/reception is supported. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| ULTxSwitchingBandPair-r16, ULTxSwitchingBandPair-v1700 Indicates UE supports dynamic UL 1Tx-2Tx switching in case of inter-band CA, SUL, and (NG)EN-DC, UL 1Tx-1Tx switching and UL 2Tx-2Tx switching in case of inter-band CA and SUL as defined in TS 38.214 [12], TS 38.101-1 [2] and TS 38.101-3 [4]. The capability signalling comprises of the following parameters: - bandIndexUL1-r16 and bandIndexUL2-r16 indicate the band pair on which UE supports dynamic UL Tx switching. bandindexUL1/bandindexUL2 xx refers to the xxth band entry in the band combination. UE shall indicate support for 2-layer UL MIMO capabilities on one of the indicated two bands in each FeatureSet entry supporting UL 1Tx-2Tx switching and indicate support for 2-layer UL MIMO capabilities on both bands in each FeatureSet entry supporting UL 2T-2Tx switching, and only the band where UE supports 2-layer UL MIMO capability can work as carrier2 as defined in TS 38.101-1 [2] and TS 38.101-3 [4]. UE shall indicate support for 1-layer UL MIMO capabilities on the indicated two bands in each FeatureSet entry supporting UL 1Tx-1Tx switching, and either of the two bands can work as carrier1 or carrier2. A UE reporting UL 1Tx-1Tx switching shall not report the same band combination in supportedBandCombinationList. - uplinkTxSwitchingPeriod-r16 indicates the length of UL Tx switching period of 1Tx-1Tx switching or 1Tx-2Tx switching per pair of UL bands per band combination when dynamic UL Tx switching is configured, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. UE shall not report the value n210us for EN-DC band combinations. n35us represents 35 µs, n140µs represents 140us, and so on, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. - uplinkTxSwitchingPeriod2T2T-r17 indicates the length of UL Tx switching period of 2Tx-2Tx switching per pair of UL bands per band combination when dynamic UL Tx switching is configured, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. n35us represents 35 µs, n140us represents 140µs, and so on, as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. - uplinkTxSwitching-DL-Interruption-r16 indicates that DL interruption on the band will occur during UL Tx switching, as specified in TS 38.133 [5] and in TS 36.133 [27]. UE is not allowed to set this field for the band combination of SUL band+TDD band, for which no DL interruption is allowed. Field encoded as a bit map, where bit N is set to "1" if DL interruption on band N will occur during uplink Tx switching as specified in TS 38.133 [5] and in TS 36.133 [27]. The leading / leftmost bit (bit 0) corresponds to the first band of this band combination, the next bit corresponds to the second band of this band combination and so on. The capability is not applicable to the following band combinations, in which DL reception interruption is not allowed: - TDD+TDD CA with the same UL-DL pattern - TDD+TDD EN-DC with the same UL-DL pattern | BC | FD | N/A | FR1 only |
| uplinkTxSwitching-OptionSupport-r16 Indicates which option is supported for dynamic UL 1Tx-1Tx or UL 1Tx-2Tx switching for inter-band UL CA and (NG)EN-DC. switchedUL represents option 1 as specified in TS 38.214 [12], dualUL represents option 2 as specified in TS 38.214 [12], both represents both option 1 and option2 as specified in TS 38.214 [12]. UE shall not report the value both for (NG)EN-DC case. UE shall not report the value dualUL or both for UL 1Tx-1Tx switching. The field is mandatory for inter-band UL CA and (NG)EN-DC case where UE supports dynamic UL 1Tx-1Tx or UL 1Tx-2Tx switching. | BC | CY | N/A | FR1 only |

END OF CHANGE