3GPP TSG-RAN WG2 Meeting #133	R2-260xxxx

Gothenburg, Sweden, 9 - 13 February 2026

Agenda Item:	10.4

Source:	Huawei, HiSilicon

Title:	Shortcomings of early DL/UL synch, and early CSI acquisition

Document for:  Discussion and Decision

# 1	Introduction

This document proposed how to capture shortcomings of early DL/UL sync and early CSI acquisition in 5G..

# 2	Discussion

Shortcomings of early DL sync:

-	complex UE/network signalling (pre-configuration of TCI states)

-	UE power consumption (for beam measurement and reporting)

-	RS overhead (when using TRS)

-	unnecessary early DL sync (if the UE does not go to the cell)

Shortcomings of early UL sync using RA:

-	complex UE/network signalling (RA to neighbour cell)

-	UE power consumption and RA resource usage (e.g., CFRA)

-	interruption for inter-frequency

-	unnecessary early UL sync (if the UE does not go to the cell)

Shortcoming of early UL sync using UE-determined TA:

-	only works when source and target are synchronized

Shortcoming of early CSI acquisition:

-	UE power consumption (for beam measurement)

-	RS overhead (CSI-RS)

-	interruption for inter-frequency

-	unnecessary early CSI acquisition (if the UE does not go to the cell)

-	only for DL throughput for FDD (not UL