3GPP TSG-RAN3 Meeting #130	R3- 258111

# Dallas, USA, 17-21, November 2025

Agenda Item:	9.2.14 - Other

Source:	Comtech Telecommunications, Polaris Wireless, FirstNet, Verizon

Title:	Discussion on addition of NR Timing Advance Type 1 in NR UL E-CID

Document for:	Discussion, Decision

- Introduction

RAN 3 added support of NR Timing Advance Type 1 in 38.455 NRPPa Release 18.1.0 as shown below.

| 2024-03 | RAN#103 | RP-240646 | 0124 | 3 | F | Introduction of NR UE Rx-Tx time difference measurement in NR UL E-CID | 18.1.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

RP-240646 added in 38.455 NRPPa 18.1.0  allows the gNB to pass gNB Rx–Tx Time Difference  and UE Rx–Tx Time Difference  to the LMF so that the LMF can calculate TADV Type 1 as follows:

NR TADV Type 1 = (UE Rx–Tx Time Difference ) + (gNB Rx–Tx Time Difference ).

Currently gNB Rx–Tx Time Difference  that  is used to calcuate NR TADV Type 1  shares the same “Timing Advance NR” field” that is used for PRACH-based TADV Type 2 in the E-CID measurement result. This field has a value range of INTEGER (0..7690).  While this range is sufficient for PRACH-based TADV Type-2, it does not accommodate the gNB Rx–Tx Time Difference  required for TADV Type 1, which may be negative.

| >>Value Timing Advance NR |  |  | INTEGER (0.. 7690) | Report mapping as defined in TS 38.133 [16] | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

This paper discusses the solution to the NR TADV Type 1  value range problem.

# 2	Discussion

The root cause of the NR TADV Type 1 value range problem is having the gNB Rx–Tx Time Difference  that  is used to calcuate NR TADV Type 1  share the same “Timing Advance NR” field” that was originally designated for PRACH-based TADV Type 2 value in the E-CID measurement result. This in turn leads to gNB Rx–Tx Time Difference  in NR TADV Type 1  sharing the same definition for PRACH based TADV Type 2 in 38.215.

PRACH based TADV Type 2 is defined in 38.215 as shown below:

“5.2.7 Timing advance (TADV)

| Definition | Timing advance (TADV) is defined as the time difference TADV = (TgNB-RX – TgNB-TX), Where: TgNB-RX is the Transmission and Reception Point (TRP) [18] received timing of uplink subframe #i containing PRACH transmitted from UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. The detected PRACH is used to determine the start of one subframe containing that PRACH. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104 [9]: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104 [9]: the Tx Transceiver Array Boundary connector. |
| --- | --- |

“

NR TADV Type 1 as shown in Figure 1 below is basically serving cell RTT. gNB Rx-Tx Time Difference and UE Rx-Tx Time Difference used to calculate TADV Type 1 may be based on timing measuremnts from downlink TRS/PRS and uplink SRS or other downlink and uplink signals, which is dfferent from the gNB Rx-Tx Time Difference in NR TADV Type 2 that is based on PRACH as shown in Figure 2 below. .

Figure 1: NR Tadv Type 1

UE

gNB

gNB Rx-Tx

Time Difference

(negative)

UE Rx-Tx

Time Difference

UE

gNB

gNB Rx-Tx

Time Difference

(positive)

UE Rx-Tx

Time Difference

NR Tadv Type 1 =  UE Rx-Tx Time Difference + gNB Rx-Tx Time Difference

UE

gNB

gNB Rx-Tx

Time Difference

PRACH

NR Tadv Type 2 =  gNB Rx-Tx Time Difference

Figure 2: Tadv Type 2

Therefore, it is incorrect for the gNB Rx–Tx Time Difference  in NR TADV Type 1 to share the same definition with the gNB Rx-Tx Time Difference in NR TADV Type 2, i.e. the TADV in the table above quoted from 38.215.

### Proposal 1: Update Timing Advance (TADV) defintion in 38.215 to include both Type 1 and Type 2 Timing Advance as shown below. See reference [1] for correponding RAN1 CR to TS 38.215.

5.2.7 Timing advance (TADV)

| Definition | Type1: Timing advance (TADV) type 1 is defined as the time difference TADV = (gNB Rx – Tx time difference) + (UE Rx – Tx time difference), where the gNB Rx – Tx time difference corresponds to the same UE that reports the UE Rx – Tx time difference. The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the Transmission and Reception Point (TRP) [18] received timing of uplink subframe #i, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104 [9]: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104 [9]: the Tx Transceiver Array Boundary connector. The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE. Type2: Timing advance (TADV) type 2 is defined as the time difference TADV = (TgNB-RX – TgNB-TX), Where: TgNB-RX is the Transmission and Reception Point (TRP) [18] received timing of uplink subframe #i containing PRACH transmitted from UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. The detected PRACH is used to determine the start of one subframe containing that PRACH. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104 [9]: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104 [9]: the Tx Transceiver Array Boundary connector. |
| --- | --- |

### Proposal 2: Update TS 38.455: add TADV type 1 value as a new field in NR ECID measurement result. See reference [2] for correponding RAN3 CR to TS 38.455.

#### 9.1.1.1 E-CID MEASUREMENT INITIATION REQUEST

This message is sent by LMF to initiate E-CID measurements.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15 ,…, 16..256) |  | YES | reject |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic,…) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 60min applies only for ng-eNB. The codepoint “extended” is not applicable. This IE is not applicable to NR Angle of Arrival. | YES | reject |
| Measurement Quantities |  | 1 |  |  | EACH | reject |
| >Measurement Quantities Item |  | 1..<maxnoMeas> |  |  | - |  |
| >>Measurement Quantities Value | M |  | ENUMERATED (Cell-ID, Angle of Arrival, Timing Advance Type 1, Timing Advance Type 2, RSRP, RSRQ,…, SS-RSRP, SS-RSRQ, CSI-RSRP, CSI-RSRQ, NR Angle of Arrival, NR Timing Advance, UE Rx-Tx Time Difference, NR Timing Advance Type 1) |  | - | - |
| … | … | … | … | … | … | … |

#### 9.2.5	E-CID Measurement Result

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving Cell ID | M |  | NG-RAN CGI 9.2.6 | NG-RAN Cell Identifier of the serving cell | - |  |
| Serving Cell TAC | M |  | TAC 9.2.11 | Tracking Area Code of the serving cell | - |  |
| … | … | … | … | … | .. | … |
| >>Value Timing Advance NR |  |  | INTEGER (0.. 7690) | Report mapping as defined in TS 38.133 [16] | YES | ignore |
| >>Value UE Rx-Tx Time Difference |  |  | INTEGER (0.. 61565) | As defined in TS 38.215 [19]. This IE is ignored when the Value Timing Advance NR Type 1 IE is included. | YES | ignore |
| >>Value Timing Advance NR Type 1 |  |  | INTEGER (0.. 7690) | Report mapping as defined in TS 38.133 [16] | YES | ignore |
| Geographical Coordinates | O |  | 9.2.46 |  | YES | ignore |
| … | .. | .. | .. | .. | .. | .. |

### Proposal 3: Clarify the Tadv measurement report mapping for type 1 and type 2 timing adavance. See reference [3 ] for correponding RAN4 CR to TS 38.133.

## Timing advance (TADV)

### 13.5.1	Report mapping

The reporting range of TADV, as defined in clause 5.2.7 of TS 38.215 [4], is defined from 0 to 3150848 Tc with 128 Tc resolution for timing advance less than 262144 Tc, and 512 Tc for timing advance greater than or equal to 262144 Tc.

Tc is defined in TS 38.211 [6].

The mapping of measured quantity is defined in table 13.5.1-1.

Table 13.5.1-1: TADV measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| TIME_ADVANCE_00 | TADV 128 | Tc |
| TIME_ADVANCE_01 | 128 TADV < 256 | Tc |
| TIME_ADVANCE_02 | 256 TADV < 384 | Tc |
| … | … | … |
| TIME_ADVANCE_2046 | 261888 TADV < 262016 | Tc |
| TIME_ADVANCE_2047 | 262016 TADV < 262144 | Tc |
| TIME_ADVANCE_2048 | 262144 TADV < 262656 | Tc |
| TIME_ADVANCE_2049 | 262656 TADV < 263168 | Tc |
| … | … | … |
| TIME_ADVANCE_7688 | 3149824 TADV < 3150336 | Tc |
| TIME_ADVANCE_7689 | 3150336 TADV < 3150848 | Tc |
| TIME_ADVANCE_7690 | 3150848 TADV | Tc |

NOTE:	For report mapping, type 2 TADV is equal to (gNB Rx – Tx time difference) + NTA_offset, where NTA_offset is based on the information n-TimingAdvanceOffset as specified in TS 38.331 [2].

# 3	Conclusions

Agree to 38.455 NRPPa CR Ref [1] and endorese the RAN1 TS 38.215 CR Ref [2] and RAN4 TS 38.133 CR Ref [3].

# 4	References

- R1-xxxxxx draft CR to TS 38.215, Update Tadv defition to include both Type 1 and Type 2 Timing Advance

- R3-258112 CR to TS 38.455, Add Tadv Type 1 measurement in NR UL E-CID

- R4-xxxxxx draft CR to TS 38.133, Clarify Tadv measurement report mapping for Type 1 and Type 2 Timing Advance