3GPP TSG-RAN WG3 Meeting #130	R3-258148

Dallas, US, November 17 - 21, 2025

Agenda Item:	13.3

Source:	ZTE Corporation

Title:	(TP to pCR 38.765) Further discussion on RAN-CN Procedures and Signalling for ISAC

Document for:	Discussion and Approval

- Introduction

In last RAN3 meeting, the RAN-CN procedures and signalling for ISAC in R20 have been discussed. The Sensing Report procedure was introduced in [1]. And the to be continued issue is given as below[2].

To continue the discussion on the necessary interface functions and the detailed signaling procedures for ISAC.

In this contribution, we provide our further consideration on RAN-CN procedures an signalling for ISAC in R20.

- Discussion

- 2.1	Basic RAN-CN procedures

In last meeting, the Sensing Reporting procedure was introduced as below.

8.x Sensing Reporting Editor’s Note: the following may need further refinement. Figure 8.x-1: Message flow for sensing reporting 1. The SF sends sensing request to the gNB. 2. The gNB sends sensing response to the SF. 3. If requested in Step 1, the gNB sends sensing report to the SF.

- For the working mechanism of the Sensing Reporting procedure, some companies thought it should be similar as the Measurement procedure and the Measurement Reporting procedure for Positioning. While, we think it is more appropriate to follow the way of Resource Status Reporting Initiation procedure and the Resource Status Reporting procedure for the Mobility Load Balancing in SON/MDT.

Both of the two mentioned mechanism support the periodic reporting and the on-demand reporting, while the difference is that the MEASUREMENT RESPONSE message contains the TRP measurement Result, and the RESOURCE STATUS RESPONSE message only indicates the measurement was successfully initiated. Considering the intention is to use the SENSING REPORT message to send the sensing data from the gNB to the SF, it is reasonable to follow the working mechanism of Mobility Load Balancing to separate the whole procedure into two procedures, i.e., Sensing  Reporting Initiation procedure and Sensing Reporting procedure, given as below.

Figure 1: Sensing Reporting Initiation procedure

Step 1. In the SENSING REQUEST message, the SF provides the sensing configuration parameters to the gNB, including the sensing area and potential information of sensing target, such as UAV speed range and UAV size.

Step 2. In the SENSING RESPONSE message, the gNB indicates the SF that the requested sensing measurement is successfully initiated.

Figure 2: Sensing Reporting procedure

- Step 1. Following a successful Sensing Reporting Initiation procedure, the gNB reports the sensing result with association information to the SF via the SENSING REPORT message.

Proposal 1: The existing Sensing Reporting procedure should be divided into Sensing Initiation procedure and Sensing Reporting procedure, following the mechanism of Mobility Load Balancing.

In addition to the Sensing Reporting Initiation procedure and Sensing Reporting procedure, a procedure triggered by the SF to cancel the on-going sensing measurement is needed. And the Sensing Abort procedure should be introduced to enable the SF to abort an on-going sensing measurement, which is given as below.

Figure 3: Sensing Abort procedure

Step 1. The SF indicates the gNB to abort an on-going sensing measurement via the SENSING ABORT message.

Proposal 2: Agree to introduce the Sensing Abort procedure to enable the SF to abort an on-going sensing measurement.

Furthermore, to provide the feasibility of the gNB to notify the SF that previously request measurement cannot be reported,  the Sensing Failure Indication procedure can be introduced , which is given as below.

Figure 4: Sensing Failure Indication procedure

Step 1. The gNB notifies the SF that the measurements previously requested with the Sensing Initiation procedure can no longer be reported.

Proposal 3: Agree to introduce the Sensing Failure Indication procedure to enable the gNB to notify the SF that the measurement previously requested cannot be reported.

- 2.2	Sensing measurements

Basically, we think the measurement metrics can be divided into six levels as RAN1 discussed.

Level 1（Raw data）

Level 2（Delay, Doppler, angle）

Level 3（Delay, Doppler, angle per path）

Level 4（Range, velocity, angle, power per point）

Level 5  (Position, velocity per object, w/o target ID)

Level 6  (Position, velocity per object, with target ID)

Regarding the study of Level 1, Level 2 and Level 3 measurement metrics which are purely physical layer measurements, it should be pending to the discussion and decision in RAN1.

Proposal 4: For Level 1, Level 2 and Level 3 measurements, the details and decision is up to RAN1 conclusion.

While, for the study of Level 4, 5 and Level 6 measurement metrics, the details could be further discussed in RAN3 and coordinated with RAN1 if needed.

- Given the massive volume of sensing results received at the gNB, how to determine the association between the sensing results and the corresponding UAV(s) needs to be further discussed especially for UAV tracking.

As shown in the below figure, the points in different colors represent the sensing data of different UAVs, and it needs to discuss how to report to SF to differentiate these sending data.

Figure 5: Multiple sensing data examples

- In reality, most UAV detection needs to involve tracking, since the trajectories of different UAVs may interact, how to trace the trajectory of a specific UAV without any confusion in SF needs to be further discussed.

Figure 6: Trajectories of different UAVs

- In addition to the sensing results including the channel parameters, whether the gNB can send some other assistance information to help the Sensing Function make the final sensing decision needs to be further discussed.

- Considering the selection on the different levels of sensing measurements, the transmitting capability of the Nx interface between gNB and SF is one key factor. Taking Xn interface as example, the maximum number of cells that served by a NG-RAN node is 16384, the transmission of the sensing measurements with raw level (e.g., Level1 and level2) has the risk of beyond the transmitting capability of the interface. Although there is no clear restriction on the Xn, the message size limitation of DTLS over SCTP is 214 bit[3], which could be regarded as the same consideration for RAN-CN interface.

‘DTLS limits the DTLS user message size to the current Path MTU minus the header sizes. For the purposes of running over SCTP, the DTLS path MTU MUST be considered to be 2^14.’

Therefore, the data size of sensing measurement should be evaluated to avoid the data transmission beyond the interface capability.

Proposal 5: RAN3 should further study the following issues:

-How to determine the association between the sensing results and the UAV?

-How to report the trajectory of UAV to Sensing Function without any confusion?

-Is there any other assistance information needed from gNB to Sensing Function?

-The data size of sensing measurements?

3	Summary

Basic RAN-CN procedures

Proposal 1: The existing Sensing Reporting procedure should be divided into Sensing Initiation procedure and Sensing Reporting procedure, following the mechanism of Mobility Load Balancing.

Proposal 2: Agree to introduce the Sensing Abort procedure to enable the SF to abort an on-going sensing measurement.

Proposal 3: Agree to introduce the Sensing Failure Indication procedure to enable the gNB to notify the SF that the measurement previously requested cannot be reported.

Sensing measurements

Proposal 4: For Level 1, Level 2 and Level 3 measurements, the details and decision is up to RAN1 conclusion.

Proposal 5: RAN3 should further study the following issues:

-How to determine the association between the sensing results and the UAV?

-How to report the trajectory of UAV to Sensing Function without any confusion?

-Is there any other assistance information needed from gNB to Sensing Function?

-The data size of sensing measurements?

4	Reference

- [1] R3-257332, (pCR to TR 38.765) Study on Integrated Sensing And Communication (ISAC) for NR, China Telecom, Xiaomi

- [2] RAN3#129bis Chairman Notes

[3] R3-206637, Discussion on the size limitation of DTLS over SCTP, Huawei, China Telecom

Annex		Text Proposal to TR 38.765

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

8.x1	Sensing Reporting Initiation

Editor’s Note: the following may need further refinement.

Figure 8.x-1: Message flow for sensing reporting initiation

1.	The SF sends sensing request to the gNB with the sensing configuration parameters.

2.	The gNB sends sensing response to the SF to indicate that the requested sensing measurement is successfully initiated.

8.x2	Sensing Reporting

Editor’s Note: the following may need further refinement.

Figure 8.x-2: Message flow for sensing reporting

1.	Following a successful Sensing Reporting Initiation procedure, the gNB sends sensing report to the SF with sensing result and association information.

8.x3	Sensing Abort

Editor’s Note: the following may need further refinement.

Figure 8.x-3: Message flow for sensing abort

1.	The SF sends sensing abort to the gNB to abort an on-going sensing measurement .

8.x4	Sensing Failure Indication

Editor’s Note: the following may need further refinement.

Figure 8.x-4: Message flow for sensing failure indication

1.	The gNB sends sensing failure indication to the SF to notify that the measurements previously requested with the Sensing Initiation procedure can no longer be reported.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>