3GPP TSG-RAN WG3 Meeting #130                                                                   R3-2508311

Dallas, US, 17 - 21, November 2025

Source:	CATT

Title:	Discussion on Topology-2 for Ambient IoT

Agenda Item:	14.2

Document for:	Discussion

1.	Introduction

In the last meeting, we started the Topology-2 for A-IoT with the following progress,

- General Arch For Rel-20 T2 discussion, based on the RRC-based solution, and RAN3 focus is indirect connectivity is applied. NGAP procedures specified for Topology 1 are base line for Topology 2. - NGAP signaling procedures (Inventory and Command) The AIOT procedures (Topology 2, RRC based solution) in TR38.769 can be as base line. - UE Reader Authorization UE Reader Authorization is performed at 5GC The UE Reader authorization status (authorized, non-authorized) is provided from the 5GC to the gNB. FFS from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU

This contribution further discusses the open issues of Ambient IoT topology-2.

Section 2 gives our detailed analysis on following aspects:

- UE reader authorization

- UE reader selection

- NGAP signaling procedures

- UE Reader Mobility

Section 3 summarizes the contribution of this contribution.

2.	Discussion

## 2.1	UE reader authorization

Regarding to UE reader authorization, so far SA2 has achieved the following interim conclusions, which were captured in SA2 TR 23.700-30-020 [1], as shown in highlight part.

SA2 TR 23.700-30-020 7 Interim Conclusion for authorization UE reader authorization and revocation part: - Subscription aspects: - The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following: - information indicating whether the UE is allowed to operate as a UE Reader. Editor’s note: Whether and what additional subscription information for the UE Reader is needed is FFS. - UE Reader subscription information is available to AMF. - If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, AMF informs NG-RAN that the UE is authorized to operate as a UE Reader. Editor’s note: Whether to inform the UE about the authorization information is FFS. Editor’s note: Whether UE reader capability is provided to AMF is FFS. - If the subscription information has been revoked, the indication that the UE is not authorized to operate as a UE Reader from the UDM, then the AMF informs NG-RAN that the UE is not authorized to operate as a UE Reader. Editor’s note: Whether and how UE Reader subscription information is provided to AIOTF is FFS. - Radio resource management for UE Reader operation: - If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

In RAN3#129-bis meeting, RAN3 also achieved the following agreements regarding UE reader authorization

UE Reader Authorization is performed at 5GC The UE Reader authorization status (authorized, non-authorized) is provided from the 5GC to the gNB.

From SA2 interim conclusions, we can see that the AMF determines whether the UE is authorized to work as reader. And the subscription data for a UE indicating the allowance for reader operation is taken into account by the AMF when authorizing the UE as AIoT Reader. Except the subscription data, other factors, such as the time and UE’s location, may also be considered. Nevertheless, before the AMF performs authorization based on the subscription data and other factors, the AMF should firstly know whether the UE would like to operate as a reader (and has the relevant capability). In other words, the AMF should only perform authorization to the UE which requests to operate as an AIoT reader. An indication to AMF for the UE’s request on AIoT reader operation can be sent to AMF. The indication can be potentially sent via UE reader’s NAS or via RRC and NGAP, which is determined by SA2.

NGAP impact

Per the objective of the WI [3], RAN2 specify the RRC CONNCTED UE reader only, we can conclude that only the RRC CONNECTED UEs can operate as AmIoT reader. Therefore, the AMF can perform authorization each time when the UE which request to operate as reader enters RRC CONNECTED mode. And the AMF should send the authorization result to the gNB, so that the gNB can decide whether to select the UE reader to perform the AIoT inventory/command service based on the the UE reader authorization status.

To achieve that, when the UE requesting the reader operation enters RRC CONNECTED mode, AMF can carry out authorization to the UE reader based on the subscription data, after which the AMF can send the UE reader authorization result (i.e., authorized or not authorized) in INITIAL CONTEXT SETUP REQUEST message sent to the gNB serving the UE reader.

Proposal 1-1:	AMF provides the UE reader authorization result to the gNB via INITIAL CONTEXT SETUP REQUEST after authorization to the UE.

In addition, the AMF can change the authorization status (e.g., from authorized to non-authorized, or from non-authorized to authorized) during the UE reader’s RRC CONNECTED mode, e.g., when the area where the UE reader resides is changed. And each time the AMF changes the authorization status of the UE reader, it can provide the latest status to the UE’s serving gNB via UE CONTEXT MODIFICATION REQUEST.

Also, when the UE performs Xn/NG handover or re-establishes RRC after RLF to another gNB, the authorization status of UE may be changed, in which case the AMF can indicate the latest status to gNB via NGAP messages, i.e., PATH SWITHCH ACKNOWLEDGE or HANDOVER REQUEST.

Proposal 1-2:	AMF can provide the authorization status of the UE reader to gNB via following NGAP messages:

- UE CONTEXT MODIFICATION REQUEST,

- PATH SWITHCH ACKNOWLEDGE,

- HANDOVER REQUEST.

XnAP impact

In the last meeting, it’s FFS whether the UE reader authorization status is sent from source/old gNB to the target/new gNB, as shown below:

FFS from the source/old gNB to the target/new gNB, and from the gNB-CU to the gNB-DU

Some companies think the authorization status under the old/source gNB may not be applicable to the target/new gNB, thus the Xn authorization status transmission is not needed. However, we believe it’s beneficial to have it. Several reasons:

- If the authorization status of the UE remains unchanged after handover, the target gNB can know the authorization status as early as possible and it can determine whether to use the UE reader as soon as possible. On the other hand, it’s not big issue if the authorization status of UE reader changes after handover, because AMF can send the updated authorization status to the target gNB through NGAP messages.

- However, if Xn does not transmit the authorization status, AMF needs to send the authorization status during each path switch and NG handover procedure.

- The authorization status idnication can be used as a UE reader indicator for the admission control by the target gNB.But if the authorization status is not transmitted over Xn, an indicator of UE reader should be added to XnAP messages.

Proposal 1-3:	Source/old gNB send the authorization status to target/new gNB during handover via following XnAP messages:

- HANDOVER REQUEST;

- RETRIEVE UE CONTEXT RESPONSE.

F1AP impact

For CU-DU split architecture in A-IoT enabled gNB in Topology 2, in the last meeting companies considered that the gNB-DU is responsible for allocating and coordinating usage of A-IoT radio resources, thus, upon the gNB-CU receives the UE reader authorization status from AMF, the gNB-CU can convey the authorization status to the gNB-DU. However, gNB may not allocate resources immediately after the UE reader is authorized, rather allocates resources upon receiving the inventory/command request from CN and intending to select the UE as reader. In addition, DU would not need to be visible to the authorization status of UE.

It is proposed to keep it FFS whether the gNB-CU should indicate the authorization status received from CN, to the gNB-DU.

Proposal 1-4:	Keep FFS whether the gNB-CU should indicate the authorization status, which is received from CN, to the gNB-DU.

## 2.2	UE reader selection

Regarding to UE reader selection, SA2 has achieved the following interim conclusions, which were captured in SA2 TR 23.700-30-020 [1], as shown in highlight part.

SA2 TR 23.700-30-020 7 Interim Conclusion for authorization UE reader selection part: Two scenarios will be supported for UE reader selection, AF providing UE reader ID case and AF providing Area case for all types of UE reader. - For AF providing UE reader ID case: - If UE Reader ID(s) is provided by the AF via the NEF for the operation, then that is used as the selected Reader(s) if authorized as UE reader. The AIOTF provides the selected UE Reader(s) to the NG-RAN. Editor’s note: How to select UE reader(s) based on the AF provided area information is FFS.

And in RAN TR 38.769, following descriptions about UE reader selection is captured:

RAN TR 38.769 6.3.6 Topology 2 aspects on the interface between UE reader and RAN For the UE reader selection, it is up to RAN3 and SA2 on whether BS or CN selects the UE reader(s) and what information is shared between BS and CN. From RAN2 perspective, it is assumed, at least for RRC based solution, there may be radio related reasons for the BS to be involved in the UE reader selection. 6.4.2.1 Solutions for Topology 2 6.4.2.1.0 General … In RRC based solution, Reader selection may need coordination between A-IoT-enabled gNB and A-IoT CN. NOTE 3: How to perform reader selection needs further discussions.

In our view, in Topology 2 RRC based solution, the final decision of UE reader selection is made by gNB, because the gNB is aware of Uu radio situation. If UE is determined to be selected as reader, the gNB takes into account the AIoT radio resources allocation for the UE reader.

Specifically, the CN can provide some assistance information to gNB to assist the gNB to coordinate/allocate A-IoT radio resource towards the UE reader(s). Recall in Topology 1, the assistance information (Requested Service Area Information) from AIOTF to the gNB includes the A-IoT Area ID(s) and/or the requested reader list to the gNB.

#### TS 38.413 #### 9.3.3.71 Requested Service Area Information This IE includes the Requested Service Area Information for the A-IoT inventory. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Requested Reader List; 0..1. >Requested Reader Item; 1..<maxnoofReaders>. >>Global gNB ID; M; 9.3.1.6. >>Reader Index; M; 9.3.1.aa1. Requested A-IoT Area List; 0..1. >Requested A-IoT Area Item; 1..<maxnoofAIoTAreas>. >>A-IoT Area ID; M; 9.3.1.aa2.

Proposal 2-1: 	The CN can provide some assistance information to gNB for inventory, which is the same as the Requested Service Area Information  in Topology 1.

Similar as R19 Topo 1, the CN can know the areas supported by gNB by means of OAM. However, due to the mobility of UE readers, the UE being used as a reader depends on when it connects to the network and being authorized. Thus, the gNB and core network may not be configured with all available UE readers and the AIoT area served by each UE reader.We analysis the four possible cases for reader selection as below:

Case 1: Neither area ID nor reader list is provided by CN

CN considers that all cells of the gNB are within the service area. In this case, the gNB can select the authorized UE readers based on its own implementation, for example, taking the radio resource allocation into account. If there is no appropriate UE reader, e.g., no UE reader is authorized or the resource allocation is failed, the gNB can respond failure message to CN.

Case 2: Only area ID(s) is provided by CN

In this case, not all coverage of the gNB is within the service area required by CN. The gNB needs to select the UE within the area indicated by CN, thus the gNB should know the available UE readers and which area the UE readers reside in.

Case 3: Only reader list is provided by CN

For the UE reader(s) indicated, CN should be able to know that whether the UE reader is available (e.g., whether UEenters the connected state, the authorized status of UE), and which area the UE reader resides in. To achieve that, some interaction between AMF and AIoTF may be needed, which will be discussed by SA2. But the gNB does not need to know where the UEs are.

Case 4: Both area ID(s) and reader list are provided by CN

For the UE reader(s) indicated, CN should be able to know whether the UE reader is available, and which area the UE reader resides in. The gNB also needs to know where the UE readers are, because it has to determine whether the UEs are within the requested area (s) so that to decide whether to select the UE.

For case 2 and case 4, i.e., when CN provides area information, gNB needs to know the AIoT area which the UE reader resides in, so as to select proper UE reader. To achieve that, the gNB may need to know the physical area of each AIoT area as well as the physical location of the UE readers.

However, it is difficult for the gNB to obtain the physical location of UE. Thus, it is suggested that the granularity of the AIoT area(s) provided by CN should not be smaller than one NR cell of the gNB.

Proposal 2-2:	The granularity of the AIoT area(s) provided by CN for inventory should not be smaller than one NR cell of the gNB.

It is required that each AIoT area consists of one or multiple NR cells. Two ways can be used for CN to indicate the requested area to gNB for inventory:

Option 1: Using Cell ID/TAC

CN directly indicates the list of Cell identity or TAC to perform the inventory/command service to gNB, and gNB selects UE within the indicated Cell(s)/TA(s). AIoTF should be configured with the mapping relation between the Cell ID/TAC and the AIoT area.

Option 2: Reusing the AIoT area ID in Rel-19

CN indicates the list of area(s) which is identified by the AIoT area ID to gNB. GNB selects UE reader in the serving cell(s) corresponding to the indicated area ID. The gNB is configured with a mapping relation between the AIoT area and the serving cell(s).

RAN3 can discuss which option to use.

Proposal 2-3:	RAN3 to discuss following two ways for CN to indicate the requested area to gNB for inventory:

- Using Cell ID/TAC,

- Reusing the AIoT area ID in Rel-19.

## 2.3	NGAP signaling procedures

As agreed in the last meeting, NGAP procedures specified for Topology 1 are base line for Topology 2. It is concluded that the AIoT NGAP messages in Topo-1 can be reused for Topo-2.

The issue is how to identify the UE reader between the gNB and CN. Because the CN can indicate a list of reader for inventory and the gNB reports the inventory results corresponding to a specific reader to the CN. It’s not clear whether the reader ID used in R19 is applicable. There are potentially two options to identify a UE reader:

Option 1: Reusing the reader ID

In Rel-19, reader is identified by a reader index which is unique within a gNB, and the reader ID list supported by the gNB is configured to CN. If the reader ID is reused, there is no need to enhance the existing AIoT NGAP messages. However, due to the mobility of UE reader, we should consider how to allocate UE reader ID. As one UE reader ID used under one gNB may be conflict with that used in another gNB when the UE changes the serving gNB, the reader ID for the same UE may need to be changed during the UE’s movement.

The ID of UE reader can be allocated by gNB or CN. If the reader ID is assigned by gNB, gNB needs to send it to CN, and this index is unique within the gNB. When UE changes the serving gNB, the reader ID will be reassigned by the new gNB. If the reader ID is allocated by CN, CN needs to send the it to gNB, this ID may be unique among multiple gNBs, and CN controls when to change the reader ID for UE.

Option 2: Using the UE NGAP ID

AIoT NGAP signalling have to be enhanced:

- UE NGAP ID list needs to be added in the Requested Service Area Information for CN to provide the UE reader list to gNB.

- A new list needs to be added to the Reader Repot List to report the inventory results generated by the UE readers.

As the UE NGAP ID is used, AMF is deeply involved in the inventory service, i.e., AMF needs to process the Inventory Request Transfer container and the Inventory Report Transfer container, which breaks the principle that the AMF is transparent to the AIoT services.

Based on above analysis, it can be observed that using the UE NGAP ID to identify UE reader may impact existing NGAP signalling significantly and involve AMF in AIoT service. Thus, we suggest reusing the reader ID to identify UE readers. Whether the UE reader ID is allocated by gNB or CN is FFS.

Proposal 3:	Reuse the reader ID to identify UE readers between gNB and CN. Whether the UE reader ID is allocated by gNB or CN is FFS.

## 2.4	UE Reader Mobility

As we discussed in the UE reader authorization section, during the NG handover, AMF can  provide the authentication status of the UE reader to the target gNB in the Handover Resource Allocation procedure. The authorization status information can be used as the indicator of UE reader for the target gNB to do access control, e.g., the target gNB can reject the UE’s handover if it does not support A-IoT. During the Xn handover, the source gNB can provide the UE reader authentication status to the target gNB in the Handover Preparation procedure. If the target gNB does not support AIoT, it can reject the UE’s handover.

Proposal 4-1:	The target gNB can reject the NG/Xn handover of UE reader if it does not support A-IoT.

Service continuity

Due to the mobility of UE reader, the UE performing an inventory/command service may change the serving cell. The issue whether the AIoT service performed by the UE reader can be continued by the same UE in the new cell should be discussed.

In our perspective, both scenarios of intra-gNB mobility and inter-gNB mobility should be considered, i.e., the UE reader moves to a new cell within the gNB or moves to a new gNB. Nevertheless, no matter what scenario is, or whether the AIoT service is area-associated or reader-associated, the device(s) which are expected to be reachable by the UE reader in the coverage of the old cell may not be reachable by the UE after the UE moving to the new cell. Therefore, the service should either be stopped or restarted with the UE reader after it moving to the new cell.

Proposal 4-2:	The AIoT service should either be stopped or restarted with the UE reader after the UE moves to a new cell.

Whether the service is restarted depends on whether this service can be performed by the new cell and UE is selected as reader after the UE’s movement. However, whether the service can be performed by the new cell depends on the request of service which the gNB receives from the CN. In some cases, this issue is irrelevant with the service continuity with respect to the UE reader, but the implementation of gNB. For example,

If neither area ID(s) nor reader list is provided by CN or only area ID(s) is provided by CN for the service performed by the UE before it moves to a new cell, and if the new cell is performing the same service and the (new)gNB still chooses the UE as a reader, then the gNB can restart the AIoT service with the UE, otherwise the UE stops performing the service.

Observation: 	Whether the service is restarted with the UE reader depends on whether this service can be performed by the new cell and the UE is selected as reader by the (new)gNB.

3. Conclusion

Based on the analysis in the Section 2, the conclusions of this paper are summarized as follows:

Proposal 1-1: 	AMF provides the UE reader authorization result to the gNB via INITIAL CONTEXT SETUP REQUEST after authorization to the UE.

Proposal 1-2:	AMF can provide the authorization status of UE reader to gNB via following NGAP messages:

- UE CONTEXT MODIFICATION REQUEST,

- PATH SWITHCH ACKNOWLEDGE,

- HANDOVER REQUEST.

Proposal 1-3:	Source/old gNB send the authorization status to target/new gNB during handover via following XnAP messages:

- HANDOVER REQUEST,

- RETRIEVE UE CONTEXT RESPONSE.

Proposal 1-4:	Keep FFS whether the gNB-CU should indicate the authorization status, which is received from CN, to the gNB-DU.

Proposal 2-1:	The CN can provide some assistance information to gNB for inventory, which is the same as the Requested Service Area Information  in Topology 1.

Proposal 2-2:	The granularity of the AIoT area(s) provided by CN for inventory should not be smaller than one NR cell of the gNB.

Proposal 2-3:	RAN3 to discuss following two ways for CN to indicate the requested area to gNB for inventory:

- Using Cell ID/TAC,

- Reusing the AIoT area ID in Rel-19.

Proposal 3:	Reuse the reader ID to identify UE readers between gNB and CN. Whether the UE reader ID is allocated by gNB or CN is FFS.

Proposal 4-1: 	The target gNB can reject the NG/Xn handover of UE reader if it does not support A-IoT.

Proposal 4-2:	The AIoT service should either be stopped or restarted with the UE reader after the UE moves to a new cell.

Observation: 	Whether the service is restarted with the UE reader depends on whether this service can be performed by the new cell and the UE is selected as reader by the (new)gNB.

4. References

- TR 23.700-30-020, Study on Architecture support of Ambient power-enabled Internet of Things (AIoT)  Phase 2 (Release 20)

- RP-252894, Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2

- SID SP-250834,  NEW SID on Architecture support of Ambient power-enabled Internet of Things - Phase 2 SA2

- 3GPP TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR (Release 19)”, V19.0.0 (2024-12)