3GPP TSG-RAN WG3 Meeting #130       	   				    R3-258352

Dallas, US, 17 – 21 November 2025

Agenda Item:		10.4

Source:			China Telecom

Title:	Consideration on 6G RAN internal functional split and interfaces

Document for:	 	Discussion & Decision

- Introduction

Last meeting, various companies had some preliminary discussions on the 6G RAN internal functional split and interfaces. We have captured the following key issues for further research:

Capture pain points and benefits (and potentially derived requirements) of 5G HLS

In this contribution, we will analyze the above issues from the perspective of RAN3.

- Discussion

2.1 6G RAN internal functional split

In 5G, the split RAN architecture is extensively specified, including CU-DU and CP-UP split. As 6G research begins, RAN3 will continue to investigate whether supports high layer split and study different functional splits between central and distributed units. NR protocol stack is taken as a basis for further discussion[1]. The following functional splits between central and distributed unit are possible, as illustrated in Figure 1.

Figure.1 The high layer split architecture

From the discussion at the last meeting, it can be seen that there are still significant differences among companies regarding whether to support HLS[2]. Both proponents and opponents have put forward compelling arguments.

The rationale for supporting High-Level Split primarily revolves around the flexibility, openness, and advantages it provides in specific scenarios:

- Deployment Flexibility and Multi-Vendor Ecosystem: The split architecture (CU-DU split) provides operators with the flexibility to choose from different deployment models based on their specific needs and enables the flexible sharing and allocation of hardware resources across multiple CUs or DUs. Continuing the openness successfully introduced in 5G helps maintain a healthy and competitive multi-vendor equipment ecosystem for 6G.

- Potential for Energy Efficiency Management: For instance, depending on traffic variation, CPUs for CU-CP, CU-UP, or DU can be put into "sleep mode" or have unused cores powered down, thereby effectively reducing network energy consumption.

- Crucial Support for Non-Terrestrial Networks (NTN): NTN is a key scenario planned for 6G Day 1. All NGSO constellations expected to be deployed in the next 10 years will be designed to support inter satellite links and carry regenerative payloads, but in order to optimize costs, most satellites will be designed to only undertake partial base station functions. HLS allows more functions to be offloaded to the ground or other satellites, which is highly attractive from an economic perspective.

However, the arguments against HLS or for a cautious approach stem from the practical challenges observed in 5G commercial deployment:

- Slow Progress in Commercial Deployment: So far, large-scale successful commercial deployment of the 5G split architecture has not been realized globally. Especially in the field of Open RAN, investment has declined, making the market outlook uncertain.

- Power Consumption Challenges: Power consumption is a key factor in network performance evaluation. The power consumption of a split architecture may be much higher than that of traditional integrated base station, posing a major obstacle for practical operator deployment.

- Increased Control Plane Latency: HLS in 5G led to the separation of RRC signaling processing, with some parameters handled by the CU and others by the DU. This split has been observed to significantly increases signaling interaction compared to integrated ones, and increasing control plane latency for the user.

- Standardization Complexity: The specifications for 5G HLS were refined over multiple releases, and optimizations for many features were added as afterthoughts in subsequent versions. This resulted in complex and sometimes incomplete F1 interface specifications

In our opinion, HLS does bring certain benefits in certain application scenarios such as NTN and network sharing, but its drawbacks cannot be ignored. The decision on whether to support HLS in 6G essentially involves a trade-off between the advantages of architectural flexibility and the shortcomings of current technical implementations. 6G standardization must squarely address the challenges encountered by 5G HLS regarding power consumption, latency, and standardization complexity, striving to resolve these issues in the early stages of standard design.

Proposal 1: The trade-offs among implementation complexity, standardization efforts, and projected benefits should be fully evaluated in order to provide a clear recommendation on whether to support the split architecture in 6G.

2.2 Inter 6G RAN Node Interface

The Xn interface is a logical interface defined within the 5G RAN nodes. The existence of Xn interface is the key to achieving high performance and reliability of 5G. Its core function is to enable direct and rapid coordination and information exchange between gNBs.

6G should support multi-node access, cooperative transmission, and flexible handover. To fulfill these requirements, a direct connection interface between 6G RAN node is necessary (functionally equivalent to the Xn). However, a key consideration arises from the 5G architecture, where the RAN includes both gNBs and ng-eNBs, interconnected via the Xn interface. Specifically, an NG-RAN node is defined as either:

An NG-RAN node is either:

-	a gNB, providing NR user plane and control plane protocol terminations towards the UE; or

-	an ng-eNB, providing E-UTRA user plane and control plane protocol terminations towards the UE.

Similarly, before defining the interface between 6G RAN nodes, it is crucial to clarify the composition of the 6G RAN node. we need to confirm whether the enhanced gNB is classified as a 6G RAN node and whether interoperability between such enhanced gNBs and new 6G NBs should be supported.

Proposal 2: Before discussing the direct connection interface between 6G RAN nodes, RAN3 needs to confirm whether the enhanced gNB is classified as a 6G RAN node.

A 6G NB inherently belongs to the 6G RAN node. The general principles for the specification of the inter 6G NB interface are as follows:

-	The inter 6G NB interface shall be open for multi-vendor interoperability;

-	The inter 6G NB interface shall support control plane and user plane separation;

-	The inter 6G NB interface shall be future proof to fulfil different new requirements, support new services and new functions.

Proposal 3: RAN3 confirms to support the direct inter 6G NB interfaces.

- Conclusions

In this paper, we have the following proposal:

Proposal 1: The trade-offs among implementation complexity, standardization efforts, and projected benefits should be fully evaluated in order to provide a clear recommendation on whether to support the split architecture in 6G.

Proposal 2: Before discussing the direct connection interface between 6G RAN nodes, RAN3 needs to confirm whether the enhanced gNB is classified as a 6G RAN node.

Proposal 3: RAN3 confirms to support the direct inter 6G NB interfaces.

- References

[1] 	3GPP TR 38.801, Study on new radio access technology: Radio access architecture and interfaces.

[2] 	RAN3#129bis Meeting Report.