3GPP TSG-RAN WG3 Meeting #130	R3-258370

Dallas, USA, 17 – 21 November 2025

Agenda item:	12.2.2

Source: 	Lenovo

Title: 	AI/ML for Intra-CU LTM

Document for:	Discussion

- 1. Introduction

The Rel-20 study item on AI/ML for NG-RAN phase 3 has the following objectives [1]:

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows: - Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios: - multi-hop UE trajectory across gNBs; - intra-CU LTM; - handover enhancements, e.g. inter-CU LTM. NOTE: No UE impacts.

This paper discusses the potential use cases of AI/ML for intra-CU LTM.

- 2. Discussion

2.1	L1 or L3 measurement based LTM

There is an open issue that

For AI/ML study we consider both L1 and L3 measurement based Intra-CU LTM.

The main motivation of LTM is to enable faster cell switch and reduce the handover interruption. The main benefits of LTM come from L1 measurement, DL/UL early sync. L1 beam measurement responds more quickly to channel variations than L3 measurements due to less filtering. This allows the gNB-DU to react faster to select appropriate cells and beams for early UL and DL synchronization and cell switching in LTM.

L3 measurement based LTM was introduced in the late stage of Release-18. The motivation is that some UE may not support L1 measurement temporarily. For these UEs, L3 measurement based LTM can be used instead.

From our view, L1 measurement based LTM is the main feature with many advantages whereas L3 measurement based LTM is a complementary and temporary solution for the UEs that are not capable of L3 measurements.

Proposal 1: L1 measurement based intra-CU LTM should be prioritized in the study item.

2.2	Beam Prediction in LTM

In RAN3#129bis meeting, it was agreed that the model training is either located in gNB-CU or OAM, whereas the model inference is located in gNB-CU. We believe that this principle should be respected in the design of AI/ML for LTM.

For CU-DU split architecture, the following two scenarios are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

In Rel-18 and Rel19 intra-CU LTM, the UE sends L1 measurement reports to the gNB-DU using either MAC CE or UCI. As such, the gNB-CU has no L1 measurement results available. Without L1 measurement results, gNB-CU is not able to perform AI/ML model training or inference for L1 measurement report based intra-CU LTM operation. It might be suggested that the gNB-CU could collect historical L1 measurement reports from the gNB-DU. However, this would violate the functional split principle between the CU and DU and bring extremely complex impacts.

Observation 1: gNB-CU is responsible for model training and/or model inference, but it has no L1 based measurement results.

For L3 based measurement, the UE measures multiple beams (at least one) of a cell and the measurements results are averaged to derive the cell quality. In doing so, the UE is configured to consider a subset of the detected beams. Filtering takes place at two different levels: at the physical layer to derive beam quality and then at RRC level to derive cell quality from multiple beams. Cell quality from beam measurements is derived in the same way for the serving cell(s) and for the non-serving cell(s). Measurement reports may contain the measurement results of the X best beams if the UE is configured to do so by the gNB.

For L1 based measurement used in LTM event-triggered measurement, the UE measures the beams configured for LTM. Filtering takes place at the physical layer to derive the beam quality. The measurement reports contain the measurement results of the X (at least one) beams and reporting is with MAC CE.

L1 beam measurement responds more quickly to channel variations than L3 measurements due to less filtering. This allows the gNB-DU to react faster to select appropriate cells and beams for early UL and DL synchronization and cell switching in LTM. The early UL/DL synchronization and cell switch are determined by L1 measurement results. The historical L1 measurement results is the key to predict the beams for early UL and DL synchronization and cell switching in LTM. Without historical L1 measurement results, gNB-CU is not able to predict the beams of candidate cells for early UL and DL synchronization and cell switching in LTM.

Observation 2: Without L1 based measurement results, gNB-CU is not able to predict the beams of candidate cells for LTM early UL and DL synchronization and LTM cell switch.

In RAN3#129bis, there is an open issue identified:

Further discuss whether to study addition of beam prediction in UE Trajectory prediction.

In LTM, beams are primarily used for DL synchronization, such as activating or deactivating the TCI states of candidate cells. Additionally, the beam (i.e., TCI state ID) can be included in the cell switch command to indicate the initial beam to be used in the target cell. Beam(s) are selected by the gNB-DU based on L1 measurement results. As noted in Observation 2, we do not consider that the gNB-CU can provide beam prediction information for DL synchronization and cell switching.

Even thought it was agreed that Study candidate beams along with candidate cells for AI/ML based Intra-CU LTM HO preparation., we do not think it is feasible and useful to support beam prediction in LTM HO preparation. Candidate cell selection is performed by the gNB-CU based on L3 measurement results. However, beam information is not utilized during the LTM preparation phase. Currently, the gNB-CU only provides candidate cells to the candidate gNB-DU without indicating associated beam(s). Beam-related configurations for candidate cells are provided from the candidate gNB-DU to the gNB-CU during the LTM preparation phase. In other words, the gNB-CU is unaware of the beam-related configurations of candidate cells prior to the LTM preparation phase. Therefore, the gNB-CU cannot provide predicted beams along with candidate cells to the candidate gNB-DU.

Observation 3: Beam prediction is not feasible and useful for LTM HO preparation.

Thus, we consider that beam prediction by the gNB-CU is not feasible for LTM.

Proposal 2: Beam prediction by the gNB-CU is not feasible and useful for LTM early UL and DL synchronization and LTM cell switch.

Proposal 3: Beam prediction by the gNB-CU is not feasible and useful for LTM HO preparation. RAN3 needs to revert the agreement made in last meeting.

2.3		TA Prediction

Another open issues were identified as:

Further discuss predicting TA value in the next meeting.

Further discuss prediction of validity time of a measured TA value.

A key aspect of UL early synchronization is obtaining the TA employed for the candidate cell. For UL early synchronization in LTM, UE performs early TA acquisition procedure before LTM switch execution.

To obtain the TA value, the UE must switch to the candidate cell and perform a RACH procedure by transmitting a preamble to it. Even though this occurs prior to the LTM cell switch, service interruption persists because the UE has to redirect its transmitter (Tx) to the candidate cell. This problem is exacerbated when multiple candidate cells are configured, as the UE may need to perform UL early synchronization for each. It also involves network signaling overhead.

It is beneficial to use AI/ML enabling network to predict the TA value for a candidate cell directly which does not need UE perform early TA acquisition procedure anymore. For example, gNB-CU can predict the TA value for a candidate prior to the LTM cell switch and transmit the predicted TA information to the UE via the LTM Cell Switch Command MAC CE. The inputs for TA value prediction can include historical UE location or measurement information associated with a TA value.

With the predicted early TA, the candidate gNB-DU does not need to prepare PRACH and preamble resource anymore and the signalling exchanged among gNB-CU, source gNB-DU and candidate gNB-DU can be significantly reduced.

Observation 4: TA prediction has the following benefits:

- it can avoid service interruption due to switching UE Tx to perform early TA acquisition procedure;

- candidate gNB-DU does not need to reserve PRACH and preamble resource any more;

- avoid signalling exchanged among gNB-CU, source gNB-DU and candidate gNB-DU.

The problem is that the TA prediction relies on measurement results and historical TA information. Our understanding is L3 measurement results is sufficient to predict TA since TA is per cell basis. However, the gNB-CU lacks the historical TA information. Currently the timing advance for a UE is maintained in the gNB-DU. The gNB DU can assign or adjust the TA value by RAR or by Timing Advance Command MAC CE. To support TA prediction in gNB-CU, it would need to collects the historical TA information of UEs.

The Output of TA prediction can include the predicted TA value and the valid TAT of the predicted /measured TA value.

Proposal 4: TA prediction in gNB-CU needs inputs from gNB-DU, e.g., historical TA information of UEs.

Proposal 5: Outputs of TA prediction may include the predicted TA value and the valid TAT of the predicted /measured TA value.

2.4		AI/ML assisted candidate cell selection

In RAN3#129bis meeting, it was agreed that

RAN3 to study AI/ML assisted candidate cell selection for LTM Handover Preparation

During the LTM preparation phase, the gNB-CU may generate a list of LTM candidate cells based on measurement reports received from UEs. For each candidate cell, the gNB-CU sends a UE CONTEXT SETUP REQUEST message to the corresponding candidate gNB-DU(s), requesting the gNB-DU to prepare resources and configurations for LTM. However, if the gNB-CU selects an inappropriate candidate cell, the LTM cell switch to this candidate cell will fail. In such cases, the UE may either recover its connection or reestablish it to a cell that is not included in the original candidate cell list.

In the meanwhile, we noticed that, it was already supported in Rel18 that gNB/gNB-CU can predict the UE trajectory which helps to improve the legacy L3 based handover. The predicted future UE trajectory can be sent from the source gNB to the target gNB over Xn interface conveyed in Handover Request message.

TS 38.423 9.1.1.1 HANDOVER REQUEST This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover. Direction: source NG-RAN node ® target NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. Source NG-RAN node UE XnAP ID reference; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the source NG-RAN node; YES; reject. …; …; …; …; …; …; …. Cell Based UE Trajectory Prediction; O; 9.2.3.180; This IE contains information about cells that a UE is predicted to be connected to.; YES; ignore.

Observation 5: In Rel18, it’s supported that gNB/gNB-CU can predict UE’s future trajectory in cell granularity.

The same AI capability to predict UE trajectory can be utilized to assist the LTM candidate cell selection with some enhancements. In particular, to mitigate potential LTM switch failures caused by inappropriate candidate cell selection, the gNB-CU can leverage AI/ML technologies to predict candidate cells with higher accuracy. Specifically, the gNB-CU may use historical UE mobility information and past measurement results to predict the probability of a UE moving to each candidate cell. Based on this predicted probability:

- Candidate gNB-DUs can prioritize accepting candidate cells with a higher UE movement probability, ensuring resource preparation aligns with actual mobility needs. Since there are multiple candidate cells (maximum number is 8) can be prepared, the UE movement probability is critical that for the candidate gNB-DU to select the final prepared candidate cells.

- The source gNB-DU can reference this probability information when selecting the target cell for the LTM Cell Switch Command MAC CE, further reducing the risk of switch failures.

Proposal 6: In addition to the Cell based UE Trajectory Prediction, the gNB-CU should also provide the candidate gNB-DU with the predicted UE movement probability for the candidate cells, to support the candidate gNB-DU in making the final candidate cell selection.

Proposal 7: In addition to the Cell based UE Trajectory Prediction, the gNB-CU should also provide the serving gNB-DU with the predicted UE movement probability for the selected candidate cells, to assist the serving gNB-DU selecting the target cell.

- 3. Conclusion

In this contribution, we discuss the potential use cases of AI/ML for intra-CU LTM:

Observation 1: gNB-CU is responsible for model training and/or model inference, but it has no L1 based measurement results.

Observation 2: Without L1 based measurement results, gNB-CU is not able to predict the beams of candidate cells for LTM early UL and DL synchronization and LTM cell switch.

Observation 3: Beam prediction is not feasible and useful for LTM HO preparation.

Observation 4: TA prediction has the following benefits:

- it can avoid service interruption due to switching UE Tx to perform early TA acquisition procedure;

- candidate gNB-DU does not need to reserve PRACH and preamble resource any more;

- avoid signalling exchanged among gNB-CU, source gNB-DU and candidate gNB-DU.

Observation 5: In Rel18, it’s supported that gNB/gNB-CU can predict UE’s future trajectory in cell granularity.

Proposal 1: L1 measurement based intra-CU LTM should be prioritized in the study item.

Proposal 2: Beam prediction by the gNB-CU is not feasible and useful for LTM early UL and DL synchronization and LTM cell switch.

Proposal 3: Beam prediction by the gNB-CU is not feasible and useful for LTM HO preparation. RAN3 needs to revert the agreement made in last meeting.

Proposal 4: TA prediction in gNB-CU needs inputs from gNB-DU, e.g., historical TA information of UEs.

Proposal 5: Outputs of TA prediction may include the predicted TA value and the valid TAT of the predicted /measured TA value.

Proposal 6: In addition to the Cell based UE Trajectory Prediction, the gNB-CU should also provide the candidate gNB-DU with the predicted UE movement probability for the candidate cells, to support the candidate gNB-DU in making the final candidate cell selection.

Proposal 7: In addition to the Cell based UE Trajectory Prediction, the gNB-CU should also provide the serving gNB-DU with the predicted UE movement probability for the selected candidate cells, to assist the serving gNB-DU selecting the target cell.

- Reference

[1] RP-252867. Revised SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3. ZTE Corporation, NEC.

[2] TS 38.300