3GPP TSG-RAN WG3 #130                                                            R3-258652

Dallas, USA, 17th – 21th November 2025

Agenda item:	10.2.1

Source:	Samsung

Title:	Discussion on RAN architecture principle and requirement

Document for:	Discussion & Decision

Introduction

In RAN#108 meeting, the 6G WG SID was approved with the following objectives on RAN architecture:

- Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2]

In this contribution, we continue the discussion on RAN architecture general principles and requirements.

Discussion

2.1 FFS in the TR

RAN3 achieved some agreements on the general principles and deployment scenarios in the last meeting, and the following agreements are capatured into RAN3 TR 38.760-3 V0.1.0:

5.1 General Principles All requirements of TR 38.914 will serve as the basis for RAN3 architecture design principles. The 6G architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality. [FFS] 5.2 Deployment Scenarios Editor’s note: This section may be used to describe the details/solutions related to deployment scenarios as per 38.914. The 6G RAN architecture shall strive to support the deployment scenarios defined in TR 38.914. - FFS on the implications of this requirement on 6G RAN architecture. - FFS whether all deployment scenarios of TR 38.914 can be supported.

The first [FFS] is about the virtualized and/or cloud-based implementation requirement. 5G RAN already supports network vritulization according to the requirements in TR38.913. It is stated that “the RAN architecture shall allow deployments using Network Function Virtualization”. Through virtualization technology, the functions of the RAN are moduled and decoupled, some functions that were once impelmented in hardware can be achieved through software. This reduces operational costs and supports multi-vendor interoperability.

Cloud-based deployments is built on veitualization by leveraging cloul-native technoloy to centrally deploy virtualized RAN functions. This centralized approach enables better resource sharing and more flexible resource allocation. Thus can reduce the costs and enahnce the network scalability. The cloud-based implementation is the use case to apply virtualized RAN function to cloud enviornment, therefore, allowing RAN function virtualization is the more fundamental requirements. While it is implementation choice to use virtualized or clould based implementation.

Proposal 1: The 6G architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality.

For the second FFS, regarding the implications of 6G RAN architecture supports the deployment scenarios defined in TR 38.914, Currently following 12 deployments scenarios are defined for 6G:

- Indoor hotspot,

- Dense urban

- Rural

- Urban

- Sub-Urban macro

- High speed

- Extreme long distance coverage in low density area

- Urban coverage for massive connection

- Air-to-Ground Scenario upward pointed

- Non-Terrestrial network

- Urban grid

- Highway Scenario

These scenarios have diverse requirements in terms of coverage, capacity, and costs. And these varying demands mean the 6G RAN architecture needs to be highly flexible and customizable. 5G RAN architecture is a good example, is highly suitable for variable environments and operational deployments. The Deployment scenarios are still under discussion, some scenarios lack specific details. So further study on the implications of this requirement on 6G RAN architecture is needed.

For the third FFS, about whether all deployment scenarios of TR 38.914 can be supported, we think 6G RAN architecture shall support all the deployment scenarios defined in TR 38.914. Basically, these are all practical deployment scenarios. Our goal is to define 6G RAN architecture that supports all of them. 6G RAN architecture is required to be flexible and support automated configuration. Through the tailored configurations in different deployment approaches, the RAN architecture can adapt to a wide range of environmental and operational scenarios.

Proposal 2: RAN architecture and interface design shall satisfy all the deployment scenarios defined in RAN plenary TR 38.914.

2.2 Open issues in the last meeting

According to the discussion in the last meeting, there were two open issues need more discussion in the following meetings. The open issues are:

- Study whether the protocol stack of RAN3-defined interfaces can evolve over the lifespan of 6G.

- Study the impact of enhanced RAN-based service awareness on 6G RAN architecture.

For the first open issue, we noticed the closest requirement to this in 5G is “The RAN architecture shall allow for the RAN and the CN to evolve independently”. The core principle for evolve independently is de-couple the core network function and RAN function. This is achieved through several key technologies:

- Serviced based architecture in 5GC. The core network is decomposed into smaller, modular and reusable network functions, such as AMF, SMF. Each function can be evolved without affecting other functions. It is possible that evolving in 5GC function has no impact to RAN function and vise verse.

- Control plane and user plane separation. The CP/UP separation create a stable interaction module between RAN and CN. CP handles signaling transmission, connection management. The UP forwards user data with high efficiency and low latency. The RAN can connect the 5GC via two distinct standardized interfaces, NGC, NGU. The separation allows the CP and UP to evolve independently. UP optimization can be performed without any changes to the CP protocols.

- Standardized interface with backward/ forward compatibility. The fully standardized interface guarantees interoperability and independent evolution between different vendor’s equipment, as long as the RAN vendor and the core vendor both correctly implement the standard interface. RAN and CN can support different release, upgrade on can be independently. i.e., RAN updated to new release doesn’t affect CN using old release, because new release is backward compatible. The nodes using different releases will operate based on the features they both understand.

The first open issue “whether the protocol stack of RAN3-defined interfaces can evolve over the lifespan of 6G” is slightly different from CN-RAN evolvement in 5G. It refers to whether radio network layer (RNL) protocol and transport network layer (TNL) protocol can evolve independently over the lifespan of 6G.

Currently, RNL and TNL are separated and decoupled. In theory they can evolve independently. While SCTP and GTP-U are relatively stable with few evolutionary versions, thus we have not discussed the impact of TNL evolution on RNL in 5G.

In 4G/5G, even RNL and TNL are separated, they still maintain certain interaction which serves as the foundational information for establishing transmission link. For example, the RNL message needs to transmit UP Transport Layer Information IE which include GTP-U tunnel related information. TNL protocol also have some information related to RNL. Such as the TNL SCTP assigns specific Payload Protocol Identifier (ppid) for the application layer protocol.

These basic associations are closely tied to the transport layer protocol. If the TNL uses different transport protocol, we can image different associated information will be carried in the RNL message. When the basic associated information is defined in the first release, theoretically the RNL or TNL can continue to evolve independently.

As for the evolution of RAN3-defined interfaces, we feel this open issue is not very clear. Is it based on the existing SCTP/GTP-U in 5G or does it not rely on the existing TNL protocols? If based on the existing SCTP/GTP-U, we think RNL and TNL can evolve independently.

Proposal 3: Radio Network Layer protocol and Transport Network Layer protocol of RAN3-defined interfaces can evolve independently, if using legacy TNL protocols.

If we speak this open issue in general terms and make no assumptions about the TNL protocol, we understand it is immature to study whether RNL and TNL can evolve independently. First, there should be a consensus on which protocols to be used for TNL. With above considerations, we suggest to remove this open issue or hold the study.

Proposal 4: Remove or hold the study on whether the protocol stack of RAN3-defined interfaces can evolve over the lifespan of 6G before having consensus on 6G TNL protocols.

There were some discussions last meeting about whether to use a new TNL protocol for control plane or user plane. In summary, there are three proposals:

- Use QUIC to replace SCTP

- Use QUIC to replace GTP-U

- Use QUIC to replace both

QUIC is the Quick UDP Internet Connection. It is a new transport layer protocol aiming to improve the speed and efficiency of network transmission. The QUIC protocol has the following advantages:

- Ensuring the security of data transmission due to the built-in TLS encryption.

- QUIC is built on UDP, avoiding the delay of TCP connection establishment and the three-way handshake, enabling faster connection establishment.

- QUIC supports sending and receiving multiple data streams simultaneously on the same connection, improving transmission efficiency and provide Low latency

- Allows for communication between the client and server without interruption, even if there is a disconnection in the middle of the current session.

On the other hand, using SCTP as TNL protocol for control plane and GTP-U for user plane also have many advantages. Firstly, they are widely used in the existing network, high compatible with the existing mobile network. Secondly, Reusing SCTP in CP and GTP-U in UP will provide mature handling in every condition and be verified in interoperability test. They have proved reliability, safety and interoperability. While using new protocols are lack of verification. It may demand new conformance and interoperability test. RAN3 needs to carefully compare the existing protocols and new protocols and involve other working groups to the discussion, e.g., CT4.

Proposal 5: Further discuss whether to adopt a new protocol at Transport Network Layer in RAN3-defined interface with coordination with CT4.

For the second open issue, study the impact of enhanced RAN-based service awareness on RAN architecture. For XR service in 5G, CN delivers more parameters reflecting application layer characteristics to the RAN. Based on these characteristics, the RAN has a certain awareness of service level information and can perform specific processing.

In 6G, it has been agreed that “The 6G RAN architecture shall support enhanced service awareness in RAN” in TR 38.914. Generally, it is recognized that providing the RAN with more application layer information helps improve resource allocation efficiency, enhance system performance, and increase user satisfaction.

We noticed 6G RAN requires a deeper service awareness. e.g., 6G new service have variable QoS requirement. It may need to introduce a new QoS to reflect application layer data characteristics. While to determine how this specifically impact RAN functionality, we need to identify which services require RAN service awareness.  We are open to discuss the impact of RAN supporting enhanced service awareness, suggest to clarify which services require enhanced RAN service awareness and then discuss the impact.

Proposal 6: RAN3 is kindly asked to identify which services require enhanced RAN service awareness before analysis the impact to RAN function/architecture.

2.3 Additional proposals

6G network should achieve sustainable green development. For a truly sustainable network, we should design network can support automatic configuration and optimization. The legacy troubleshooting mechanism is also very useful in 6G network. In 5G, different mechanisms are defined for troubleshooting and validation testing, e.g., SON, MDT, ANR. To improve network operation and maintenance efficiency, 6G network should support SON/MDT/ANR from day 1.

Proposal 7: RAN3 is kindly asked to support SON/MDT/ANR in the first release.

There are some important features of 5G that will be supported from the beginning of 6G. What specific features will be supported are being discussed in the RAN plenary meeting. After the RAN plenary meeting makes a decision, RAN3 can further discuss the details. Currently, the RAN plenary meeting has agreed that

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

In 5G, from RAN point of view, we only support MOCN where RAN can connect to different operators' CN. In 5G, MOCN supports NG-RAN sharing with or without multiple Cell-ID broadcast. RAN sharing with multiple Cell-ID broadcast means separate Cell-ID is used for different operators. It appears as independent logical cells sharing the same physical resources. The benefit is to ensure isolation and independence between different operators. It also supports RAN sharing without multiple Cell-ID broadcast, where the shared operators use the same Cell-ID. These two methods are used in different use case, can continue to be used in 6G. Based on current situation, the RAN sharing scenario should be captured in the TR, and we can use 5G RAN sharing principle as the baseline in 6G.

Figure 1. 6G RAN sharing

Proposal 8: RAN3 is kindly asked to capture the RAN sharing in the appropriate section of TR, and to use the 5G RAN sharing principle as the baseline for 6G.

We assume the 5G principles in NG interface and Xn interface can be reused in 6G, such as RAN interface should be open, should support modular procedure design. In 5G, RAN procedures of each interface can be basically divided into two groups:

- Non-UE-associated services: They are related to the whole interface instance utilizing a non-UE-associated signalling connection.

- UE-associated services: They are related to one UE. Functions that provide these services are associated with a UE-associated signalling connection that is maintained for the UE.

Proposal 9: It is proposed basic principles for 5G procedures design (e.g., procedures divided into UE associated and non-UE associated) can be reused for 6G.

From the above discussion and proposal, we provide the corresponding TP in the annex.

Proposal 10: It is proposed to agree the TP in Annex.

Text Proposal

This contribution provides initial discussion on RAN architecture requirements and principles, and the following proposals are captured,

Proposal 1: The 6G architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality.

Proposal 2: RAN architecture and interface design shall satisfy all the deployment scenarios defined in RAN plenary TR 38.914.

Proposal 3: Radio Network Layer protocol and Transport Network Layer protocol of RAN3-defined interfaces can evolve independently over the lifespan of 6G, if using legacy TNL protocols.

Proposal 4: Remove or hold the study on whether the protocol stack of RAN3-defined interfaces can evolve over the lifespan of 6G before having consensus on 6G TNL protocols.

Proposal 5: Further discuss whether to adopt a new protocol at Transport Network Layer in RAN3-defined interface with coordination with CT4.

Proposal 6: RAN3 is kindly asked to identify which services require enhanced RAN service awareness before analysis the impact to RAN function/architecture.

Proposal 7: RAN3 is kindly asked to support SON/MDT/ANR in the first release.

Proposal 8: RAN3 is kindly asked to capture the RAN sharing in the appropriate section of TR, and to use the 5G RAN sharing principle as the baseline for 6G.

Proposal 9: It is proposed basic principles for 5G procedures design (e.g., procedures divided into UE associated and non-UE associated) can be reused for 6G.

Proposal 10: It is proposed to agree the TP in Annex.

Annex: pCR for TR 38.760-3

5.1.	General Principles

All requirements of TR 38.914 will serve as the basis for RAN3 architecture design principles.

The 6G architecture shall allow for virtualized and/or cloud-based implementations of 6G RAN functionality.

Basic principles for 5G procedures design (e.g., procedures divided into UE associated and non-UE associated) can be reused for 6G.

5.2	Deployment Scenarios

Editor’s note: This section may be used to describe the details/solutions related to deployment scenarios as per 38.914.

The 6G RAN architecture shall strive to support the deployment scenarios defined in TR 38.914.

-  FFS on the implications of this requirement on 6G RAN architecture.

5.2.x	RAN Sharing

6G RAN should support sharing of the RAN between multiple operators. 5G sharing is used as baseline.

Figure 5.2.x-1. RAN sharing

x.y.	SON/MDT/ANR

Self-organizing network (SON) functions such as support for mobility robustness optimization, mobility load balancing, RACH optimization, energy savings, etc. were introduced in 5G to make deployment easier and cheaper. Equivalent functionality shall also be considered for the 6G.

References

[1] RP-252912, New SID: Study on 6G Radio

[2] TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies

[3] TR 38.914, Study on 6G Scenarios and Requirements

[4] TR 38.760-3 Study on 6G Radio RAN3 aspects