3GPP TSG RAN3 Meeting #120	R3-258697

Dallas, US, 17 – 21 November 2025

Agenda item: 	    10.5.1

Source: 	China Unicom

Title: 	Considerations on new 6G AI/ML Use Cases for RAN

Document for:	Discussion and Decision

- Introduction

At RAN#108 meeting, the Study on 6G Radio has been approved [1]. Following the schedule of the study item, this contribution provides our views on the use cases of 6G AI/ML for RAN according to previous RAN3#129bis meeting.

- Discussion on the use cases of 6G AI/ML for RAN

  - AI/ML Use Cases in 5G

The design of AI/ML algorithms and models for RAN3 led use cases are implementation-specific and out of RAN3 scope.

The following use cases will be studied in RAN3:

- AI/ML based Network energy saving

- AI/ML based mobility optimization

The use cases of AI.ML for 5G is studies since Rel-18, and several SIs/WIs are started across different RAN working groups, and the summaries are captured in TR 21.918 [2]. The applications of AI/ML in 5G/5G-A are categorized as AI for network (AI4Net) and network for AI (Net4AI). In 5G-A Rel-19, some use cases are studied and supported in 3GPP RAN, e.g. CSI feedback, Beam management, positioning accuracy enhancements, AI/ML-based Network Energy Saving, Load Balancing, Mobility Optimization, Slicing and Coverage and Capacity Optimization (CCO), etc. Considering the progress of 3GPP standardization and the requirements of 5G network deployments, the features including CSI feedback, energy saving are the most promising features for 5G commercial deployments in near future. As the issues of power saving/energy saving, positioning, beam management, and mobility are critical for operators, the market demands for the above use cases are still very strong from commercial deployment in the future. Therefore, to summarize from the existing use cases from 5G, it is proposed that the use cases of power saving/energy saving, positioning, beam management, and mobility optimization (including handover of both network and UE sides) are the high priority features. For the other cases of load balancing as well as other features without commercial needs and consensus in 3GPP, it is kindly proposed to exclude such use cases in 6G.

In last RAN3#129bis meeting, two use cases are agreed to study how to support first in the above. While the other use cases are needed for further discussion to expand 6G network AI/ML functionalities, e.g. AI/ML assisted QoE, cross domain collaboration for AI/ML, Agentic AI, AI computing resource management, AI/ML-enable RAN digital twin, AI/ML-assisted resilience, etc.

Observation 1: New use cases for AI/ML are needed for further discussion to expand 6G network capability.

  - New Use Cases in 6G

There are some new potential use cases of 6G AI/ML are discussed during RAN plenary and workshop before. Considering the discussion on new features and AI/ML, one example use case is that AI/ML use cases should combine the new features with AI, e.g. sensing and AI/ML for digital twins for E-environment reconstruction. The potential scenario of the digital twin solution for E-environment reconstruction is utilized the sensing function based on the radio measurement report as well as other measurement results, which can save the cost of extensive field test and reduce the complexity and workload for the time-consuming network optimization.

For the other use cases of AI4Net, the AI/ML algorithm design for 6G is out of the scope of 6G RAN. While cross-layer and end-to-end Qos management and scheduling enhancement with the capability of service awareness are one of the most important features for operators to enhance the 6G network capabilities. To support this feature, the parallel discussion on how to define uniform design of data collection mechanism is also needed. The exposure of the application layer information combining with service information and user experience information as well as other higher layer information to 6G RAN can be utilized to optimize the 6G network resource from core network to RAN nodes and guarantee the service experience as required.

For the use cases of Net4AI, the infrastructure of 6G network can access and utilize for model training, data sharing and inference if the latency as well as other service requirements (e.g. local processing) of the AI-enabled service cannot be fulfilled using the legacy solutions as 5G. 6G network is considering to share the resource with UE as well as the application service providers in the future. From the network side, it is obvious that 6G network is superior to 5G network with the advantages of security, flexibility, end-to-end capabilities. For the use cases of Net4AI, it needs more time to discuss the potential use cases as the service and requirements are not very clear.

Observation 2: The combination solution of sensing and AI techniques can be used for digital twin for E-environment reconstruction in 6G.

Proposal 1: RAN3 discuss and support digital twin for E-environment reconstruction as the new AI/ML use case.

Proposal 2: RAN3 continues the discussion on how to support service awareness in 6G RAN with AI/ML.

- Conclusion

The observations and proposals of this contribution are as follows.

Observation 1: New use cases for AI/ML are needed for further discussion to expand 6G network capability.

Observation 2: The combination solution of sensing and AI techniques can be used for digital twin for E-environment reconstruction in 6G.

Proposal 1: RAN3 discuss and support digital twin for E-environment reconstruction as the new AI/ML use case.

Proposal 2: RAN3 continues the discussion on how to support service awareness in 6G RAN with AI/ML.

- References

- RP‑251881 New SID: Study on 6G Radio 	NTT DOCOMO

- TR 21.918 Release 18 Description; Summary of Rel-18 Work Items ETSI