3GPP TSG-RAN WG3 Meeting #131	R3-260413

Gothenburg, SE, 09th ~ 13th Feb, 2026

Title:	6G Data Collection

Agenda Item:	10.2

Source:	Huawei

Document for:	Discussion

# 1	Introduction

The 6G data collection was discussed in last RAN3 meeting and the following general principles were identified:

The 6G RAN architecture supports data collection according to the following principles:

- Reusability of collected data is supported.

- Data collected or generated by the 6G RAN can be used by the RAN and it can be made available to other entities, if needed. FFS which entities.

- The 6G RAN can request data, and use the data collected from other entities. FFS which entities.

NOTE 1:	The collection, storage and usage of data follows the security principles which will be defined by relevant WGs.

In addition to the above, the following FFS was captured:

Triggering of data collection needs further discussion

In the following sections we discuss how to resolve these open issues, but also propose additional principles to be captured.

# 2	Discussion

## 2.1	Progress in other groups

RAN2 reached the following agreements:

Agreements

Study the follow aspects as a starting point:

Data types (non-real time)

-	AI/ML (data collection for offline training)

-	SON/MDT, QoE - (details on what is included in 6G depends on RANP)

-	Sensing (the detailed characteristics (including what cases are real time or not) will be discussed in later phase, after some RAN1 and RAN2 discussions in the sensing session)

QoS (e.g. data volume and latency)

-	Define some categories we want to base our discussion on.   Details of QoS for some cases may depend on other WGs.

Termination points (impacted group)

-	UE to RAN

-	UE to CN

-	UE to OAM

-	UE to UE server

Understand the consumer (may be different than the termination) of the data and whether data has to be decodable by RAN.

It can be observed that RAN2 is planning to investigate the data required to be transferred over Uu for different use cases, and the associated QoS. They are also planning to discuss where the information is sent and whether the data is visible to RAN.

Observation 1: RAN2 is just at the beginning of data collection/transfer phase, and only some data types have been selected for study, such as understanding QoS requirements, termination points (impacted group).

Observation 2: In RAN2 understanding, the entity which could request the data from UE could be RAN, CN, OAM or UE server, which should be pending on different use cases.

SA2 captured the following for KI#21 in 23.801-01:

## 5.21	Key Issue #21: 6G data framework in SA2

NOTE 1:	Coordination (as requested by SA plenary LS SP-251261) with SA WG5 is required. Common functionalities for data handling may be identified between SA WG2 and SA WG5 to be coordinated with each other to avoid incompatible or duplicated solutions for data framework.

The Key issue is based on WT#5, which includes the following aspects:

1.	Identify high level use cases (e.g. AIML in the core, UE data collection for UE-sided model training, Sensing) potentially subject to the data framework.

...

2.	Study required data framework functionalities to support the identified use cases, e.g.:

a)	data discovery and data registration.

b)	data collection and transfer (including configuration of the data source (e.g. about collected data, reporting period and mechanisms to use), transfer of the collected data (including data distribution)).

c)	data labelling/metadata handling.

d)	data processing (e.g. data anonymization, data analysis, data generation, etc.).

e)	data storage and retrieval.

f)	data exposure.

The functionalities above may need to consider the quality of data, latency and data volume.

...

...3.	Study potential system impacts of user consent, privacy, security and data governance from SA WG2 architectural and system level perspective for data generated in 6G system corresponding to the use cases and functionalities in aspects 1 and 2.

...

As can be seen above, SA2 will start working on identifying use cases (e.g. Sensing) and study the required functionalities.

## 2.2	Scope in RAN3

In the FFS captured in last meeting, we interpret the questions to relate to which entities outside 6G RAN that can request data from 6G RAN and which entities can provide data to 6G RAN. This discussion would be outside RAN3 scope and would therefore need to wait for progress in other groups, i.e. SA2 and SA5.

In our view, the data collection framework may include sending information to RAN nodes but also other network entities, like CN or OAM.

The information sent to OAM, RAN and CN would require requesting and reporting the information on different interfaces. Taking 5G as an example, the information sent to CN is triggered on NGAP, information for OAM would be triggered over the management interface and information to other RAN nodes would be triggered on the Xn interface.

Therefore, it seems like a reasonable assumption that the solution for unified data collection may differ depending on where the information is sent. Unlike the work in e.g. RAN2, where the information can be sent form the UE to RAN in a unified way, we should not exclude the possibility that the reporting framework is different depending on where the information is sent.

In any case, it seems reasonable to wait with further discussing the solution for entities outside RAN until the other groups has progressed on at least the use cases.

RAN3 should wait for the progress in other working group on the conclusion of use cases, before identifying the entities for data request and reception.

RAN3 should collaborate with other working groups, e.g. RAN2, to discuss the triggering of data collection which is also pending on different use cases.

For the information exchange within RAN however, the scope is slightly clearer. In the past we have used different principles for exchanging information collected between nodes for different features (SON, AI/ML, eICIC) and also different principles in different releases (e.g. load reporting for 4G vs 5G). But it seems one important task in 6G would be to further study how this framework can be unified, also within RAN.

## 2.3	Data collection between RAN nodes

With the assumption of a framework that allows different RAN nodes to request data from another RAN node, we believe the following set of principles should apply:

- The nodes can be configured (by e.g. OAM) either to which nodes a data request can be sent, to which node a report is allowed to be sent, or both.

Firstly, this definition of the roles is something we usually ask SA5 to provide. There could for example be cases where the operator prefer that one node is not involved in this kind of functionality, thereby e.g. not allowing the establishment of the request/report interface for some of the nodes in the network. We think the configuration could also include additional aspects, e.g. allowed type/intensity/volume of measurements, but this is something that needs to be further developed together with SA5.

- A node receiving a request would decide whether the requested measurement can be performed.

Secondly, an important principle is that it is the reporting node that determines which measurements are feasible. Admitting a measurement will consume resources in the reporting node and only the reporting node would have a good enough understanding on the available resources. A potential future discussion here could be what the response would include and whether negotiation is possible.

To start the discussion, we propose to capture the following two aspects to the general principles for exchange between RAN nodes:

For data collection between RAN nodes, the nodes can be configured either to which nodes a data request can be sent, to which node a report is allowed to be sent, or both.

A node receiving a request would decide whether the requested measurement can be performed.

# 3	Conclusion

This document has the following observations and proposals:

Observation 1: RAN2 is just at the beginning of data collection/transfer phase, and only some data types have been selected for study, such as understanding QoS requirements, termination points (impacted group).

Observation 2: In RAN2 understanding, the entity which could request the data from UE could be RAN, CN, OAM or UE server, which should be pending on different use cases.

Proposal 1:	RAN3 should wait for the progress in other working group on the conclusion of use cases, before identifying the entities for data request and reception.

Proposal 2:	RAN3 should collaborate with other working groups, e.g. RAN2, to discuss the triggering of data collection which is also pending on different use cases.

Proposal 3:	For data collection between RAN nodes, the nodes can be configured either to which nodes a data request can be sent, to which node a report is allowed to be sent, or both.

Proposal 4:	A node receiving a request would decide whether the requested measurement can be performed.