3GPP TSG-RAN WG3 Meeting #131	R3-260718

Gothenburg, SE, 09th ~ 13th Feb, 2026

Title:	(TP for BLCR 38.401) Inter-CU LTM

Agenda Item:	11.2.1

Source:	Huawei, Nokia, Qualcomm

Document Type:	other

# 1	Introduction

This is a TP for 38.401.

# Annex A:	TP for TS 38.401

## 7.12	MRO support for LTM and Inter-CU LTM

The gNB-CU receives an RLF report associated to an LTM mobility event from the UE, via the Failure indication message over Xn, via the Handover Report message over Xn. The gNB-CU performs initial analysis. In case of failures due to inappropriate cell switch triggering or wrong target cell selection for LTM cell switch, the gNB-CU forwards the RLF report to the last serving gNB-DU in case of too late LTM cell switch, or to the source gNB-DU in case of too early LTM cell switch or LTM cell switch to wrong cell.

If a UE is served by the gNB-CU and then re-establishes or recovers in the same gNB-CU after LTM cell switch failure, or if the gNB-CU receives a Failure Indication without an RLF report from another gNB, the gNB-CU may send a failure report without an RLF report to the last serving gNB-DU in case of too late LTM cell switch, or to the source gNB-DU in case of too early LTM cell switch or LTM cell switch to wrong cell.

The target gNB-DU identifies that a Beam Failure Recovery (BFR) has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt). The target gNB-DU performs initial analysis and sends the recovery beam information to the gNB-CU(s) which may further forward the information to the source gNB-DU in case the source gNB-DU was responsible for triggering LTM cell switch.

The target gNB-DU identifies that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell. The target gNB-DU sends the re-established or recovery beam information, the TA value used at successful RACH-based access, or both  to the gNB-CU(s) which may further forward the information to the source gNB-DU in case the source gNB-DU was responsible for triggering LTM cell switch.

End of Change