3GPP TSG-RAN WG3 #131	R3-260660

Gothenburg, Sweden, 9th – 13th February 2026

Agenda Item:	9.2.3

Source:	Ericsson (Moderator)

Title: 	Summary of offline discussions: UAV regulation

Document for:	Discussion and Approval

- Introduction

CB: # 10_UAVregulation

- Discuss multiple thresholds from the same NEF

- Try to converge on NGAP, XnAP, and stage 2 CRs

- Reply LS to SA2, if CRs are agreeable

(Ericsson - moderator)

- For Chairman

********************To be updated ******************

3 Discussion

RAN3 has discussed the Reply LS from SA2 in R3-260014. It says:

RAN3 asks: “whether the gNB can be provided with multiple aerial UE reporting configurations from the AMF for the same aerial UE”. SA2 replies: Aerial UE location reporting configuration shall enable altitude reporting whenever the UE’s altitude meets any of the configured altitude threshold values. It cannot be ruled out that different pairs of altitude thresholds (i.e., minimum and maximum altitude thresholds) are derived by the UAS NF / NEF for each of the TA/Cell/NG-RAN in the assigned trajectory. The assigned trajectory event is described in clause 5.2.2.3.1 of TS 23.502, and the Stage 3 implementation (see clauses 6.2.6.2.35-37 in TS 29.518) allows to set minimum and maximum altitude thresholds specific to a TA/Cell/NG-RAN. RAN3 asks: “whether this would require the gNB to configure the aerial UE with multiple aerial UE flight information configurations to the concerned aerial UE”. SA2 replies: Possible, under the assumption that the UAS NF / NEF derived different pairs of altitude thresholds for each of the TA/Cell/NG-RAN in the assigned trajectory and the gNB configures the aerial UE at once for the entire flight. RAN3 asks: “whether the gNB is expected to report multiple flight information reporting to AMF in accordance with each configuration” SA2 replies: gNB is expected to report to the AMF whenever the concerned aerial UE reports its altitude information to the gNB in accordance with the relevant configuration (i.e., a pair of altitude thresholds that can be specific to a particular TA/cell/NG-RAN node).

Related to “It cannot be ruled out that different pairs of altitude thresholds (i.e., minimum and maximum altitude thresholds) are derived by the UAS NF / NEF for each of the TA/Cell/NG-RAN in the assigned trajectory.”

Do we understand that SA2 says one pair of altitude thresholds per TA or Per Cell or Per NG-RAN node, or multiple pair?

| Company Name | Comments |
| --- | --- |
| Ericsson | Yes, only one pair of thresholds per TA, or Per Cell or Per NG-RAN node |
| CMCC | Yes, I think SA2 means only one pair of thresholds per TA/Cell/NG-RAN, anyway, they may not have some scheme to guarantee it. |
| ZTE | Same view with E/ |

If the threshold pair is per Cell ( for example Cell1, Cell 2, Cell3), during handover, should this be sent to the target? Note that in the current specification, the “Aerial UE Flight Information Reporting Control” is sent to the target gNB.

| Company Name | Comments |
| --- | --- |
| Ericsson | RAN3 can understand that the threshold pair is only valid for the given Cell, thus there is no point to send over to the target. However this would mean the change to the existing specification. We should ask SA2 if this is what they want. |
| CMCC | RAN3 may further discuss as enhancement whether when handover, a list of pair of thresholds (for different cells) can be sent to target as UE context. But this is not a high priority issue in this meeting, suggest to focus on NG interface first. And it is RAN3 business, don’t think SA2 can provide some guidance on it. |
| ZTE | We are also ok to let SA2 further clarify this issue if needed. Based on our understanding, the altitude reporting is configured for a specific UE. If we only configure it for the current serving cell (or serving gNB), when handover happens, CN has to re-configure the altitude reporting to the target node. This understanding does not align with the existing mechanism of location reporting control. We should notice that the Area of Interest can be a large area. The UE may only exist in some of the configured areas during a period. Similarly, CN may configure a list of Cell IDs with thresholds, but CN does not expect the UE to appear in all configured cells in all cases. In addition, let’s check this from another view. If a UAV flying path is a loop (e.g., Cell_A => Cell_B => Cell_A), by applying E//’s view, CN has to configure altitude reporting for this UE again and again. This does not look efficient enough |

If the threshold pair is gNB, does it mean that AMF will send a list of the thresholds pair /gNB, so the serving gNB will include this list to the target gNB, if the target gNB is in the list.

| Company Name | Comments |
| --- | --- |
| Ericsson | We do not see the benefit that a list of the thresholds pair/gNB is sent to the serving gNB. But we need to clarify this to SA2. |
| CMCC | Can ask SA2 to clarify on this scenario. We don’t think it is SA2’s intention to configure different pair thresholds for different gNB, and in their attached CR, it is clear that no NG-RAN node ID included via N2. |
| ZTE | We are ok to further check with SA2. However, our view is that regardless of whether the target gNB is inside or outside the area/list, the configuration shall be forwarded to the target gNB. In other words, the key issue is whether gNB shall directly release the configuration when a UE moves out of the area scope. Based on our understanding, this decision should be up to CN implementation. Based on various factors (e.g., AI prediction, UE history trajectory), if CN believes that UE will shortly re-enter the area, CN will decide to keep the configuration. If not, the existing location reporting control mechanism can be used for CN to explicitly release the configuration. And this does not need any additional enhancement for existing mechanism. |

Do we need to support different UAS NF/NEF sending the thresholds pair to gNB, so that for example RAN3 has to introduce Routing ID?

| Company Name | Comments |
| --- | --- |
| Ericsson | We think it is possible. But we should ask SA2 to clarify this point. |
| CMCC | We are open to this discussion, but maybe next meeting. Not bad to ask for SA2 clarification in the reply LS. |
| ZTE | OK |

4 Moderator Summary