3GPP TSG-RAN WG3 Meeting #131	R3-260654

Gothenburg, Sweden, 9th ~13th Feb 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1415 | Rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the inclusion of security parameter for A-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Telecom, China Unicom, CATT, CMCC, Nokia, Samsung, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2026-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 agreed that: - Add a 1 bit optionality bit for 128bits security field in paging message. For now, we state in our specification that this bit is set to present in this release according to SA3 TS. If SA3 confirms that it can be optional after LS reply it shall be updated. In SA#110 meeting, there was no consensus on whether it can be optional used. Based on this, in RAN3, it is better for us to align the NGAP handling as the MAC layer, i.e., optionally include the IE, and clarify that it shall be provided in the procedural text part. 38.391: - Security Parameter Present Indication (SPPI): The length of the field is 1 bit. This field indicates whether Security Parameter field is present (when set to 1) or absent (when set to 0). In this release, this field is set to 1 according to TS 33.369 [6]. - Security Parameter: The length of the field is 128 bits if present. This field contains the parameter RANDAIOT_n as specified in TS 33.369 [6]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include RANDAIOT_n BIT STRING (SIZE (128, ...)) as an optional sub-IE of the Inventory Request Transfer IE, and clarify that in this release it shall be included. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only adds the missed security parameter. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN is not able to receive the security parameter from CN for AIoT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.20.1.2, 9.3.6.1, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------Start of the Change-----------------------

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

//skip unchanged part

[58]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[59]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[60]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

[x]	3GPP TS 33.369: "Security aspects of Ambient Internet of Things (AIoT) services for isolated private networks".

---------------------Start of the Change-----------------------

8.20.1	Inventory Request

8.20.1.1	General

The purpose of the Inventory Request procedure is to request the NG-RAN node to perform A-IoT paging over the A-IoT radio. This procedure applies only if the NG-RAN node is a gNB.

8.20.1.2	Successful Operation

Figure 8.20.1.2-1: Inventory Request, successful operation.

The A-IoT CN node triggers the procedure by sending an INVENTORY REQUEST message to the NG-RAN node. If the NG-RAN node accepts the request, the NG-RAN node responds with the INVENTORY RESPONSE message.

The NG-RAN node shall, include the same values of the AIOTF Identifier IE and A-IoT Correlation Identifier IE in the INVENTORY RESPONSE message as received from the INVENTORY REQUEST message.

Upon receiving the Inventory Assistance Information IE contained in the Inventory Request Transfer IE in the INVENTORY REQUEST message, the NG-RAN node may take the information into account as defined in TS 38.300 [8].

Upon receiving the A-IoT RAND_n IE contained in the Inventory Request Transfer IE in the INVENTORY REQUEST message, the NG-RAN node shall, if supported, behave as specified in TS 33.369 [x].

Interaction with Inventory Report procedure

When starting the requested inventory, the NG-RAN node shall, include the same values of the AIOTF Identifier IE and A-IoT Correlation Identifier IE in the INVENTORY REPORT message(s) as received from the INVENTORY REQUEST message

If the Follow on Command Indication IE set to "true" is included in the Inventory Request Transfer IE in the INVENTORY REQUEST message, the NG-RAN node shall consider that there will be command to be transmitted to the device(s), and shall allocate and include a RAN A-IoT Device NGAP ID IE in each Device Report Item IE within the Inventory Report Transfer IE in the INVENTORY REPORT message(s).

---------------------Start of the Change-----------------------

9.3.6.1	Inventory Request Transfer

This IE provides the inventory request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.69 |  | YES | reject |
| A-IoT Device Identification Requested | M |  | 9.3.3.73 |  | YES | reject |
| Requested Service Area Information | M |  | 9.3.3.71 |  | YES | reject |
| Inventory Assistance Information | M |  | 9.3.3.72 |  | YES | reject |
| Follow on Command Indication | O |  | ENUMERATED (true, ...) | This IE indicates that there is command(s) to be transmitted. | YES | reject |
| A-IoT RAND_n | O |  | BIT STRING (SIZE (128, ...)) | Contains the RANDAIOT_n as defined in TS 33.369 [x]. In this version of specification, this IE shall be included. | YES | reject |

---------------------Start of the Change-----------------------

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AIOTFIdentifier,

id-AIoT-CorrelationIdentifier,

id-AIoT-DeviceIdentificationRequested,

id-AIoT-RAND-n,

id-AIoT-RequestedServiceAreaInformation,

id-AIoT-InventoryAssistanceInformation,

id-AIoT-FollowonCommandIndication,

id-RAN-AIOT-Device-NGAP-ID,

id-AIoT-CommandAssistanceInformation,

id-AIoT-NASPDU,

id-AlternativeQoSParaSetList,

id-AssistanceInformationQoE-Meas,

id-AdditionalCancelledlocationReportingReferenceIDList,

id-BurstArrivalTimeDownlink,

id-Cause,

//skip unchanged part

AIoT-InventoryAssistanceInformation ::= SEQUENCE {

expectedD2RMsgSize							INTEGER (1..256, ...),

approximateNoofTargetDevices				INTEGER (1..65535, ...)				OPTIONAL,

timeInterval								INTEGER (1..256, ...) 				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-InventoryAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

AIoT-InventoryAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-NASPDU  ::= OCTET STRING

AIoT-RAND-n  ::= BIT STRING (SIZE(128, ...))

AIoT-ReaderReportList ::= SEQUENCE (SIZE(1..maxnoofReaders)) OF AIoT-ReaderReportItem

AIoT-ReaderReportItem ::= SEQUENCE {

readerIndex					AIoT-ReaderIndex,

deviceReportList			AIoT-DeviceReportList,

iE-Extensions		ProtocolExtensionContainer { { AIoT-ReaderReportItem-ExtIEs} } OPTIONAL,

...

}

AIoT-ReaderReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

//skip unchanged part

InventoryRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { InventoryRequestTransferIEs} },

...

}

InventoryRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIoT-CorrelationIdentifier				CRITICALITY reject	TYPE AIoT-CorrelationIdentifier							PRESENCE	mandatory	}|

{ ID id-AIoT-DeviceIdentificationRequested		CRITICALITY reject	TYPE AIoT-DeviceIdentificationRequested				PRESENCE	mandatory	}|

{ ID id-AIoT-RequestedServiceAreaInformation	CRITICALITY reject	TYPE AIoT-RequestedServiceAreaInformation			PRESENCE mandatory	}|

{ ID id-AIoT-InventoryAssistanceInformation		CRITICALITY reject	TYPE AIoT-InventoryAssistanceInformation				PRESENCE	mandatory	}|

{ ID id-AIoT-FollowonCommandIndication			CRITICALITY reject	TYPE AIoT-FollowonCommandIndication					PRESENCE	optional		}|

{ ID id-AIoT-RAND-n								CRITICALITY reject	TYPE AIoT-RAND-n											PRESENCE	optional		},

...

}

---------------------Start of the Change-----------------------

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

//skip unchanged part

id-MonitoringRequestonAvailableBitrate					ProtocolIE-ID ::= 489

id-MMSID												ProtocolIE-ID ::= 490

id-Indication-of-Bitrate-Adaptation						ProtocolIE-ID ::= 491

id-SCGActivationTime									ProtocolIE-ID ::= 492

id-Aerial-UE-FlightInformationReporting					ProtocolIE-ID ::= 493

id-Aerial-UE-FlightInformationReportingControl			ProtocolIE-ID ::= 494

id-LPWUSDisableIndication								ProtocolIE-ID ::= 495

id-AIoT-RAND-n											ProtocolIE-ID ::= xxx

END

-- ASN1STOP

---------------------End of the Change-----------------------