3GPP TSG-RAN WG3 Meeting #131	R3-26xxxx

# Goteborg, Sweden,  9 – 13 February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1686 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of AI/ML-based CCO for XnAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT,CMCC, ZTE Corporation, Samsung, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_NGRAN_enh-Core |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Abnormal Conditions for AI/ML-based CCO is missing in current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new Abnormal Conditions, when NG-RAN node2 received Future Coverage Modification List IE which do not match any previous one, the NG-RAN node2 shall discard the received IE. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR is backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<Start of Change >>>>>>>>>>>>>>>>>>>>

#### 8.4.2.4	Abnormal Conditions

If the NG-RAN node1 after initiating NG-RAN node Configuration Update procedure receives neither NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message nor NG-RAN NODE CONFIGURATION UPDATE FAILURE message, the NG-RAN node1 may reinitiate the NG-RAN node Configuration Update procedure towards the same NG-RAN node2, provided that the content of the new NG-RAN NODE CONFIGURATION UPDATE message is identical to the content of the previously unacknowledged NG-RAN NODE CONFIGURATION UPDATE message.

If the Future Coverage Modification List IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message and at least one but not all instances of the Predicted Coverage Modification Cause IE within the Future Coverage Modification List IE are set to "cancel, then the NG-RAN Node shall ignore the Future Coverage Modification List IE.

If the Future Coverage Modification List IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message and if:

- all instances of the Predicted Coverage Modification Cause IE within the Future Coverage Modification List IE are set to "cancel", and

- the list of cells and beam listed in the Future Coverage Modification List IE is not the same as the list of  cells and beam listed in a previously received Future Coverage Modification List IE,

then the NG-RAN Node shall ignore the Future Coverage Modification List IE.

<<<<<<<<<<<<<<<<<<<<End of Change >>>>>>>>>>>>>>>>>>>>