3GPP TSG-RAN WG3 Meeting #131	R3-260xxx

Gothenburg, Sweden, 9-13 February 2026

Agenda item:	10.3.2.1

Source: 	Lenovo

Title: 	On RAN-CN Point to Point Interface

Document for:	Discussion

# Introduction

This paper provides pCR to TR 38.760-3 regarding the P2P interface.

# Annex – pCR to TR 38.760-3

-------------------------------------------------1st Change-----------------------------------------

##### 6.1.3.1.1 Point to Point (P2P)

A RAN-CN P2P interface refers to application layer communication between the 6G RAN node and the CN entity for 6G by means of elementary procedures, either triggered by the 6G RAN node or by the CN entity for 6G.

Editor’s Note 1: FFS whether multiple CN entities can be involved.

Potential options for the 6G P2P protocol stack are as follows:

Editor's Note 2:	Other options (e.g., Application (ASN.1) / HTTP / QUIC / UDP / IP) are not precluded. /Alternative 1 for the new protocol stack/

-	Option 1: SCTP based

-	Option 2: QUIC based

The protocol stacks of the option 1 and option 2 of RAN-CN P2P interface are shown as in Figure 6.1.3.1.1-X:

(a) SCTP-based								(b) QUIC based

Figure 6.1.3.1.1-X.	Protocol Stacks for RAN-CN P2P interface - Option 1 and Option 2

/Alternative 2 for the new protocol stack/

-	Option 3: Application (ASN.1) / HTTP / QUIC / UDP / IP based

The protocol stacks of option 3 of RAN-CN P2P interface is shown as in Figure 6.1.3.1.1-Y:

Figure 6.1.3.1.1-Y.	Protocol Stacks for RAN-CN P2P interface - Option 3

A high-level description of SCTP and QUIC/UDP is summarized in Table 6.1.3.1.1-X. Editor's Note 3: Additional features might be added

Table 6.1.3.1.1-X High-level description of SCTP and QUIC/UDP

| Feature | SCTP | QUIC/UDP |
| --- | --- | --- |
| Protocol Stack Design | Kernel-space IP-based (RFC 4960) Message-oriented | User-space UDP-based (RFC 9000 series) Byte-stream-oriented |
| Connection Establishment | 4-way handshake | 0-RTT/1-RTT handshake |
| Security | External security (e.g., by DTLS) | Integrated with TLS 1.3 |
| Multi-stream Mechanism | Multiple streams within a single association | Dynamically created bidirectional/unidirectional streams within a connection |
| Multi-homing / Connection Migration | Native multi-homing with independent congestion windows per destination address | Connection ID decouples IP/port, supports seamless migration Single global congestion window for the connection with more flexible path switching |