3GPP TSG-RAN WG3 Meeting #131	R3-260319

Gothenburg, Sweden, 9-13 February 2026

Agenda item:	10.3.2.1

Source: 	Lenovo

Title: 	On RAN-CN Point to Point Interface

Document for:	Discussion

# Introduction

This paper provides pCR to TR 38.760-3 regarding the P2P interface.

# Annex – pCR to TR 38.760-3

-------------------------------------------------1st Change-----------------------------------------

##### 6.1.3.1.1 Point to Point (P2P)

A RAN-CN P2P interface refers to application layer communication between the 6G RAN node and the CN entity for 6G by means of elementary procedures, either triggered by the 6G RAN node or by the CN entity for 6G.

Editor’s Note 1: FFS whether multiple CN entities can be involved.

Potential options for the 6G P2P protocol stack are as follows:

Editor's Note 2:	Other options are not precluded.

-	Option 1: SCTP based

The protocol stacks of option 1 for RAN-CN P2P interface is shown as in Figure 6.1.3.1.1-X1:

Figure 6.1.3.1.1-X1. (Option 1) SCTP based

-	Option 2: QUIC based

The protocol stacks of option 2 for RAN-CN P2P interface are shown as in Figure 6.1.3.1.1-X2:

Figure 6.1.3.1.1-X2.								(Option 2) QUIC based

Editor's Note x:	FFS on the name of Nx-AP.

Editor’s Note x+1: FFS the structure of the application layer. FFS on whether the application layer may contain HTTP.

A high-level description about SCTP and QUIC/UDP is summarized in Table 6.1.3.1.1-X.

Table 6.1.3.1.1-X High-level comparison between SCTP and QUIC/UDP

| Feature | SCTP | QUIC |
| --- | --- | --- |
| Standards specification | IEFT RFC 9260 | IETF RFC 9000-9002 |
| Protocol aspects | Message-oriented transport protocol Endpoints establish an SCTP association with an association ID The SCTP association supports multiple streams SCTP supports IP multihoming | Stream-oriented transport protocol Endpoints establish a QUIC connection with a connection ID The QUIC connection supports multiple streams QUIC supports connection migration to new IP addresses. |
| Establishment of association/connection | 4-way handshake | 0-RTT/1-RTT handshake |
| Security support | No integrated security | Integrated security (TLS 1.3). Use of security is mandatory. |
| Multi-stream Mechanism | Multiple streams within a single association Supports unordered/partial reliability | Dynamically created bidirectional/unidirectional streams within a connection Stream ID self-assigned with no negotiation overhead |
| Multi-homing / Connection Migration | IP multi-homing with independent congestion windows per destination address | Connection ID decouples IP/port, supports seamless migration. Single congestion window for the connection |
| Keepalive mechanism | Via build-in heartbeat mechanism | Via ping frames triggered by upper layers |

Editor's Note x:	The content of the table may need further discussion. Other features are not precluded.