3GPP TSG-RAN WG3 Meeting #131	R3-260319

Gothenburg, Sweden, 9-13 February 2026

Agenda item:	10.3.2.1

Source: 	Lenovo

Title: 	On RAN-CN Point to Point Interface

Document for:	Discussion

# Introduction

This paper provides pCR to TR 38.760-3 regarding the P2P interface.

# Annex – pCR to TR 38.760-3

-------------------------------------------------1st Change-----------------------------------------

##### 6.1.3.1.1 Point to Point (P2P)

A RAN-CN P2P interface refers to application layer communication between the 6G RAN node and the CN entity for 6G by means of elementary procedures, either triggered by the 6G RAN node or by the CN entity for 6G.

Editor’s Note 1: FFS whether multiple CN entities can be involved.

Potential options for the 6G P2P protocol stack are as follows:

Editor's Note 2:	Other options are not precluded.

-	Option 1: SCTP based

The protocol stacks of option 1 for RAN-CN P2P interface is shown as in Figure 6.1.3.1.1-X1:

Figure 6.1.3.1.1-X1. (Option 1) SCTP based

-	Option 2: QUIC based

The protocol stacks of option 2 for RAN-CN P2P interface are shown as in Figure 6.1.3.1.1-X2:

Figure 6.1.3.1.1-X2.								(Option 2) QUIC based

Editor's Note x:	FFS on the name of Nx-AP.

A high-level description about SCTP and QUIC/UDP is summarized in the following Table 6.1.3.1.1-X.

Table 6.1.3.1.1-X High-level comparison between SCTP and QUIC/UDP

| Feature | SCTP | QUIC/UDP |
| --- | --- | --- |
| Protocol Stack Position | Transport Layer (kernel-space/) IP-based (RFC 4960) | User-space UDP-based (RFC 9000 series) |
| Core Design | Message-oriented Native multi-streaming & multi-homing | Byte-stream-oriented integrates UDP/TCP/TLS capabilities |
| Node level Connection Establishment | 4-way handshake External security (DTLS, IPSec), mandatorily support, optionally use. | 0-RTT/1-RTT handshake Native encryption (TLS 1.3 integrated), mandatorily use. Session identified by connection ID |
| Multi-stream Mechanism | Multiple streams within a single association Supports unordered/partial reliability | Dynamically created bidirectional/unidirectional streams within a connection Stream ID self-assigned with no negotiation overhead |
| Multi-homing / Connection Migration | Native multi-homing with independent congestion windows per destination address SCTP bundling may cause unnecessary bundling of the upper layer AP messages | Connection ID decouples IP/port, supports seamless migration Multi-Homing with MP QUIC. |
| Connection keepalive detection mechanism | Native Heartbeat | Implemented in QUIC library and allow the use of adaptative interval, e.g. more frequent PING in a less stable BH network. |
| NAT/Firewall | Big issues with NAT/Firewall Dependency on IANA, e.g. IANA assigned port number 38412 for NGAP | QUIC uses UDP port number. No issue for NAT/Firewall. No Dependency on IANA. |

Editor's Note x:	The content of the table may need further discussion. Other features are not precluded.