3GPP TSG-RAN WG3 Meeting #131	R3-260xxx

Gothenburg, Sweden, 9-13 February 2026

Agenda item:	10.3.2.1

Source: 	Lenovo, ZTE

Title: 	On RAN-CN Point to Point Interface

Document for:	Discussion

# Introduction

This paper provides pCR to TR 38.760-3 regarding the P2P interface.

# Annex – pCR to TR 38.760-3

-------------------------------------------------1st Change-----------------------------------------

##### 6.1.3.1.1 Point to Point (P2P)

A RAN-CN P2P interface refers to application layer communication between the 6G RAN node and the CN entity for 6G by means of elementary procedures, either triggered by the 6G RAN node or by the CN entity for 6G.

Editor’s Note 1: FFS whether multiple CN entities can be involved.

Potential options for the 6G P2P protocol stack are as follows:

Editor's Note 2:	Other options are not precluded.

-	Option 1: SCTP based

The protocol stack of option 1 for RAN-CN P2P interface is shown as in Figure 6.1.3.1.1-X1:

Figure 6.1.3.1.1-X1. (Option 1) SCTP based

-	Option 2: QUIC based

The protocol stack of option 2 for RAN-CN P2P interface are shown as in Figure 6.1.3.1.1-X2:

Figure 6.1.3.1.1-X2.	(Option 2) QUIC based

Editor's Note x:	FFS on the name of Nx-AP.

Editor’s Note x+1: FFS on whether option 2 may contain HTTP.

A high-level description of SCTP and QUIC/UDP is summarized in the following Table 6.1.3.1.1-X.

Table 6.1.3.1.1-X High-level description of SCTP and QUIC/UDP

| Feature | SCTP | QUIC/UDP |
| --- | --- | --- |
| Standardization | RFC 4960 | RFC 9000-9002 |
| Core Design | Message-oriented | Supports both Stream-oriented mode and datagram mode. |
| Establishment of node level association/connection | 4-way handshake | 0-RTT/1-RTT handshake |
| Security | External security (e.g., DTLS, IPSec) Integrated security (optional) | Integrated security (TLS 1.3) enables end to end security. Use of security is mandatory. |
| Multi-stream Mechanism | Multiple streams within a single association. Supports unordered/partial reliability. Support concurrent streams. | Multiple streams within a connection. Stream ID self-assigned with no negotiation overhead. Support concurrent streams. |
| Multi-homing / Connection Migration | IP multi-homing with independent congestion windows per destination address | Connection ID decouples IP/port, Single congestion window for the connection. MP QUIC is active internet draft in version 19 expected to become RFC in 1H26. Concurrent Multipath Support |

Editor's Note x:	The content of the table may need further discussion. Other features are not precluded.