# 3GPP TSG-RAN3 Meeting #131	R3-26xxxx

# Goteborg, Sweden, 9th Feb 2026 - 17th Feb 2026

# Agenda Item:	10.3.2.2

# Source:	ZTE Corporation

# Title:	pCR to TR 38.760-3 on 6G RAN-CN  SBI options

# Document for:	Discussions & Approval

# 1	Introduction

This pCR intends to capture the RAN3 agreements in RAN3#131 on 6G RAN-CN SBI options.

# 2	pCR to TR 38.760-3

##### 6.1.3.1.2 Service Based Interface (SBI)

A RAN-CN SBI (service-based interface) refers to application layer communication between the 6G RAN node and the CN entity for 6G by means of services provided/exposed by either the 6G RAN node or the CN entity for 6G.

Communication between 6G RAN node and CN entity for 6G may be either direct or indirect.

-	For the direct communication case, the 6G RAN node and the CN entity for 6G are configured to directly communicate with each other.

-	For the indirect communication case, the CN entity for 6G and the 6G RAN node are configured to communicate indirectly via a communication proxy, with the service request containing request address and discovery and selection information so that the communication proxy is able to route the request to a suitable service producer. The communication proxy is either configured accordingly or can perform discovery with a discovery function.

Editor’s Note 1: FFS whether multiple CN entities can be involved.

Potential options for the 6G SBI protocol stack are as follows:

Editor's Note 2:	Other options are not precluded.

-	Option 1: TCP+ HTTP/2 based SBI:

The protocol stack for TCP+HTTP/2 based SBI is shown below.

Figure 6.1.3.1.2-1: SBI - Opt1: TCP+HTTP/2 based SBI Protocol Stack

-	Option 2: QUIC+HTTP/3 based SBI

The protocol stack for QUIC+HTTP/3 based SBI is shown below.

Figure 6.1.3.1.2-2: SBI - Opt2: QUIC+HTTP/3 based SBI Protocol Stack

---------------End of the Change------------------