# 3GPP TSG-RAN3 Meeting #131	R3-26xxxx

# Goteborg, Sweden, 9th Feb 2026 - 17th Feb 2026

# Agenda Item:	10.3.2.2

# Source:	ZTE Corporation

# Title:	pCR to TR 38.760-3 on 6G RAN-CN  SBI options

# Document for:	Discussions & Approval

# 1	Introduction

This pCR intends to capture the RAN3 agreements in RAN3#131 on 6G RAN-CN SBI options.

# 2	pCR to TR 38.760-3

##### 6.1.3.1.2 Service Based Interface (SBI)

A RAN-CN SBI (service-based interface) refers to application layer communication between the 6G RAN node and the CN entity for 6G by means of services provided/exposed by either the 6G RAN node or the CN entity for 6G.

The 6G RAN node may be configured with the profile of the CN entity for 6G, e.g., via OAM.

In one option, the profile of the CN entity for 6G includes an FQDN or IP address,  Based on this FQDN or IP address, the 6G RAN node directly communicates with the CN NF for 6G.

In another option, the profile of the CN entity for 6G includes parameters related to discovery and selection of an CN entity instance.  The 6G RAN node sends these parameters to a Service Communication Proxy (SCP) in an initial request message.  Based on these parameters, the SCP discovers an appropriate CN entity instance. The 6G RAN node then communicates indirectly with the CN entity through the SCP. The SCP relayes messages between the 6G RAN node and CN entity instance. . The SCP can serve for security isolation and message aggregation between 6G RAN and CN for 6G.

Editor’s Note 1: FFS whether multiple CN entities can be involved.

Potential options for the 6G SBI protocol stack are as follows:

Editor's Note 2:	Other options are not precluded.

-	Option 1: TCP+ HTTP/2 based SBI:

For TCP+HTTP/2 based SBI, it uses TCP and HTTP/2 protocol as already supported in 5GC.

Figure 6.1.3.1.2-1: SBI - Opt1: TCP+HTTP/2 based SBI Protocol Stack

-	Option 2: QUIC+HTTP/3 based SBI

For QUIC+HTTP/3 based SBI, it uses QUIC and HTTP/3 protocol in 6G.

Figure 6.1.3.1.2-2: SBI - Opt2: QUIC+HTTP/3 based SBI Protocol Stack

---------------End of the Change------------------