3GPP TSG-RAN WG3 #131	R3-260662

Goteborg, Sweden, 9 – 13 Feb. 2026

Agenda Item:	10.4.1. Higher Layer Split (HLS)

Source:	NTT DOCOMO INC. (moderator)

Title:	Summary of offline discussion on higher layer split in 6G

Document for:	Approval

Introduction

CB: # 19_6G_HLS

- Revise 0633 to resolve comments e.g., by simplifying the text, adding notes where needed, etc. Move bullet #4 into a single sentence e.g. “The standardized F1 interface enables operators to select CU and DU components from multiple vendors.”

- Revise 0482, clarifying the meaning of “traffic handling” in the first bullet, and simplify/clarify the newly added bullet (remove “Inter vendor interoperability complexity” and focus on technical issues)

- Perhaps a general sentence can be added that not all benefits/challenges are seen in all deployments.

(moderator - Docomo)

Summary of offline disc R3-260662

For the Chairman’s Notes

Discussion on TP

## 6.2.2	Disaggregated RAN Architecture

RAN3 acknowledges that there are benefits of HLS:

-	CU centralization and resource pooling: HLS architecture enables CU and DU functions to be deployed on cloud infrastructure. This enables pooling of compute resources across cells. CP pooling may be achieved in some deployments.

-	Elastic scalability: CU and DU capacity can be scaled independently, reducing the need for per‑site over‑provisioning and supporting incremental addition of cells or small‑cell deployments.

-	Support for diverse deployment scenarios: HLS architecture supports flexible placement of DU, CU‑UP, and CU‑CP at cell sites, edge locations, or central data centers according to latency, capacity, and cost requirements. This enables adaptation to different traffic patterns and service requirements.

The standardized F1, E1 interface enable operators to select CU (CU-CP, CU-UP) and DU components from multiple vendors.

The main areas of study (as a starting point) RAN3 is going to address within this study item for HLS are:

-	UE context handling and control plane signalling handling between CU and DU which has impact on the service latency and UE experience

-	F1-U interface optimizations (e.g. flow control)

-	Limited CU–DU capability exchange

In 5G, at least 18 Technical Specifications are created and maintained to finalize the split architecture.

Conclusions (agreed TP)

TBD

References

- R3-260633, “(pCR to TR 38.760-3) Benefits of the disaggregated RAN architecture”, Rakuten Mobile, Inc, Qualcomm, Samsung, NTT DOCOMO, Verizon, KDDI, Mavenir, Fujitsu, Jio Platforms

- R3-260482, “Challenges and benefits of Higher Layer Split (HLS)”, Ericsson