3GPP TSG-RAN WG3 #131	R3-260662

Goteborg, Sweden, 9 – 13 Feb. 2026

Agenda Item:	10.4.1. Higher Layer Split (HLS)

Source:	NTT DOCOMO INC. (moderator)

Title:	Summary of offline discussion on higher layer split in 6G

Document for:	Approval

# Introduction

CB: # 19_6G_HLS

- Revise 0633 to resolve comments e.g., by simplifying the text, adding notes where needed, etc. Move bullet #4 into a single sentence e.g. “The standardized F1 interface enables operators to select CU and DU components from multiple vendors.”

- Revise 0482, clarifying the meaning of “traffic handling” in the first bullet, and simplify/clarify the newly added bullet (remove “Inter vendor interoperability complexity” and focus on technical issues)

- Perhaps a general sentence can be added that not all benefits/challenges are seen in all deployments.

(moderator - Docomo)

Summary of offline disc R3-260662

# For the Chairman’s Notes

# Discussion on TP

## 6.2.2	Disaggregated RAN Architecture

RAN3 acknowledges that, depending on deployment scenarios and availability of resources, there are benefits of disaggregated RAN:

-	CU centralization and resource pooling: Multiple DUs may be connected to a common CU, allowing non-delay critical processing to be centralized at regional or central sites. This enables pooling of compute resources across cells. CP pooling may be achieved in some deployments.

-	Elastic scalability: CU and DU capacity can be scaled independently.

-	Support for diverse deployment scenarios: HLS architecture supports flexible placement of DU, CU‑UP, and CU‑CP at cell sites, edge locations, or central data centers depending on latency, capacity, and cost requirements.

A standardised disaggregated RAN enables operators to select CU and DU components from multiple vendors.

The main areas of study (as a starting point) RAN3 is going to address within this study item for HLS are:

-	UE context handling and traffic handling (e.g. UP configuration via RAN internal interfaces) between CU and DU which has impact on the service latency and UE experience

-	F1-U interface optimizations (e.g. flow control)

-	Limited and streamlined CU–DU capability exchange as well as dependencies (regarding e.g. scheduling and resource allocation, beam management, radio resource management, …) for both basic and advanced features. This includes reduced coordination between CU and DU.

In 5G, at least 18 Technical Specifications are created and maintained to finalize the split architecture.

# Conclusions (agreed TP)

TBD

# References

- R3-260633, “(pCR to TR 38.760-3) Benefits of the disaggregated RAN architecture”, Rakuten Mobile, Inc, Qualcomm, Samsung, NTT DOCOMO, Verizon, KDDI, Mavenir, Fujitsu, Jio Platforms

- R3-260482, “Challenges and benefits of Higher Layer Split (HLS)”, Ericsson