3GPP RAN WG3 Meeting #131	R3-26xxxx

Gothenburg, Sweden, 9th – 13th February 2025

Source:	CATT,Offino

Title:	TP on interfaces between 6G RAN nodes

Agenda Item:	10.4.2

Document for:	Discussion and decision

- Introduction

This document provides TP on principles and general functions of inter-6G-RAN-node interface.

- pCR for TR 38.760-3

## 6.2	RAN Internal Interfaces

Editor’s Note: This chapter shall comprehensively address study on CU-DU and CP-UP aspects including functional split considering interim milestone TSG#112 (June/2026) to take a decision during TSG#115: March 2027

Editor’s Note: This chapter should also address the interfaces between 6G RAN nodes (equivalent to Xn).

6.2.1	Interface between 6G RAN nodes

#### 6.2.1.1	General Principles

Editor’s note: The aim of this section is to describe general design principles and requirements for RAN Internal Interfaces

The general principles for the specification of the 6G RAN-RAN interface are as follows:

-	the interface between 6G RAN nodes supports the exchange of signalling information between 6G RAN nodes, and the forwarding of user data to the respective tunnel endpoints;

-	the interface between 6G RAN nodes supports control plane and user plane separation;

-	the interface between 6G RAN nodes supports future enhancements;

-	the interface between 6G RAN nodes supports all possible RAN deployment scenarios;

-	the control plane interface between 6G RAN nodes supports reliable signalling transmission;

-	the interface between 6G RAN nodes is a point-to-point interface, from logical point of view.

Editor’s note: Additional principles maybe added e.g. based on the RAN-CN interface discussion

#### 6.2.1.2	Functions

The interface between 6G RAN nodes supports the following functions:

-	UE mobility management: function to manage the UE mobility between 6G RAN nodes, specifically respect for mobility between two 6NBs, intra-system inter-RAT mobility between 5G and 6G, and inter-system mobility between 5G and 6G.

-	Data forwarding.

-	Interface Management and error handling.