3GPP TSG-RAN WG3 Meeting #131	R3-26XXXX

Gothenburg, Sweden, 09 - 13 February, 2026

Agenda Item:	13.3

Source:	China Telecom (moderator)

Title:	Summary of Offline Discussion for CB#22 ISAC procedures

Document for:	Discussion

1	Introduction

This SoD is based on the following online discussion：

The SF should be able to modify an ongoing sensing operation

gNB decides TRP(s) for sensing.

CB: # 22_ISAC procedures

- Further discuss gNB selection information (Q1 in SA2 LS) and sensing request parameters (Q4 in SA2 LS)

- Further discuss whether event-based reporting should be supported (Q3 in SA2 LS)

- Progress other Editor’s Notes in the TR, if time allows

(moderator - China Telecom)

2	For the Chair Notes

Editor’s Note: For Rel-20 study/work items, please consider that when agreements/FFSes are captured in a TP, additional inclusion in the Chair Notes may be unnecessary (particularly for stage 3 details).

Propose the following:

- TP to TR38.765 for basic procedures and signalling - XXX (R3-26XXXX) if any

Propose to capture the following in Chair Notes:

To be continued:

TBD

3	Discussion on RAN-CN procedures and signaling (13.3)

3.1	Capture online agreements

The guidances and agreements made online were listed below.

gNB decides TRP(s) for sensing. The SF should be able to modify an ongoing sensing operation - Further discuss gNB selection information (Q1 in SA2 LS) and sensing request parameters (Q4 in SA2 LS) - Further discuss whether event-based reporting should be supported (Q3 in SA2 LS) - Progress other Editor’s Notes in the TR, if time allows

3.2	ISSUE#1: gNB selection information (Q1 in SA2 LS)

Based on the contributions and online discussion, the following gNB information in addition to the sensing area can be discussed:

- Cell load of gNB?

- Sensing physical layer resources (e.g. sensing frequency and bandwidth)?

- Supported reporting periodicity?

Moderator summary:

The additional gNB includes….?

No consensus on the the additional gNB information?

Contribution summary (not in an exhaustive way)

| Information | Support companies |
| --- | --- |
| Supported Sensing Area | Xiaomi: (e.g., one or more Sensing Area ID(s)), Tejas Networks, CATT (e.g., a list of sensing area ID) Ercisson(a set of points corresponding to corners of a polygon with optional altitude information), Lenovo |
| Supported sensing measurement level if multiple formats are defined. | OPPO: (e.g., Level-A/B/C/D), InterDigital, TNO, KPN N.V., ITRI, Lenovo |
| Supported reporting periodicity | Ericsson, Jio Platforms, China Unicom, ZTE, Lenovo |
| Sensing resources status | Tejas Networks (e.g., specific spectrum allocations or uplink/downlink frequency information.), Sony (e.g. the bandwidth of the sensing reference signal, available beams), KPN N.V., TNO (network load), Lenovo |
| Supported sensing mode | Tejas Networks (e.g., gNB-based monostatic sensing (as applicable in Release-20).) |

3.3	ISSUE#2: Sensing request parameters (Q4 in SA2 LS)

The following information was discussed online without conclusion.

- Target Assistance Information (e.g., expected target type/path)?

- Sensing performance requirements (details FFS)?

- measurement level?

- Target sensing duration?

- Reporting Mode to support event-based triggers (e.g., UAV entering an area): see issue#3

Comments:

Moderator summary:

The additional sensing request parameters includes….?

No consensus on the rest information?

Contribution summary (not in an exhaustive way)

| Information | Support companies |
| --- | --- |
| Sensing Measurement ID that is defined as an identifier which uniquely identifies a sensing session initiated by a SF. | HW, QC, Xiaomi, Hanbat, Lenovo, CATT |
| Target sensing area | Hanbat, Tejas Networks, Lenovo, CATT |
| Report mode (one time, periodic) | Xiaomi: To support event-based triggers, ZTE, NEC QC: (e.g., response time, periodic interval). OPPO: (event-triggered and/or semi-persistent reporting mode, FFS) Hanbat, Tejas Networks, Sony |
| Sensing Duration | Hanbat (e.g., duration in milliseconds), ZTE, NEC, CATT, Ericsson |
| Sensing Service type | Xiaomi (e.g., Object Detection, Object Tracking) QC (e.g., object detection, object entry/exit in a defined area, etc.). OPPO (e.g., UAV detection, UAV tracking) NEC, Lenovo, CATT, Ericsson |
| Sensing related QoS | Xiaomi: Sensing performance requirements (e.g., Detection Probability, Maximum False Alarm Rate) for level D at least QC: Performance Parameters (e.g., accuracy). OPPO: (e.g., accuracy of positioning estimate for horizontal/vertical direction, accuracy of velocity estimate, missed detection, and false alarm). Hanbat: (e.g., Horizontal/Vertical accuracy, Detection probability) NEC, Lenovo, CATT, Ericsson (Desired accuracy of velocity/position metrics) |
| Sensing data reporting level, if multiple formats are defined. | QC: (e.g., measurements, point-(cloud), sensing result) OPPO: (e.g., level A/B/C/D) InterDigital, Sony, TNO, KPN N.V., ITRI |
| Sensing target related information | NEC (Trajectory assistance information), Lenovo (object description), Sony (sensing assistance information), CATT(sensing target type), Ericsson(Object type), Xiaomi |
| SF ID if AMF forwards the sensing request message | CATT |

3.2	ISSUE#3: Whether event-based reporting is supported (Q3 in SA2 LS)

We may first discuss the use case of the event-based report, then discuss whether the event-based reporting is needed or not.

Comments:

Moderator summary:

TBD

3.2	ISSUE#4: Progress other Editor’s Notes in the TR if time allows

- Sensing abort

Copied from the Chairman notes The SF-initiated Sensing abort procedure shall be defined as a Class 2 procedure. LGE, NEC: class 1 is needed What information would be needed in a Response?

Proposal: The SF-initiated Sensing abort procedure is defined as a Class 2 procedure?

Moderator summary:

The above proposal is agreed?

Contribution summary (not in an exhaustive way)

| Options | Support companies |
| --- | --- |
| Class 1 | Hanabt, NEC, Lenovo, LGE |
| Class 2 | Xiaomi, China Telecom, HW, QC, BUPT, ZTE, InterDigital, NOKIA, CATT, Ericsson |

- Sensing modification/update procedure

Copied from the Chairman notes

The SF should be able to modify an ongoing sensing operation

It was agreed that SF should be able to modify an ongoing sensing operation, the moderator intends to move a further step, and has the following proposal.

Proposal: ?? Introduce a new Sensing Modification procedure (including Request/Response/Failure message), where the sensing measurement ID, sensing area, and the reporting mode (e.g., one time, periodic) may be included in the sensing modification request message.

Proposal: ??if the above proposal is agreed, rename the sensing request/response message as sensing initiation request/response message.

Moderator summary:

The above proposal is agreed??

Contribution summary (not in an exhaustive way)

| Options | Support companies |
| --- | --- |
| Reuse sensing request/abort | OPPO, China Telecom, ZTE |
| Introduce a new Class-1 procedure | Xiaomi, HW, QC, Tejas Networks, NEC, InterDigital, NOKIA, Canon, TNO, KPN N.V., ITRI |
| Introduce a new Class-2 procedure | Ericsson? |

- The definition of sensing measurement ID and target sensing area

Editor’s Note 3: FFS on the definition of sensing measurement ID and the target sensing area.

The sensing measurement ID: Defined as an identifier which uniquely identifies a sensing operation initiated by a SF.

Sensing Area: Defined as the area where sensing can be performed in the 3D space by the gNB.

Moderator summary:

The above definitions are agreed?

3.1 Terms For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1]. example: text used to clarify abstract rules by applying them literally. Sensing Function: as defined in TR 23.700-14 [x1].

- Sensing results in the sensing response message

Editor’s Note 5: FFS whether the sensing results can be included in the Sensing Response message.

Could we remove the editor’s note since the sensing signaling and sensing report would have different endpoints as agreed in SA2?

Moderator summary:

The above proposal is agreed?

- Other proposals to be discussed if time allowed

Proposal: One or more sensing measurement IDs are also included in the Sensnig Request, Sensing Response, Sensing Abort, Sensing Failure Indication messages.

Proposal: Include the endpoint address for sensing report in the sensing request/sensing modification request or other messages?

Moderator summary:

6	Open issues for next meeting (To be updated)

TBD

Reference

| 13. Study on Integrated Sensing And Communication (ISAC) for NR SID [FS_Sensing_NR_bis]: RP-253246 (target: RAN #112) [TU: 1 (1, 1, 1)] QUOTA: 2 |  |  |
| --- | --- | --- |
| 13.1. General Work plan, BL pCR to TR38.765 |  |  |
| R3-260028 | (BL pCR to TR 38.765) Study on Integrated Sensing and Communication (ISAC) for NR (China Telecom, Xiaomi) | pCR |
| R3-260331 | Work plan for study on Integrated Sensing And Communication (ISAC) for NR (China Telecom, Xiaomi) | Work Plan |
| 13.2. Network architecture Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. From RAN3#130: Focus on the following candidate protocols for sensing data transmission, based on RAN1 and SA2 progress, considering the transmission requirements (e.g., data volumes, efficiency).: - SCTP-based - GTP-U-based -Websocket-based (Streaming based data reporting for trace and MDT, refer to TS 28.532) Coordination with SA2 is needed. |  |  |
| R3-260011 | LS on Sensing aspects related to RAN coordination (SA2(OPPO)) | LS in |
| R3-260047 | (TP to pCR 38.765) ISAC architecture and protocol (Xiaomi) | other |
| R3-260049 | (TP to BL pCR) Network architecture for ISAC (Huawei) | other |
| R3-260057 | (TP to pCR 38.765) Network Architecture and Protocol Aspects for NR Sensing Support (Qualcomm Incorporated) | other |
| R3-260092 | [DRAFT] Reply LS on Sensing aspects related to RAN coordination (OPPO) | LS out To: SA2 CC: RAN1, RAN2 |
| R3-260093 | (TP to BL pCR) Discussion on RAN architecture for sensing (OPPO) | other |
| R3-260134 | Network architecture enhancements for NR ISAC (Hanbat National University) | discussion |
| R3-260137 | (TP to pCR 38.765) Network Architecture and Protocol stack for NR ISAC (Tejas Network Limited) | other |
| R3-260144 | (TP to pCR 38.765) Further discussion on Network Architecture for ISAC (ZTE Corporation) | other |
| R3-260190 | (TP to BL pCR of TR38.765) The discussion on ISAC network architecture (NEC) | other |
| R3-260207 | NR ISAC Network Architecture (InterDigital Inc.) | other |
| R3-260282 | (TP to pCR for TR 38.765) Discussion on architecture for ISAC (Nokia) | other |
| R3-260328 | (TP to TR 38.765) Discussion on ISAC network architecture (Lenovo) | other |
| R3-260332 | (TP to 38.473) On potocal stack for ISAC (China Telecom) | other |
| R3-260371 | (TP to pCR 38.765) Network architecture for ISAC and draft Reply LS (CATT) | other |
| R3-260416 | ISAC Logical Architecture and Data Transmission (Ericsson, Jio Platforms) | discussion |
| R3-260426 | Discussion on the reply for LSin from SA2 (KPN N.V.) | discussion |
| R3-260474 | Discussion on Network Architecture for ISAC (AT&T) | discussion |
| R3-260507 | Discussions on the network architecture design of ISAC (China Unicom) | discussion |
| R3-260545 | Discussion on Network Architecture for ISAC (CMCC) | discussion |
| R3-260590 | Discussion on network architecture for ISAC (Samsung) | discussion |
| 13.3. RAN-CN procedures and signaling Study the procedures, signaling between RAN and CN to support ISAC. From RAN3#130: WA: gNB decides TRP(s) for sensing. Discuss interface connection setup if direct connectivity is agreed by SA2. Discuss the need of sensing modify/update procedure Discuss which procedure is used to transfer the gNB information (e.g., supported sensing area) from gNB to SF for gNB selection, if signalling approach is needed. |  |  |
| R3-260048 | (TP to pCR 38.765) RAN-CN procedure and signalling for ISAC (Xiaomi) | other |
| R3-260050 | (TP to BL pCR) RAN-CN procedures and signaling for ISAC and reply LS to SA2 (Huawei, Deutsche Telekom) | other |
| R3-260058 | (TP to pCR 38.765) Signalling and Procedures for NR Sensing Support (Qualcomm Incorporated) | other |
| R3-260094 | Discussion on RAN-CN procedures and signalling for sensing (OPPO) | discussion |
| R3-260135 | RAN-CN procedures and signaling for ISAC (Hanbat National University) | discussion |
| R3-260138 | (TP to pCR 38.765) RAN-CN procedures and signaling for NR ISAC (Tejas Network Limited) | other |
| R3-260145 | (TP to pCR 38.765) Further discussion on RAN-CN Procedures and Signalling for ISAC (ZTE Corporation) | other |
| R3-260191 | (TP to BL pCR of TR38.765)The discussion on ISAC RAN-CN procedure and signaling (NEC) | other |
| R3-260208 | NR ISAC RAN-CN Procedures and Signaling (InterDigital Inc.) | other |
| R3-260283 | (TP to pCR for TR 38.765) Discussion on RAN-CN procedures and signaling for ISAC (Nokia) | other |
| R3-260329 | (TP to TR 38.765) Discussion on general procedures for gNB-based sensing (Lenovo) | other |
| R3-260333 | On RAN-CN procedures for supporting ISAC (China Telecom, BUPT) | discussion |
| R3-260347 | Discussion for the support of ISAC (CANON Research Centre France) | discussion |
| R3-260349 | Discussion on sensing aspects related to RAN coordination (Sony) | discussion |
| R3-260372 | (TP to pCR 38.765) RAN-CN procedures and signaling for ISAC (CATT) | other |
| R3-260427 | Discussion on RAN-CN procedures and signaling for ISAC (TNO, KPN N.V., ITRI) | discussion |
| R3-260435 | (TP to pCR) RAN-CN procedures and signaling for ISAC (KPN N.V., TNO) | other |
| R3-260447 | Discussion on RAN-CN ISAC procedures and signalling content with reply LS and TP (Ericsson, Jio Platforms, China Unicom) | other |
| R3-260475 | Discussion on RAN-CN Procedures and Signaling for ISAC (AT&T) | discussion |
| R3-260504 | Discussions for ISAC RAN-CN aspects (LG Electronics Inc.) | discussion |
| R3-260505 | (TP to BL pCR to TR 38.765) ISAC RAN-CN aspects (LG Electronics Inc.) | other |
| R3-260508 | Standard impacts of RAN-CN procedures for ISAC (China Unicom) | discussion |
| R3-260522 | Discussion on RAN-CN procedures and signaling for ISAC (Google) | discussion |
| R3-260538 | Discussion on RAN-CN Procedures Supporting ISAC (CMCC) | discussion |
| R3-260591 | Discussion on RAN-CN procedures and signaling (Samsung) | discussion |