3GPP TSG-RAN WG3 #131	R3-26xxxx

Goteborg, Sweden, 9 – 13 Feb. 2026

Agenda Item:	8.1. New Incoming LSs

Source:	NTT DOCOMO INC.

Title:	Offline discussion on NR to LTE mobility restriction

Document for:	Discussion, approval

Introduction

RAN3 received LS from SA2 [S2-2511202] and they specified NR to LTE mobility restriction function in release 20 specification [S2-2509537], therefore RAN3 needs to specify necessary enhancements.

Following were captured during online session:

| R3-260166 | Enhancement of NR to LTE mobility restriction [Enhanced Mobility] (NTTDOCOMO, INC., Nokia, ZTE) | CR1418r, TS 38.413 v19.1.0, Rel-20, Cat. F HW, E///: on cover page, CR category should be B, and work item code should be TEI20_NRLTEREST Rev in R3-260643 CB: # 2_NRtoLTEmobrest - check CR details - WI code should be TEI20_NRLTEREST, Cat-B - align XnAP CR with all NGAP changes - Reply LS to SA2 informing that CR has been endorsed? (moderator - DCM) |
| --- | --- | --- |
| R3-260213 | Enhancement of NR to LTE mobility restriction [Enhanced Mobility] (Nokia, NTTDOCOMO, INC.) | CR1676r, TS 38.423 v19.1.0, Rel-20, Cat. F Rev in R3-260644 |

Reply LS

RAN3 received LS from SA2 [S2-2511202] and reply LS is needed. A draft LS in annex of R3-260165 would be a baseline.

This LS would be drafted directly on the FTP server:

http://10.10.10.10/ftp/RAN/RAN3/Inbox/Drafts/CB%20%23%202_NRtoLTEmobrest/draft_R3-26xxxx_Reply%20LS%20for%20NR%20to%20LTE%20mobility%20restriction_v1.docx

CR for 38.413

R3-20166(DCM) should be a baseline which introduces Forbidden Area Information EPS. Following points need to be discussed.

| Forbidden Area Information EPS |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Forbidden Area information as specified in TS 23.501 [9]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >Forbidden EPS TACs |  | 1..<maxnoofForbTACs> |  |  | - |  |
| >>EPS TAC | M |  | 9.3.3.16 | The EPS TAC of the forbidden EPS TAI. | - |  |

1) Do we need to change legacy IE?

R3-260232(HW) proposes to change legacy IE, i.e. add “for 5GS” to Forbidden Area Information IE.

| Forbidden Area Information for 5GS |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Forbidden Area information for 5GS as specified in TS 23.501 [9]. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

| Company | View (Y/N) | Comments |
| --- | --- | --- |

Conclusion: TBD

2) Which criticality should be adopted, ignore or reject?

R3-260414(E///) proposes to set the criticality of Forbidden Area Information EPS as “reject”, while others propose “ignore”.

| Forbidden EPS Area Information |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Forbidden EPS Area information as specified in TS 23.501 [9]. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >Forbidden EPS TACs |  | 1..<maxnoofForbEPSTACs> |  |  | - |  |
| >>EPS TAC | M |  | 9.3.3.16 | The TAC of the forbidden EPS TAI. | - |  |

| Company | View (ignore/reject) | Comments |
| --- | --- | --- |

Conclusion: TBD

3) Do we need to introduce new range bound for the number of Forbidden EPS TACs?

R3-260414(E///) also proposes to introduce new range bound “maxnoofForbEPSTACs” for the maximum number of Forbidden EPS TACs.

| Range bound | Explanation |
| --- | --- |
| maxnoofEPLMNs | Maximum no. of equivalent PLMNs. Value is 15. |
| maxnoofEPLMNsPlusOne | Maximum no. of allowed PLMNs. Value is 16. |
| maxnoofForbTACs | Maximum no. of forbidden Tracking Area Codes. Value is 4096. |
| maxnoofForbEPSTACs | Maximum no. of forbidden EPS Tracking Area Codes. Value is 4096. |
| maxnoofAllowedAreas | Maximum no. of allowed or not allowed Tracking Areas. Value is 16. |

| Company | View (Y/N) | Comments |
| --- | --- | --- |

Conclusion: TBD

CR for 38.423

R3-260213(Nok) should be a baseline which introduces Forbidden Area Information EPS same as NGAP. Following points need to be discussed.

| Forbidden Area Information EPS |  | 0..<maxnoofPLMNs> |  | This IE contains Forbidden Area information as specified in TS 23.501 [9]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >PLMN Identity | M |  | 9.2.2.4 |  | - |  |
| >Forbidden EPS TACs |  | 1..<maxnoofForbiddenTACs> |  |  | - |  |
| >>EPS TAC | M |  | OCTET STRING (2) | The EPS TAC of the forbidden EPS TAI. | - |  |

R3-260233(HW) proposes to change legacy IE, i.e. add “for 5GS” to Forbidden Area Information IE. This point should align with the conclusion of question 1).

| Forbidden Area Information for 5GS |  | 0..<maxnoofPLMNs> |  | This IE contains Forbidden Area information for 5GS as specified in TS 23.501 [7]. | – |  |
| --- | --- | --- | --- | --- | --- | --- |

4) Do we need to newly introduce/define EPS TAC IE?

R3-260233(HW) also proposes to newly define EPS TAC IE in section 9.2.2 and Forbidden Area Information for EPS refers to it.

- 9.2.2.aaa	EPS TAC

This information element is used to uniquely identify an EPS Tracking Area within a PLMN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TAC | M |  | OCTET STRING (SIZE (2)) |  |

| Company | View (Y/N) | Comments |
| --- | --- | --- |

Conclusion: TBD

Others

Please provide your comments if you have any other comments on any other issues.

| Company | Comments |
| --- | --- |

Conclusions

TBD

References

- S2-2511202

- S2-2509537

- R3-260165, DCM (DP)

- R3-260305, ZTE (draft LS)

- R3-260415, Ericsson (draft LS)

- R3-260166, DCM (CR for 38.413)

- R3-260232, Huawei (CR for 38.413)

- R3-260414, Ericsson (CR for 38.413)

- R3-260213, Nokia (CR for 38.423)

- R3-260233, Huawei (CR for 38.423)

- R3-260304, ZTE (CR for 38.300)

- R3-260489, Nokia (CR for 36.413)

- R3-260231, Huawei (DP)

- R3-260488, Nokia (DP)