3GPP TSG-RAN WG3 #131	R3-26xxxx

Goteborg, Sweden, 9 – 13 Feb. 2026

Agenda Item:	8.1. New Incoming LSs

Source:	NTT DOCOMO INC.

Title:	Offline discussion on NR to LTE mobility restriction

Document for:	Discussion, approval

Introduction

RAN3 received LS from SA2 [S2-2511202] and they specified NR to LTE mobility restriction function in release 20 specification [S2-2509537], therefore RAN3 needs to specify necessary enhancements.

Following were captured during online session:

| R3-260166 | Enhancement of NR to LTE mobility restriction [Enhanced Mobility] (NTTDOCOMO, INC., Nokia, ZTE) | CR1418r, TS 38.413 v19.1.0, Rel-20, Cat. F HW, E///: on cover page, CR category should be B, and work item code should be TEI20_NRLTEREST Rev in R3-260643 CB: # 2_NRtoLTEmobrest - check CR details - WI code should be TEI20_NRLTEREST, Cat-B - align XnAP CR with all NGAP changes - Reply LS to SA2 informing that CR has been endorsed? (moderator - DCM) |
| --- | --- | --- |
| R3-260213 | Enhancement of NR to LTE mobility restriction [Enhanced Mobility] (Nokia, NTTDOCOMO, INC.) | CR1676r, TS 38.423 v19.1.0, Rel-20, Cat. F Rev in R3-260644 |

Reply LS

RAN3 received LS from SA2 [S2-2511202] and reply LS is needed. A draft LS in annex of R3-260165 would be a baseline.

This LS would be drafted directly on the FTP server:

http://10.10.10.10/ftp/RAN/RAN3/Inbox/Drafts/CB%20%23%202_NRtoLTEmobrest/draft_R3-26xxxx_Reply%20LS%20for%20NR%20to%20LTE%20mobility%20restriction_v1.docx

CR for 38.413

R3-20166(DCM) should be a baseline which introduces Forbidden Area Information EPS. Following points need to be discussed.

| Forbidden Area Information EPS |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Forbidden Area information as specified in TS 23.501 [9]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >Forbidden EPS TACs |  | 1..<maxnoofForbTACs> |  |  | - |  |
| >>EPS TAC | M |  | 9.3.3.16 | The EPS TAC of the forbidden EPS TAI. | - |  |

1) Do we need to change legacy IE?

R3-260232(HW) proposes to change legacy IE, i.e. add “for 5GS” to Forbidden Area Information IE.

| Forbidden Area Information for 5GS |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Forbidden Area information for 5GS as specified in TS 23.501 [9]. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

| Company | View (Y/N) | Comments |
| --- | --- | --- |
| Huawei | Slightly Yes | At least the “for 5GS” in the semantic descriptions is needed. Suggest to update the legacy IE name as “Forbidden Area Information 5GS”, and new IE name as “Forbidden Area Information EPS”. |

Conclusion: TBD

2) Which criticality should be adopted, ignore or reject?

R3-260414(E///) proposes to set the criticality of Forbidden Area Information EPS as “reject”, while others propose “ignore”.

| Forbidden EPS Area Information |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Forbidden EPS Area information as specified in TS 23.501 [9]. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >Forbidden EPS TACs |  | 1..<maxnoofForbEPSTACs> |  |  | - |  |
| >>EPS TAC | M |  | 9.3.3.16 | The TAC of the forbidden EPS TAI. | - |  |

| Company | View (ignore/reject) | Comments |
| --- | --- | --- |
| Huawei | Ignore | We don’t see the need to let CN be aware whether the NG-RAN supports the functionality. And it seems no further action for the CN. |

Conclusion: TBD

3) Do we need to introduce new range bound for the number of Forbidden EPS TACs?

R3-260414(E///) also proposes to introduce new range bound “maxnoofForbEPSTACs” for the maximum number of Forbidden EPS TACs.

| Range bound | Explanation |
| --- | --- |
| maxnoofEPLMNs | Maximum no. of equivalent PLMNs. Value is 15. |
| maxnoofEPLMNsPlusOne | Maximum no. of allowed PLMNs. Value is 16. |
| maxnoofForbTACs | Maximum no. of forbidden Tracking Area Codes. Value is 4096. |
| maxnoofForbEPSTACs | Maximum no. of forbidden EPS Tracking Area Codes. Value is 4096. |
| maxnoofAllowedAreas | Maximum no. of allowed or not allowed Tracking Areas. Value is 16. |

| Company | View (Y/N) | Comments |
| --- | --- | --- |
| Huawei | No |  |

Conclusion: TBD

CR for 38.423

R3-260213(Nok) should be a baseline which introduces Forbidden Area Information EPS same as NGAP. Following points need to be discussed.

| Forbidden Area Information EPS |  | 0..<maxnoofPLMNs> |  | This IE contains Forbidden Area information as specified in TS 23.501 [9]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >PLMN Identity | M |  | 9.2.2.4 |  | - |  |
| >Forbidden EPS TACs |  | 1..<maxnoofForbiddenTACs> |  |  | - |  |
| >>EPS TAC | M |  | OCTET STRING (2) | The EPS TAC of the forbidden EPS TAI. | - |  |

R3-260233(HW) proposes to change legacy IE, i.e. add “for 5GS” to Forbidden Area Information IE. This point should align with the conclusion of question 1).

| Forbidden Area Information for 5GS |  | 0..<maxnoofPLMNs> |  | This IE contains Forbidden Area information for 5GS as specified in TS 23.501 [7]. | – |  |
| --- | --- | --- | --- | --- | --- | --- |

4) Do we need to newly introduce/define EPS TAC IE?

R3-260233(HW) also proposes to newly define EPS TAC IE in section 9.2.2 and Forbidden Area Information for EPS refers to it.

- 9.2.2.aaa	EPS TAC

This information element is used to uniquely identify an EPS Tracking Area within a PLMN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TAC | M |  | OCTET STRING (SIZE (2)) |  |

| Company | View (Y/N) | Comments |
| --- | --- | --- |
| Huawei | No strong view | Both could work. |

Conclusion: TBD

Others

Please provide your comments if you have any other comments on any other issues.

| Company | Comments |
| --- | --- |

Conclusions

TBD

References

- S2-2511202

- S2-2509537

- R3-260165, DCM (DP)

- R3-260305, ZTE (draft LS)

- R3-260415, Ericsson (draft LS)

- R3-260166, DCM (CR for 38.413)

- R3-260232, Huawei (CR for 38.413)

- R3-260414, Ericsson (CR for 38.413)

- R3-260213, Nokia (CR for 38.423)

- R3-260233, Huawei (CR for 38.423)

- R3-260304, ZTE (CR for 38.300)

- R3-260489, Nokia (CR for 36.413)

- R3-260231, Huawei (DP)

- R3-260488, Nokia (DP)