3GPP TSG-RAN WG3 #131	R3-26xxxx

Goteborg, Sweden, 9 – 13 Feb. 2026

Agenda Item:	8.1. New Incoming LSs

Source:	NTT DOCOMO INC.

Title:	Offline discussion on NR to LTE mobility restriction

Document for:	Discussion, approval

Introduction

RAN3 received LS from SA2 [S2-2511202] and they specified NR to LTE mobility restriction function in release 20 specification [S2-2509537], therefore RAN3 needs to specify necessary enhancements.

Following were captured during online session:

| R3-260166 | Enhancement of NR to LTE mobility restriction [Enhanced Mobility] (NTTDOCOMO, INC., Nokia, ZTE) | CR1418r, TS 38.413 v19.1.0, Rel-20, Cat. F HW, E///: on cover page, CR category should be B, and work item code should be TEI20_NRLTEREST Rev in R3-260643 CB: # 2_NRtoLTEmobrest - check CR details - WI code should be TEI20_NRLTEREST, Cat-B - align XnAP CR with all NGAP changes - Reply LS to SA2 informing that CR has been endorsed? (moderator - DCM) |
| --- | --- | --- |
| R3-260213 | Enhancement of NR to LTE mobility restriction [Enhanced Mobility] (Nokia, NTTDOCOMO, INC.) | CR1676r, TS 38.423 v19.1.0, Rel-20, Cat. F Rev in R3-260644 |

Conclusions

- Update only the semantics of legacy Forbidden Area Information IE to clarify it is for forbidden 5GS area.

- Set the criticality of new Forbidden Area Information EPS as “ignore”.

- No new range bound for Forbidden EPS TACs is needed.

- No new IE definition for EPS TAC in XnAP is needed.

Reply LS

RAN3 received LS from SA2 [S2-2511202] and reply LS is needed. A draft LS in annex of R3-260165 would be a baseline.

This LS would be drafted directly on the FTP server:

http://10.10.10.10/ftp/RAN/RAN3/Inbox/Drafts/CB%20%23%202_NRtoLTEmobrest/draft_R3-26xxxx_Reply%20LS%20for%20NR%20to%20LTE%20mobility%20restriction_v1.docx

CR for 38.413

R3-20166(DCM) should be a baseline which introduces Forbidden Area Information EPS. Following points need to be discussed.

| Forbidden Area Information EPS |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Forbidden Area information as specified in TS 23.501 [9]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >Forbidden EPS TACs |  | 1..<maxnoofForbTACs> |  |  | - |  |
| >>EPS TAC | M |  | 9.3.3.16 | The EPS TAC of the forbidden EPS TAI. | - |  |

1) Do we need to change legacy IE?

R3-260232(HW) proposes to change legacy IE, i.e. add “for 5GS” to Forbidden Area Information IE.

| Forbidden Area Information for 5GS |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Forbidden Area information for 5GS as specified in TS 23.501 [9]. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

| Company | View (Y/N) | Comments |
| --- | --- | --- |
| Huawei | Slightly Yes | At least the “for 5GS” in the semantic descriptions is needed. Suggest to update the legacy IE name as “Forbidden Area Information 5GS”, and new IE name as “Forbidden Area Information EPS”. |
| CMCC | Yes, only semantic description | Suggest only update the semantic description to add “for 5GS”, no need to change the IE name which will introduce extra ASN.1 change. As we suggested in R3-260540. |
| Ericsson | No need | No need to change the legacy IE name; OK to update its semantics (“This IE contains Forbidden 5GC Area information…”). Then, it would be beneficial to clearly mark the new IE also in the semantics: “This IE contains Forbidden EPS Area information…”. Or something similar. |
| Nokia | No need | Ok for semantics update |
| ZTE | No need | Ok for semantics update |
| LGE | No need | Ok for semantics update |
| Samsung | No need |  |

Conclusion: Update only the semantics of legacy Forbidden Area Information IE to clarify it is for forbidden 5GS area.

2) Which criticality should be adopted, ignore or reject?

R3-260414(E///) proposes to set the criticality of Forbidden Area Information EPS as “reject”, while others propose “ignore”.

| Forbidden EPS Area Information |  | 0..<maxnoofEPLMNsPlusOne> |  | This IE contains Forbidden EPS Area information as specified in TS 23.501 [9]. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >Forbidden EPS TACs |  | 1..<maxnoofForbEPSTACs> |  |  | - |  |
| >>EPS TAC | M |  | 9.3.3.16 | The TAC of the forbidden EPS TAI. | - |  |

| Company | View (ignore/reject) | Comments |
| --- | --- | --- |
| Huawei | Ignore | We don’t see the need to let CN be aware whether the NG-RAN supports the functionality. And it seems no further action for the CN. |
| CMCC | Ignore | No further action CN needs. |
| Ericsson | Reject | The point is not for CN to be aware of what happens in RAN, but RAN behavior upon receiving this IE. According to NGAP Sec. 10, if this IE is received and not comprehended, the NG-RAN will ignore it and proceed as if the IE had not been sent. So, the UE will be allowed to connect to a forbidden EPS area. This negates the intention of adding this IE. |
| Nokia | reject |  |
| ZTE |  | Ericsson’s comments seem reasonable. |
| LGE | Reject | Agree with Ericsson’s comment |
| Samsung | Ignore | If “ignore” is used, the RAN node which cannot understand this IE will trigger the handover from 5G to 4G, and the handover can be rejected by the CN. If “reject” is used, CN node cannot do anything to resolve this. So, the RAN node will also initiate the handover from 5G to 4G, and the handover can be rejected by CN. So, it seems that “reject” or “ignore” cannot change the behavior of RAN. Then, “ignore” can ensure that other IEs in Mobility restriction list can be used by the RAN node. |

Conclusion: set the criticality of new Forbidden Area Information EPS as “ignore”.

3) Do we need to introduce new range bound for the number of Forbidden EPS TACs?

R3-260414(E///) also proposes to introduce new range bound “maxnoofForbEPSTACs” for the maximum number of Forbidden EPS TACs.

| Range bound | Explanation |
| --- | --- |
| maxnoofEPLMNs | Maximum no. of equivalent PLMNs. Value is 15. |
| maxnoofEPLMNsPlusOne | Maximum no. of allowed PLMNs. Value is 16. |
| maxnoofForbTACs | Maximum no. of forbidden Tracking Area Codes. Value is 4096. |
| maxnoofForbEPSTACs | Maximum no. of forbidden EPS Tracking Area Codes. Value is 4096. |
| maxnoofAllowedAreas | Maximum no. of allowed or not allowed Tracking Areas. Value is 16. |

| Company | View (Y/N) | Comments |
| --- | --- | --- |
| Huawei | No |  |
| CMCC | No |  |
| Ericsson | Yes | But no strong view; maxnoofForbTACs would also work. |
| Nokia | No |  |
| ZTE | No |  |
| LGE | No |  |
| Samsung | No |  |

Conclusion: No new range bound for Forbidden EPS TACs is needed.

CR for 38.423

R3-260213(Nok) should be a baseline which introduces Forbidden Area Information EPS same as NGAP. Following points need to be discussed. The criticality should align with NGAP.

| Forbidden Area Information EPS |  | 0..<maxnoofPLMNs> |  | This IE contains Forbidden Area information as specified in TS 23.501 [9]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >PLMN Identity | M |  | 9.2.2.4 |  | - |  |
| >Forbidden EPS TACs |  | 1..<maxnoofForbiddenTACs> |  |  | - |  |
| >>EPS TAC | M |  | OCTET STRING (2) | The EPS TAC of the forbidden EPS TAI. | - |  |

R3-260233(HW) proposes to change legacy IE, i.e. add “for 5GS” to Forbidden Area Information IE. This point should align with the conclusion of question 1).

| Forbidden Area Information for 5GS |  | 0..<maxnoofPLMNs> |  | This IE contains Forbidden Area information for 5GS as specified in TS 23.501 [7]. | – |  |
| --- | --- | --- | --- | --- | --- | --- |

4) Do we need to newly introduce/define EPS TAC IE?

R3-260233(HW) also proposes to newly define EPS TAC IE in section 9.2.2 and Forbidden Area Information for EPS refers to it.

- 9.2.2.aaa	EPS TAC

This information element is used to uniquely identify an EPS Tracking Area within a PLMN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TAC | M |  | OCTET STRING (SIZE (2)) |  |

| Company | View (Y/N) | Comments |
| --- | --- | --- |
| Huawei | No strong view | Both could work. |
| CMCC | No |  |
| Ericsson | No strong view | Considering EPS TAC IE is non-extendable, there is not much difference. |
| Nokia | No |  |
| ZTE | No |  |
| LGE | No |  |
| Samsung | No |  |

Conclusion: No new IE definition for EPS TAC in XnAP is needed.

Others

Please provide your comments if you have any other comments on any other issues.

| Company | Comments |
| --- | --- |

Conclusions

- Update only the semantics of legacy Forbidden Area Information IE to clarify it is for forbidden 5GS area.

- Set the criticality of new Forbidden Area Information EPS as “ignore”.

- No new range bound for Forbidden EPS TACs is needed.

- No new IE definition for EPS TAC in XnAP is needed.

References

- S2-2511202

- S2-2509537

- R3-260165, DCM (DP)

- R3-260305, ZTE (draft LS)

- R3-260415, Ericsson (draft LS)

- R3-260166, DCM (CR for 38.413)

- R3-260232, Huawei (CR for 38.413)

- R3-260414, Ericsson (CR for 38.413)

- R3-260213, Nokia (CR for 38.423)

- R3-260233, Huawei (CR for 38.423)

- R3-260304, ZTE (CR for 38.300)

- R3-260489, Nokia (CR for 36.413)

- R3-260231, Huawei (DP)

- R3-260488, Nokia (DP)