# 3GPP TSG RAN WG3#131																			R3-26xxxx

# Goteborg, Sweden, 9 - 13, February 2026

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0526 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on WAB-gNB configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Nokia, China Telecomm, Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2026-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In case the PLMN/SNPN broadcast by WAB-gNB is different from the BH PLMN, or when the WAB-MT is using the NTN access technology, WAB-gNB should formulate the Additional ULI based on MWABs geo-location. Thus, the configuration to WAB-gNB on mapping of a WAB-node’s geo-location to the Additional ULI information is needed. However, this configuration has not been captured in any spec. - According to TS 23.501, the address of the OAM server can be configured to a WAB-gNB, and the OAM provides other parameters to WAB-gNB considering the location of WAB-gNB. However, it’s not clear in TS 38.401 that the configurations of a WAB-node are managed by the OAM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the description that WAB-gNB can be configured with mapping of the WAB-node’s geo-location to the Additional ULI. - Make clear that the configurations of a WAB-node are managed by OAM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lead to a defective spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.1, 12.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - Rev#1 updates the covepage. |  |  |  |  |  |  |  |  |

>>>>>>>>>>>> Start of changes <<<<<<<<<<<<

12.2	Configuration of WAB-node

12.2.1	General

The following configurations of a WAB-node may need to be updated as the WAB-node moves:

-	The parameters that enable the WAB-gNB to select and connect to the AMF(s) that serve the UE(s).

-	The parameters that enable the WAB-gNB to connect to, and communicate with, the OAM system.

-	The parameters that the WAB-gNB should broadcast, e.g., the TAC(s), the cell ID(s), the RANAC(s).

A WAB-node may be provisioned with the parameters pertinent to different potential locations of the WAB-node by the OAM. Alternatively, the OAM can provision configuration parameters to the WAB-node, based on the location of the node. In that case, the continuity of OAM connectivity needs to be ensured as the WAB-node moves.

>>>>>>>>>>>> Next Change <<<<<<<<<<<<

12.5	Additional User Location Information for WAB

For UEs served by a WAB-gNB, in addition to the User Location Information (ULI), the WAB-gNB also provides the core network with Additional ULI, which includes a TAI and a NR CGI pertinent to the UE’s serving PLMN/SNPN.

If the UE’s serving PLMN/SNPN is the same as the PLMN/SNPN serving the WAB-MT, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the Additional ULI for UEs served by the WAB-gNB includes the TAI and the NR CGI of the cell serving the WAB-MT.

If the PLMN/SNPN serving the WAB-MT is different from the UE’s serving PLMN/SNPN, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the Additional ULI for UEs served by the WAB-gNB is determined by the WAB-gNB, based on the WAB-node’s geo-location. The WAB-gNB can be configured with mapping of the WAB-node’s geo-location to the Additional ULI which includes a TAI and a NR CGI of the PLMN/SPN broadcasted by the WAB-gNB.

If the WAB-MT connects to the BH-gNB by means of a non-terrestrial link, the Additional ULI for UEs served by WAB-gNB is determined by the WAB-gNB, based on WAB-node’s geo-location. This applies regardless of whether the PLMN/SNPN serving the WAB-MT is the same as, or different than, the UE’s serving PLMN/SNPN. The WAB-gNB can be configured with mapping of the WAB-node’s geo-location to the Additional ULI which includes a TAI and a NR CGI of the PLMN/SPN broadcasted by the WAB-gNB.

In case Additional ULI for UEs served by a WAB-gNB changes, e.g., due to WAB-node movement, the WAB-gNB derives the new Additional ULI and reports it via legacy procedures to the core network.

The WAB-gNB can indicate the Additional ULI to the core network via the NG Setup and RAN Configuration Update procedures.

>>>>>>>>>>>> End of changes <<<<<<<<<<<<