3GPP TSG-RAN WG3 Meeting #131                                                            R3-26xxxx

Goteborg, Sweden, 9th – 13th February 2026

Title:	Reply LS on MWAB-gNB Configurations

Response to:	S5-255704/R3-260022

Release:	Release 20

Work Item:	AdNRM_Ph4

Source:	RAN3

To:	SA5, SA2

Cc:

Contact person:	Mengzhen Wang

wang.mengzhen@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

Overall description

RAN3 thanks SA5 for the LS on MWAB-gNB configurations.

Regarding the following optional configurations for a WAB-gNB required from the OAM server, RAN3 has discussed and made the following conclusions.

…………………………………………………………………………………………………………………………

Specifically, besides the required configurations for a normal gNB, a MWAB-gNB would require the following optional configurations from the OAM server of the MWAB Broadcasted PLMN/SNPN:

- QoS related information for the BH PDU sessions (TS 23.501 clause 5.49.1.3);

- Mapping of the S-NSSAI(s) of the MWAB Broadcasted PLMN/SNPN to the traffic descriptor type of information for the BH PLMN (TS 23.501 clause 5.49.1.4);

- Different configuration parameter sets each associated to a different area (TS 23.501 clause 5.49.2.2);

- Pre-configured authorization information, e.g. location or time to turn on/shut down the MWAB operation (TS 23.501 clause 5.49.3.3);

- Mapping of a MWAB’s geo-location to the ULI information (TS 23.501 clause 5.49.4).

…………………………………………………………………………………………………………………………

For bullets #1 and #2, they are introduced and specified by SA2 and therefore should be dependent on SA2. But from RAN3’s perspective, the OAM of the WAB-gNB shall not configure the WAB-MT. In addition, for inter-PLMN scenario, it is not feasible for the OAM of the WAB-gNB to configure the WAB-MT.

For bullets #3, #4 and #5, RAN3 confirms that optional configurations are required from the OAM server, according to the corresponding normative text in TS 38.401, clauses 12.2.1, 12.4, and 12.5, respectively. For bullet #5, RAN3 would like to further clarify that it should be the “Mapping of a MWAB’s geo-location to the Additional ULI information” (the term “Additional” was missing in the original sentence).

Action

To SA5 and SA2:

ACTION: RAN3 kindly requests SA5 and SA2 to take the above information into account.

Dates of the next TSG RAN WG3 meetings

TSG RAN3 Meeting #131bis	13th – 17th  Apr, 2026	St. Julian’s, Malta

TSG RAN3 Meeting #132 	18th – 22th  May, 2026	Dalian, China