3GPP TSG-RAN WG3 Meeting #131	R3-260724

Gothenburg Metropolitan Area, SE, 9th – 13th February, 2026

Title: 	Further discussion on AI/ML assisted intra-CU LTM

Source: 	Huawei

Agenda item:	12.2.2

Document Type:	pCR

# 1. Introduction

This is a pCR to TR 38.745 capturing agreements reached at RAN3#131 on AI/ML-assisted intra-CU LTM.

# pCR for TR 38.745 (based on TR 38.745 V1.1.0 (2026-02))

<<<<<<<<<< Start of Changes >>>>>>>>>>

### 4.2.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

*** skipped unmodified text ***

#### 4.2.2.2	Input data of AI/ML based Intra-CU LTM

For AI/ML optimization of intra-CU LTM the following information can be considered as input data:

From local node:

-	L3 measurement results

-	UE mobility history

-	Measured/Predicted radio resource status per cell/SSB area

-	Measured/Predicted cell-based UE trajectory

-	Historical UE’s candidate LTM cell and beam list(s)

-	Measured TA values

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

<<<<<<<<<< Next Change >>>>>>>>>>

#### 4.2.2.3	Output data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as output data:

-	Predicted candidate cell and beam for LTM HO Preparation

-	Predicted target cell and beam for cell switch command

-	Predicted cell(s) and beam(s) for early UL synchronization

-	Predicted TA value(s) for early UL synchronization

-	Predicted time for cell switch for L3-based LTM

-	Predicted time for early UL synchronization for L3-based LTM

-	Predicted best beam of the 1st cell in the predicted UE trajectory

-	Predicted TA validity time

<<<<<<<<<< Next Change >>>>>>>>>>

#### 4.2.2.4	Feedback of AI/ML based Intra-CU LTM

The following data can be considered as feedback data for intra-CU LTM:

-	LTM target cell and beam for each cell switch event

-	Measured TA value(s)

-    Actual time of occurrence of cell switch

-	Actual time of occurrence of early synchronization

<<<<<<<<<< End of Changes >>>>>>>>>>