3GPP TSG-RAN WG3 Meeting #131	R3-260724

Gothenburg Metropolitan Area, SE, 9th – 13th February, 2026

Title: 	Further discussion on AI/ML assisted intra-CU LTM

Source: 	Huawei, Qualcomm, Ericsson, Nokia, ZTE, NEC, CATT, Samsung, 	CMCC, LG Electronics

Agenda item:	12.2.2

Document Type:	pCR

# 1. Introduction

This is a pCR to TR 38.745 capturing agreements reached at RAN3#131 on AI/ML-assisted intra-CU LTM.

# pCR for TR 38.745 (based on TR 38.745 V1.1.0 (2026-02))

<<<<<<<<<< Start of Changes >>>>>>>>>>

### 4.2.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

*** skipped unmodified text ***

#### 4.2.2.2	Input data of AI/ML based Intra-CU LTM

For AI/ML optimization of intra-CU LTM the following information can be considered as input data:

From local node:

-	L3 measurement results

-	UE history information

-	Measured/Predicted radio resource status per cell/SSB area

-	Measured/Predicted cell-based UE trajectory

-	Historical UE’s candidate LTM cell and beam list(s)

-	Measured TA values

From the UE:

-	UE measurement reports

-     Mobility history information

<<<<<<<<<< Next Change >>>>>>>>>>

#### 4.2.2.3	Output data of AI/ML based Intra-CU LTM

For AI/ML optimization of Intra-CU LTM the following information can be considered as output data:

-	Predicted candidate cell(s) and beam(s) for LTM HO Preparation

-	Predicted target cell and beam for cell switch command

-	Predicted cell(s) and beam(s) for early UL synchronization

-	Predicted TA value(s) for early UL synchronization

-	Predicted timing to trigger cell switch for L3 measurement based LTM

-	Predicted timing to trigger early UL synchronization for L3 measurement based LTM

-	Predicted best beam of the 1st cell in the predicted cell-based UE trajectory

-	Predicted TA validity time

NOTE: Applicability of the predicted TA validity time to a measured TA value and/or a predicted TA value will be assessed in the normative phase.

<<<<<<<<<< Next Change >>>>>>>>>>

#### 4.2.2.4	Feedback of AI/ML based Intra-CU LTM

The following data can be considered as feedback data for intra-CU LTM:

-	LTM target cell and beam for cell switch

-	Measured TA value(s)

-	SON Reports (RLF, SHR, etc.)

-    Timing of cell switch execution

-	Timing of early UL synchronization

<<<<<<<<<< Next Change >>>>>>>>>>

#### 4.2.2.5	Potential standard impacts

F1 impacts are expected to support transferring of the information listed in clause 4.2.2.2, clause 4.2.2.3 and clause 4.2.2.4, if applicable.

<<<<<<<<<< End of Changes >>>>>>>>>>