3GPP TSG-RAN WG3 Meeting #131	R3-260719

Gothenburg, Sweden, 9–13 February 2026

Agenda Item:	12.2.1

Source:	Ericsson, Jio Platform, ZTE, Huawei, NEC, Qualcomm, Samsung, China Telecom, CATT, FiberCop

Title: (pCR to TR 38.745) AIML based mobility – Multi-hop UE Trajectory

Document Type: pCR

# 4	Use cases and Solutions

<<<<<<<<<<<<<<<<<<<< Changes start here >>>>>>>>>>>>>>>>>>>>

## 4.1	Multi-hop UE trajectory

### 4.1.1	Use case description

In Rel-18, the cell-based UE trajectory prediction and measured cell-based UE trajectory are limited to the first-hop target NG-RAN node.

Predicted multi-hop UE trajectory across gNBs consists of a list of cells belonging to gNBs where the UE is expected to connect and these cells are listed in chronological order.

Measured multi-hop UE trajectory across gNBs consists of a list of cells belonging to gNBs where the UE was connected and these cells are listed in chronological order.

### 4.1.2	Solutions and standard impacts

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions are considered for supporting multi-hop UE trajectory:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2	Input data of multi-hop UE trajectory

To predict the multi-hop UE trajectory, a gNB may need the following information as input data:

From the UE:

-	UE measurement report related to serving cell and neighbouring cells, e.g., RSRP, RSRQ, SINR

-	Mobility History Information

From the neighbouring RAN nodes:

-	UE History Information

From the local node:

-	Collected measured multi-hop UE trajectory

#### 4.1.2.3	Output data of multi-hop UE trajectory

For multi-hop UE trajectory, the following information can be considered as output data:

-	Predicted multi-hop UE trajectory, including a list of cells that UE is expected to connect to in chronological order and the associated expected time UE stays in the cell

#### 4.1.2.4	Feedback of multi-hop UE trajectory

For multi-hop UE trajectory, the following information can be considered as feedback:

-	Measured UE Trajectory collected at each visited gNB

#### 4.1.2.5	Standard impact

Multi-hop UE trajectory prediction is transferred to the target NG-RAN nodes over the Xn interface via the Handover Preparation procedure. For the subsequent handovers, multi-hop predicted UE trajectory includes a list of cells where the UE is expected to connect to in chronological order.

For the subsequent hops, Data Collection Reporting Initiation procedure needs to be in place between the initial source gNB and a subsequent gNB to request the reporting of measured UE trajectory. A subsequent gNB receives, via the Handover Preparation procedure, information needed to then provide the measured UE trajectory. A subsequent gNB visited by the UE directly transmits the collected measured UE trajectory back to the initial source gNB over the Xn interface via the Data Collection Reporting procedure.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>