3GPP TSG-RAN WG3 Meeting #131	R3-260730

Gothenburg, Sweden, 09th ~ 13th Feb, 2026

Title:	(TP to 38.413 BL CR) UE Reader List related

Agenda Item:	14.2

Source:	Huawei, China Unicom, ZTE, LG Electronics, NEC, Nokia

Document Type:	Other

# Introduction

This paper provides the TP to reflect the following agreements/WA achieved during this meeting:

- Include UE Reader Report List in the Inventory Report Transfer IE.

- WA: Include UE Reader List in Requested Service Area Information IE in Inventory Request Transfer IE

# Text Proposal to TS 38.413 BLCR

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.20.2	Inventory Report

#### 8.20.2.1	General

The purpose of the Inventory Report procedure is to enable the NG-RAN node to provide inventory report related information to the A-IoT CN node, following a successful Inventory Request procedure. This procedure applies only if the NG-RAN node is a gNB.

#### 8.20.2.2	Successful Operation

Figure 8.20.2.2-1: Inventory Report

The NG-RAN node initiates the procedure by sending an INVENTORY REPORT message for the Inventory service.

If Reader Report List IE is included in the Inventory Report Transfer IE contained in the INVENTORY REPORT message, the A-IoT CN node shall take it into account as specified in TS 23.369 [60].

If UE Reader Report List IE is included in the Inventory Report Transfer IE contained in the INVENTORY REPORT message, the A-IoT CN node shall take it into account as specified in TS 23.369 [60].

If Inventory Complete Indication IE is included in the Inventory Report Transfer IE contained in the INVENTORY REPORT message, the A-IoT CN node shall consider that the inventory operation for the A-IoT session is completed in the NG-RAN node.

Interactions with the A-IoT Session Release procedure:

In case the Inventory Complete Indication IE is included in the Inventory Report Transfer IE contained in the INVENTORY REPORT message and set to “true”, if there is no follow-on command to be transmitted, or all the follow-on command transmissions have been completed, the A-IoT CN node should initiate the A-IoT Session Release procedure for the A-IoT session denoted by the AIOTF Identifier IE and the A-IoT Correlation Identifier IE contained in the INVENTORY REPORT message.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.71	Requested Service Area Information

This IE includes the Requested Service Area Information for the A-IoT inventory.

| IE/Group Name | Presence |  | Range |  | IE type and reference |  |  | Semantics description |  | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Requested Reader List |  |  | 0..1 |  |  |  |  |  |  | - |  |
| >Requested Reader Item |  |  | 1..<maxnoofReaders> |  |  |  |  |  |  | - |  |
| >>Global gNB ID | M |  |  |  | 9.3.1.6 |  |  |  |  | - |  |
| >>Reader Index | M |  |  |  | 9.3.1.276 |  |  |  |  | - |  |
| Requested A-IoT Area List |  |  | 0..1 |  |  |  |  |  |  | - |  |
| >Requested A-IoT Area Item |  |  | 1..<maxnoofAIoTAreas> |  |  |  |  |  |  | - |  |
| >>A-IoT Area ID | M |  |  |  | 9.3.1.277 |  |  |  |  | - |  |
| Requested UE Reader List |  |  |  | 0..1 |  |  |  |  | YES | ignore |
| >Requested UE Reader Item |  |  |  | 1..<maxnoofReaders> |  |  |  |  | - |  |
| >>UE Reader ID |  | M |  |  |  | FFS |  |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers. Value is 65535. |
| maxnoofAIoTAreas | Maximum no. of A-IoT Areas. Value is 256. |

Editor’s Note: the inclusion of the Requested UE Reader List IE in the Requested Service Area Information IE is based on the WA.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.6.4	Inventory Report Transfer

This IE provides the inventory report related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.69 |  | - |  |
| Global gNB ID | M |  | 9.3.1.6 |  | - |  |
| Reader Report List |  | 0..1 |  |  | - |  |
| >Reader Report Item |  | 1..< maxnoofReaders> |  |  | - |  |
| >>Reader Index | M |  | 9.3.1.276 |  | - |  |
| >>Device Report List |  | 1 |  |  | - |  |
| >>>Device Report Item |  | 1..<maxnoofDevices> |  |  | - |  |
| >>>>A-IoT NAS PDU | M |  | OCTET STRING |  | - |  |
| >>>>RAN A-IoT Device NGAP ID | O |  | 9.3.3.74 |  | - |  |
| Inventory Complete Indication | O |  | ENUMERATED (true, ...) |  | - |  |
| UE Reader Report List |  | 0..1 |  |  | YES | ignore |
| >UE Reader Report Item |  | 1..< maxnoofReaders> |  |  | - |  |
| >>UE Reader ID | M |  | FFS |  | - |  |
| >>Device Report List |  | 1 |  |  | - |  |
| >>>Device Report Item |  | 1..<maxnoofDevices> |  |  | - |  |
| >>>>A-IoT NAS PDU | M |  | OCTET STRING |  | - |  |
| >>>>RAN A-IoT Device NGAP ID | O |  | 9.3.3.74 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers served by a gNB. Value is 65535. |
| maxnoofDevices | Maximum no. of Devices within a reader. Value is 65535. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AIOTFIdentifier,

id-AIoT-CorrelationIdentifier,

id-AIoT-DeviceIdentificationRequested,

id-AIoT-RequestedServiceAreaInformation,

id-AIoT-RequestedUEReaderList,

id-AIoT-UEReaderReportList,

id-AIoT-InventoryAssistanceInformation,

id-AIoT-FollowonCommandIndication,

id-RAN-AIOT-Device-NGAP-ID,

id-AIoT-CommandAssistanceInformation,

id-AIoT-NASPDU,

id-AlternativeQoSParaSetList,

id-AssistanceInformationQoE-Meas,

id-AdditionalCancelledlocationReportingReferenceIDList,

id-BurstArrivalTimeDownlink,

id-Cause,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

AIoT-ReaderReportList ::= SEQUENCE (SIZE(1..maxnoofReaders)) OF AIoT-ReaderReportItem

AIoT-ReaderReportItem ::= SEQUENCE {

readerIndex					AIoT-ReaderIndex,

deviceReportList			AIoT-DeviceReportList,

iE-Extensions		ProtocolExtensionContainer { { AIoT-ReaderReportItem-ExtIEs} } OPTIONAL,

...

}

AIoT-ReaderReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-ReaderIndex ::= INTEGER (1..65535, ...)

AIoT-RequestedReaderList ::= SEQUENCE (SIZE(1..maxnoofReaders)) OF AIoT-RequestedReaderItem

AIoT-RequestedReaderItem ::= SEQUENCE {

globalgNB-ID				GlobalGNB-ID,

readerIndex					AIoT-ReaderIndex,

iE-Extensions		ProtocolExtensionContainer { { AIoT-RequestedReaderItem-ExtIEs} } OPTIONAL,

...

}

AIoT-RequestedReaderItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-RequestedAIoTAreaList ::= SEQUENCE (SIZE(1..maxnoofAIoTAreas)) OF AIoTAreaID

AIoT-RequestedServiceAreaInformation ::= SEQUENCE {

requestedReaderList				AIoT-RequestedReaderList					OPTIONAL,

requestedAIoTAreaList			AIoT-RequestedAIoTAreaList					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-RequestedServiceAreaInformation-ExtIEs} } OPTIONAL,

...

}

AIoT-RequestedServiceAreaInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-AIoT-RequestedUEReaderList		CRITICALITY ignore	EXTENSION AIoT-RequestedUEReaderList	PRESENCE optional},

...

}

AIoT-RequestedUEReaderList ::= SEQUENCE (SIZE(1..maxnoofReaders)) OF AIoT-RequestedUEReaderItem

AIoT-RequestedUEReaderItem ::= SEQUENCE {

uEReaderID					FFS,

iE-Extensions		ProtocolExtensionContainer { { AIoT-RequestedUEReaderItem-ExtIEs} } OPTIONAL,

...

}

AIoT-RequestedUEReaderItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIOTSessionReleaseCommandTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {AIOTSessionReleaseCommandTransferIEs} },

...

}

AIOTSessionReleaseCommandTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIoT-CorrelationIdentifier		CRITICALITY reject	TYPE AIoT-CorrelationIdentifier										PRESENCE	mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause															PRESENCE mandatory	},

...

}

AIOTSessionReleaseCompleteTransfer ::= SEQUENCE {

correlationIdentifier		AIoT-CorrelationIdentifier,

criticalityDiagnostics		CriticalityDiagnostics													OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {AIOTSessionReleaseCompleteTransfer-ExtIEs} }	OPTIONAL,

...

}

AIOTSessionReleaseCompleteTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIOTSessionReleaseRequestTransfer ::= SEQUENCE {

correlationIdentifier		AIoT-CorrelationIdentifier,

cause						Cause,

iE-Extensions				ProtocolExtensionContainer { {AIOTSessionReleaseRequestTransfer-ExtIEs} }	OPTIONAL,

...

}

AIOTSessionReleaseRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-Support ::= ENUMERATED {aIoT-only, aIoT-and-NRUu, ...}

AIoT-UEReaderReportList ::= SEQUENCE (SIZE(1..maxnoofReaders)) OF AIoT-UEReaderReportItem

AIoT-UEReaderReportItem ::= SEQUENCE {

uEReaderID					FFS,

deviceReportList			AIoT-DeviceReportList,

iE-Extensions		ProtocolExtensionContainer { { AIoT-UEReaderReportItem-ExtIEs} } OPTIONAL,

...

}

AIoT-UEReaderReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevelARP				PriorityLevelARP,

pre-emptionCapability			Pre-emptionCapability,

pre-emptionVulnerability		Pre-emptionVulnerability,

iE-Extensions		ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationAndRetentionPriority-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

InventoryReportTransfer ::= SEQUENCE {

correlationIdentifier			AIoT-CorrelationIdentifier,

globalgNB-ID					GlobalGNB-ID,

readerReportList				AIoT-ReaderReportList													OPTIONAL,

inventoryCompleteIndication		ENUMERATED {true, ...}													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { InventoryReportTransfer-ExtIEs} }		OPTIONAL,

...

}

InventoryReportTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-AIoT-UEReaderReportList		CRITICALITY ignore	EXTENSION AIoT-UEReaderReportList	PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-PduSetDelayBudgetUplink								ProtocolIE-ID ::= 485

id-PduSetErrorRateDownlink								ProtocolIE-ID ::= 486

id-PduSetErrorRateUplink								ProtocolIE-ID ::= 487

id-DLPDUSetInformationMarkingSupportIndication			ProtocolIE-ID ::= 488

id-MonitoringRequestonAvailableBitrate					ProtocolIE-ID ::= 489

id-MMSID												ProtocolIE-ID ::= 490

id-Indication-of-Bitrate-Adaptation						ProtocolIE-ID ::= 491

id-SCGActivationTime									ProtocolIE-ID ::= 492

id-Aerial-UE-FlightInformationReporting					ProtocolIE-ID ::= 493

id-Aerial-UE-FlightInformationReportingControl			ProtocolIE-ID ::= 494

id-LPWUSDisableIndication								ProtocolIE-ID ::= 495

id-AIoT-RequestedUEReaderList							ProtocolIE-ID ::= 800  --to be allocated--

id-AIoT-UEReaderReportList								ProtocolIE-ID ::= 801  --to be allocated--

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>