# 3GPP TSG-RAN WG4 Meeting #117	R4-2520261

Dallas, USA, 17th-21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | - | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 36.133 RRM meas accuracy, reference NPRACH config for IoT_NTN_TDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Cambridge Consultants LTD, Iridium Satellite LLC, THALES |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_TDD-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of RRM performance requirement for IoT NTN TDD mode. IoT NTN TDD mode is introduced in Rel-19 under IOT_NTN_TDD work item operating in band 249. RRM core requirements are completed in TS 36.133 at RAN4#116 under RP-252407. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRM performance requirements for measurement accuracy, related reference measurement channel parameters for IoT NTN TDD mode under TS 36.133. New clauses have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IoT NTN TDD does not have above mentioned RRM perfromance requirements defined under Release 19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.5.1A, 9.1.22B (new), A.3.1.5.10 (new), A.3.2.3.3.9 (new), A.3.18, B.3.25A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------<<< Start of Change 1 >>>----------------------------------

### 3.5.1A	Groups of bands for satellite access

The grouping of bands for satellite access for NB-IOT is specified in Table 3.5.1A-1.

Table 3.5.1A-1: Band groups for NB-IoT for satellite access

| Group | E-UTRA FDD |  |
| --- | --- | --- |
|  | Band group notation | Operating bands |
| A | NFDD_SAB_A |  |
| B | NFDD_SAB_B |  |
| C | NFDD_SAB_C |  |
| D | NFDD_SAB_D |  |
| E | NFDD_SAB_E |  |
| F | NFDD_SAB_F |  |
| G | NFDD_SAB_G | 253, 254, 255, 256, 252 |
| H | NFDD_SAB_H |  |
| I | NFDD_SAB_I |  |
| J | NFDD_SAB_J |  |
| K | NFDD_SAB_K |  |
| L | NFDD_SAB_L |  |
| M | NFDD_SAB_M |  |
| N | NFDD_SAB_N |  |
| O | NTDD_SAB_O | 249 |

## The grouping of bands for satellite access for category M1 is specified in Table 3.5.1A-2.

------------------------------<<< End of Change 1 >>>-------------------------

------------------------------<<< Start of Change 2 >>>----------------------------------

### 9.1.22A	Measurement accuracy for UE Category NB1 for satellite access

#### 9.1.22A.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1

The requirements for absolute accuracy of NRSRP in this clause apply to a cell on the same frequency as that of the serving cell for UE Category NB1 for stand-alone, guard-band and in-band deployments. For a UE capable of NSSS-based RRM measurement, provided that nsss-NumOccDiffPrecoders value n1 has been indicated by higher layers, the accuracy requirement as specified in Table. 9.1.22A.1-2 shall apply. Otherwise, the accuracy requirement as specified in Table 9.1.22A.1-1 shall apply.

The accuracy requirements in Table 9.1.22A.1-1 and Table 9.1.22A.1-2 are valid under the following conditions:

Narrowband reference signals are transmitted either from one or two ports.

Conditions defined in Clause 7.3B of TS 36.102 [60] for reference sensitivity are fulfilled.

NRSRP|dBm according to Annex B.3. 25A for a corresponding Band

At least 1 DL subframe per radio frame of measured cell is available at the UE for NRSRP measurement assuming measured cell is identified cell.

Table 9.1.22A.1-1: NRSRP Intra frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 6 | 9 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| 8 | 11 | -6 dB | NFDD_SAB_G | N/A | -70 | -50 |
| 10.3 | 13.3 | -15≤Ês/Iot≤--6 dB | NFDD_SAB_G | - 122.9 | N/A | -70 |
| 12.3 | 15.3 | -15≤Ês/Iot≤--6 dB | NFDD_SAB_G | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table 9.1.22A.1-2: NRSRP Intra frequency absolute accuracy for UE Category NB1 under NSSS-based measurement

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 4 | 7 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| 6 | 9 | -6 dB | NFDD_SAB_G | N/A | -70 | -50 |
| 6 | 9 | -15≤Ês/Iot≤--6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| 8 | 11 | -15≤Ês/Iot≤--6 dB | NFDD_SAB_G | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA/NR operating band groups are as defined in Section 3.5 |  |  |  |  |  |  |

#### 9.1.22A.2	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1

The requirements for absolute accuracy of NRSRQ in this clause apply to a cell on the same frequency as that of the serving cell for NB-IoT UE for stand-alone, guard-band and in-band deployments. For a UE capable of NSSS-based RRM measurement, provided that nsss-NumOccDiffPrecoders value n1 has been indicated by higher layers, the accuracy requirement as specified in Table. 9.1.22A.2-2 shall apply. Otherwise, the accuracy requirement as specified in Table 9.1.22A.2-1 shall apply.

The accuracy requirements in Table 9.1.22A.2-1 and Table 9.1.22A.2-2 are valid under the following conditions:

Narrowband reference signals are transmitted either from one or two antenna ports.

Conditions defined in Clause 7.3B of TS 36.102 [60] for reference sensitivity are fulfilled.

NRSRP|dBm according to Annex B.3. 25A for a corresponding Band

At least 1 DL subframe per radio frame of measured cell is available at the UE for NRSRQ measurement assuming measured cell is identified cell.

Table 9.1.22A.2-1: NRSRQ Intra frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 3 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel |
| 5.2 | 8.2 | -3 dB | NFDD_SAB_G | -122.9 | -50 |
| 7.2 | 10.2 | -6 dB | Note 2 | Note 2 | Note 2 |
| 9.5 | 12.5 | -15≤Ês/Iot≤--6 dB | NFDD_SAB_G | -122.9 | -50 |
| 11.5 | 14.5 | -15≤Ês/Iot≤--6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table 9.1.22A.2-2: NRSRQ Intra frequency absolute accuracy for UE Category NB1 under NSSS-based measurement

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 3.2 | 6.2 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| 5.2 | 8.2 | -6 dB | NFDD_SAB_G | N/A | -70 | -50 |
| 5.2 | 8.2 | -15≤Ês/Iot≤--6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| 7.2 | 10.2 | -15≤Ês/Iot≤--6 dB | NFDD_SAB_G | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### 9.1.22A.3	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1

The requirements for absolute accuracy of NRSRP in this clause apply to a cell that has different carrier frequency from the serving cell. For a UE capable of NSSS-based RRM measurement, provided that nsss-NumOccDiffPrecoders value n1 has been indicated by higher layers, the accuracy requirement as specified in Table. 9.1.22A.3-2 shall apply. Otherwise, the accuracy requirement as specified in Table 9.1.22A.3-1 shall apply.

The accuracy requirements in Table 9.1.22A.3-1 and Table 9.1.22A.3-2 are valid under the following conditions:

-	Narrowband reference signals are transmitted either from one or two antenna ports.

-	Conditions defined in Clause 7.3 of TS 36.102 [60] for reference sensitivity are fulfilled.

-	NRSRP|dBm according to Annex B.3.[26A] for a corresponding Band

-	At least 1 DL subframe per radio frame of measured cell is available at the UE for NRSRP measurement assuming measured cell is identified cell.

Table 9.1.22A.3-1: NRSRP Inter frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 6 | 9 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| 8 | 11 | -6 dB | NFDD_SAB_G | N/A | -70 | -50 |
| 10.3 | 13.3 | -15≤Ês/Iot≤--6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| 12.3 | 15.3 | -15≤Ês/Iot≤--6 dB | NFDD_SAB_G | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

Table 9.1.22A.3-2: NRSRP Inter frequency absolute accuracy for UE Category NB1 under NSSS-based measurement

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 4 | 7 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| 6 | 9 | -6 dB | NFDD_SAB_G | N/A | -70 | -50 |
| 6 | 9 | -15≤Ês/Iot≤--6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| 8 | 11 | -15≤Ês/Iot≤--6 dB | NFDD_SAB_G | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### 9.1.22A.4	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1

The requirements for absolute accuracy of NRSRQ in this clause apply to a cell that has different carrier frequency from the serving cell. For a UE capable of NSSS-based RRM measurement, provided that nsss-NumOccDiffPrecoders value n1 has been indicated by higher layers, the accuracy requirement as specified in Table. 9.1.22A.4-2 shall apply. Otherwise, the accuracy requirement as specified in Table 9.1.22A.4-1 shall apply.

The accuracy requirements in Table 9.1.22A.4-1 and Table 9.1.22A.4-2 are valid under the following conditions:

-	Narrowband reference signals are transmitted either from one or two antenna ports.

-	Conditions defined in Clause 7.3 of TS 36.102 [60] for reference sensitivity are fulfilled.

-	NRSRP|dBm according to Annex B.3.[26A] for a corresponding Band

-	At least 1 DL subframe per radio frame of measured cell is available at the UE for NRSRQ measurement assuming measured cell is identified cell.

Table 9.1.22A.4-1: NRSRQ Inter frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 3 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel |
| 5.2 | 8.2 | -3 dB | NFDD_SAB_G | -122.9 | -50 |
| 7.2 | 10.2 | -6 dB | Note 2 | Note 2 | Note 2 |
| 9.5 | 12.5 | -15≤Ês/Iot≤--6 dB | NFDD_SAB_G | -122.9 | -50 |
| 11.5 | 14.5 | -15≤Ês/Iot≤--6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |

Table 9.1.22A.4-2: NRSRQ Inter frequency absolute accuracy for UE Category NB1 under NSSS-based measurement

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 3.2 | 6.2 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| 5.2 | 8.2] | -6 dB | NFDD_SAB_G | N/A | -70 | -50 |
| 5.2 | 8.2 | -15≤Ês/Iot≤--6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| 7.2 | 10.2 | -15≤Ês/Iot≤--6 dB | NFDD_SAB_G | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### 9.1.22A.5	NRSRP Measurement Report Mapping

The reporting range of NRSRP is the same as defined in section 9.1.22.9.

#### 9.1.22A.6	NRSRQ Measurement Report Mapping

The reporting range of NRSRQ is the same as defined in section 9.1.22.14.

#### 9.1.22A.7	MSG3-based Measurement Report Mapping for UE Category NB1

The MSG3-based Measurement Report Mapping for UE Category NB1 is the same as defined in section 9.1.22.15

#### 9.1.22A.8	Downlink Channel Quality Measurement Accuracy for UE Category NB1

The requirements for accuracy of downlink channel quality reporting in this clause apply to the serving cell on the anchor carrier and non-anchor carrier for UE Category NB1.

The accuracy requirements in Table 9.1.22A.8-1 are valid under the following conditions:

-	Cell specific reference signals are transmitted either from one or two ports.

-	Conditions defined in TS 36.102 [60] Clause 7.3 for reference sensitivity are fulfilled.

-	NRSRP|dBm according to Annex B.3.[25A] for a corresponding Band.

Table 9.1.22A.8-1: Downlink channel quality reporting accuracy for UE Category NB1

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups NOTE 2 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R/4 NOTE 1 | >1 | -6 dB | NFDD_SAB_G | -122.9 | N/A | -70 |
| R NOTE 1 | ≤1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | - 122.9 | N/A | -70 |
| R/8 NOTE 1 | >1 | -15 ≤ Ês/Iot ≤ -6 dB | NFDD_SAB_G | - 122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

### 9.1.22B	Measurement accuracy for UE Category NB1 for satellite access in NTN TDD

#### 9.1.22B.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1

The requirements for absolute accuracy of NRSRP in this clause apply to a cell on the same frequency as that of the serving cell for UE Category NB1 for stand-alone, deployments. The accuracy requirement based on NRS measurements as specified in Table 9.1.22B.1-1 shall apply.

The accuracy requirements in Table 9.1.22B.1-1 are valid under the following conditions:

Narrowband reference signals are transmitted either from one or two ports.

Conditions defined in Clause 7.3B of TS 36.102 [60] for reference sensitivity are fulfilled.

NRSRP|dBm according to Annex B.3. 25A for a corresponding Band

Table 9.1.22B.1-1: NRSRP Intra frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 6 | 9 | -1 dB | NTDD_SAB_O | -122.9 | N/A | -70 |
| 8 | 11 | -1 dB | NTDD_SAB_O | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### 9.1.22B.2	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1

The requirements for absolute accuracy of NRSRQ in this clause apply to a cell on the same frequency as that of the serving cell for NB-IoT UE for stand-alone deployments. The accuracy requirement based on NRS measurements as specified in Table 9.1.22B.2-1 shall apply.

The accuracy requirements in Table 9.1.22B.2-1 are valid under the following conditions:

Narrowband reference signals are transmitted either from one or two antenna ports.

Conditions defined in Clause 7.3B of TS 36.102 [60] for reference sensitivity are fulfilled.

NRSRP|dBm according to Annex B.3. 25A for a corresponding Band

Table 9.1.22B.2-1: NRSRQ Intra frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 3 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel |
| 5.2 | 8.2 | -1 dB | NTDD_SAB_O | -122.9 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### 9.1.22B.3	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1

The requirements for absolute accuracy of NRSRP in this clause apply to a cell that has different carrier frequency from the serving cell. The accuracy requirement based on NRS measurements as specified in Table 9.1.22B.3-1 shall apply.

The accuracy requirements in Table 9.1.22B.3-1 are valid under the following conditions:

-	Narrowband reference signals are transmitted either from one or two antenna ports.

-	Conditions defined in Clause 7.3 of TS 36.102 [60] for reference sensitivity are fulfilled.

-	NRSRP|dBm according to Annex B.3 [26A] for a corresponding Band.

Table 9.1.22B.3-1: NRSRP Inter frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 2 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| 6 | 9 | -1 dB | NTDD_SAB_O | -122.9 | N/A | -70 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

#### 9.1.22B.4	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1

The requirements for absolute accuracy of NRSRQ in this clause apply to a cell that has different carrier frequency from the serving cell. The accuracy requirement based on NRS measurements as specified in Table 9.1.22B.4-1 shall apply.

The accuracy requirements in Table 9.1.22B.4-1 are valid under the following conditions:

-	Narrowband reference signals are transmitted either from one or two antenna ports.

-	Conditions defined in Clause 7.3 of TS 36.102 [60] for reference sensitivity are fulfilled.

-	NRSRP|dBm according to Annex B.3.[26A] for a corresponding Band

Table 9.1.22B.4-1: NRSRQ Inter frequency absolute accuracy for UE Category NB1

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA/NR operating band groups Note 3 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz | dBm/BWChannel |
| 5.2 | 8.2 | -1 dB | NTDD_SAB_O | -122.9 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |

#### 9.1.22B.5	NRSRP Measurement Report Mapping

The reporting range of NRSRP is the same as defined in section 9.1.22.9.

#### 9.1.22B.6	NRSRQ Measurement Report Mapping

The reporting range of NRSRQ is the same as defined in section 9.1.22.14.

#### 9.1.22B.7	MSG3-based Measurement Report Mapping for UE Category NB1

The MSG3-based Measurement Report Mapping for UE Category NB1 is the same as defined in section 9.1.22.15

#### 9.1.22B.8	Downlink Channel Quality Measurement Accuracy for UE Category NB1

The requirements for accuracy of downlink channel quality reporting in this clause apply to the serving cell on the anchor carrier and non-anchor carrier for UE Category NB1.

The accuracy requirements in Table 9.1.22B.8-1 are valid under the following conditions:

-	Cell specific reference signals are transmitted either from one or two ports.

-	Conditions defined in TS 36.102 [60] Clause 7.3 for reference sensitivity are fulfilled.

-	NRSRP|dBm according to Annex B.3.[25A] for a corresponding Band.

Table 9.1.22B.8-1: Downlink channel quality reporting accuracy for UE Category NB1

| NPDCCH Repetition | Pm-Dsg (%) | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Ês/Iot | Io NOTE 1 range |  |  |  |
|  |  |  | E-UTRA/NR operating band groups NOTE 2 | Minimum Io |  | Maximum Io |
|  |  | dB |  | dBm/15kHz | dBm/BWChannel | dBm/BWChannel |
| R NOTE 1 | ≤1 | -1 dB | NTDD_SAB_O | -122.9 | N/A | -70 |
| R/4 NOTE 1 | >1 | -1 dB | NTDD_SAB_O | -122.9 | N/A | -70 |
| NOTE 1: R is the reported NPDCCH repetition level that UE has reported in CQI-NPDCCH-NB or CQI-NPDCCH-Short-NB. NOTE 2: Io is assumed to have constant EPRE across the bandwidth. NOTE 3: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |

------------------------------<<< End Change 2 >>>-------------------------

------------------------------<<< Start of Change 3 >>>----------------------------------

Annex A (normative):
Test Cases

------------------------------ Unmodified Sections (for Change 3) ---------------------------------

# A.3	RRM test configurations

## A.3.1	Reference Measurement Channels

#### A.3.1.5.3	HD-FDD standalone operation

Table A.3.1.5.3-1: NPDSCH Reference Channel for UE category NB1 for standalone operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.18 HD-FDD | R.19 HD-FDD |
| Channel bandwidth | KHz | 200 | 200 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 72 Note 2 | 72 Note 2 |
| For Sub-Frame 4, 9 | Bits | Note 3 | Note 3 |
| For Sub-Frame 0, 5 | Bits | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 1 Note 4 | 1 Note 4 |
| For Sub-Frame 4, 9 |  | Note 5 | Note 5 |
| For Sub-Frame 0, 5 |  | 0 | 0 |
| Maximum number of repetitions | - | Note 6 | Note 6 |
| Cell ID | - | Note 7 | Note 7 |
| Note 1: Shall depend upon the NPDSCH scheduling Note 2: Only apply for subframes scheduled with NPDSCH. Note 3: 72 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 4: only apply for subframes scheduled with NPDSCH.. Note 5: 1 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 6: Maximum number of repetitions shall depend upon the test configuration. Note 7: Cell ID shall depend upon the test configuration. |  |  |  |

------------------------------ Unmodified Sections (for Change 3) ----------------------------------

#### A.3.1.5.9	TDD guard band operation

Table A.3.1.5.9-1: NPDSCH Reference Channel for UE category NB1 for guard band operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.22 NB-TDD | R.23 NB-TDD |
| LTE Carrier bandwidth | MHz | 10 | 10 |
| Allocated resource blocks for NB-IoT | PRB | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 4 | Bits | 72 Note 2 | 72 Note 2 |
| For Sub-Frame 0, 5, 9 | Bits | 0 | 0 |
| For Sub-Frame 1, 6 (DwPTS) | Bits | 40 or 72 Note 6 | 40 or 72 Note 6 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 4 |  | 1 Note 2 | 1 Note 2 |
| For Sub-Frame 0, 5, 9 |  | 0 | 0 |
| For Sub-Frame 1, 6 (DwPTS) |  | 1 | 1 |
| Maximum number of repetitions | - | Note 3 | Note 3 |
| Cell ID | - | Note 4 | Note 4 |
| Note 1: Shall depend upon the NPDSCH scheduling. Note 2: Only apply for subframes scheduled with NPDSCH. Note 3: Maximum number of repetitions shall depend upon the test configuration. Note 4: Cell ID shall depend upon the test configuration. Note 5: SIB1-NB is transmitted in subframe 0 in odd radio frames. Note 6: Set 72 when NPDSCH is transmitted with repetition. Set 40 when NPDSCH is transmitted without repetition. |  |  |  |

#### A.3.1.5.10	NTN-TDD standalone operation

Table A.3.1.5.10-1: NPDSCH Reference Channel for UE category NB1 for standalone operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.24 NB-TDD | R.25 NB-TDD |
| Channel bandwidth | KHz | 200 | 200 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 3, 6, 7, 8 | Bits | 72 Note 2 | 72 Note 2 |
| For Sub-Frame 4, 9 | Bits | Note 3 | Note 3 |
| For Sub-Frame 0, 5 | Bits | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 3, 6, 7, 8 |  | 1 Note 4 | 1 Note 4 |
| For Sub-Frame 4, 9 |  | Note 5 | Note 5 |
| For Sub-Frame 0, 5 |  | 0 | 0 |
| Maximum number of repetitions | - | Note 6 | Note 6 |
| Cell ID | - | Note 7 | Note 7 |
| Note 1: Shall depend upon the NPDSCH scheduling Note 2: Only apply for subframes scheduled with NPDSCH. Note 3: 72 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 4: only apply for subframes scheduled with NPDSCH.. Note 5: 1 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 6: Maximum number of repetitions shall depend upon the test configuration. Note 7: Cell ID shall depend upon the test configuration. |  |  |  |

## ------------------------------<<< End of Change 3 >>>----------------------------------

## ------------------------------<<< Start of Change 4 >>>----------------------------------

### A.3.2.3	OCNG Patterns for Narrowband IoT

------------------------------ Unmodified Sections (for Change 4)  ----------------------------------

#### A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT

Table A.3.2.3.3-1: NOP.3 FDD: OCNG FDD Pattern 3

| Allocation | Relative power level [dB] |  |  | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |
|  | 0, 4 | 5, 9 | 1-3, 6-8 |  |
| 0 | 0 (Note 2) | 0 (Note 2) | 0 | Note 1 |
| Note 1: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in TS 36.213. Note 2: Value of is applicable to PRBs not used for transmission of NPSS, NSSS and NPBCH in anchor cell (= 0). Note 3: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over NOCNG. |  |  |  |  |

------------------------------ Unmodified Sections (for Change 4)  ----------------------------------

#### A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT

Table A.3.2.3.8-1: NOP.3 TDD: OCNG TDD Pattern 3 for uplink-downlink configurations 1 and 2

| Allocation | Relative power level [dB] |  |  | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |
|  | 0, 5, 9 | 1, 6Note 5 | 3, 4, 8Note 3 |  |
| 0 | 0 (Note 2) | 0 | 0 | Note 1 |
| Note 1: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in TS 36.213. Note 2: Value of is applicable to PRBs not used for transmission of NPSS, NSSS and NPBCH in anchor cell (= 0). Note 3: Subframes available for DL transmission depends on the Uplink-Downlink configuration in Table 4.2-2 in TS 36.211 [16]. Note 4: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over NOCNG. Note 5: Whether NPDSCH transmissions are carried out in DwPTS depends on the special subframe configuration in use. |  |  |  |  |

#### A.3.2.3.3.9	Narrowband IoT OCNG NTN TDD pattern 4: standalone NB-IoT

Table A.3.2.3.3-1: NOP.4 TDD: OCNG TDD Pattern 4

| Allocation | Relative power level [dB] |  |  | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |
|  | 0, 4 | 5, 9 | 3, 6-8 |  |
| 0 | 0 (Note 2) | 0 (Note 2) | 0 | Note 1 |
| Note 1: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in TS 36.213. Note 2: Value of is applicable to PRBs not used for transmission of NPSS, NSSS and NPBCH in anchor cell (= 0). Note 3: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over NOCNG. |  |  |  |  |

------------------------------<<< End of Change 4 >>>-------------------------

------------------------------<<< Start of Change 5 >>>----------------------------------

## A.3.18	Reference NPRACH Configurations

Table A.3.18-1, A.3.18-2 and A.3.18.3 define the reference NB-IoT PRACH configurations for a NB-IoT RRM test case where the UE is required to transmit NPRACH during the testing procedure, but the testing purpose of the RRM test case does not include testing NPRACH performance.

Table A.3.18-1: NPRACH.R-1: HD-FDD Reference NPRACH Configuration

| Field | Value |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH coverage level |  |  |  |  |  |
| rsrp-ThresholdsPrach | {rsrp1, rsrp2} |  |  | The values of NPRACH RSRP thresholds for will be set according the requirement of individual test cases |  |
| nprach-CP-Length | us66dot7 |  |  | NPRACH format 0 |  |
| Parameters per NPRACH coverage Level |  |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 |  | Valid values as defined in TS 36.331 [2] |  |
| nprach-Periodicity | ms40 | ms40 | ms40 |  | {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560} |  |
| nprach-StartTime | ms8 | ms8 | ms8 |  | {ms8, ms16, ms32, ms64, ms128, ms256, ms512, ms1024} |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  | {n0, n12, n24, n36, n2, n18, n34} |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  | {n12, n24, n36, n48} |  |
| nprach-SubcarrierMSG3-RangeStart | {one} | {one} | {one} |  | {zero, oneThird, twoThird, one} |  |
| maxNumPreambleAttemptCE | n3 | n5 | n7 |  | {n3, n4, n5, n6, n7, n8, n10} |  |
| numRepetitionsPerPreambleAttempt | n1 | n8 | n32 |  | {n1, n2, n4, n8, n16, n32, n64, n128} |  |
| npdcch-NumRepetitions-RA | r1 | r8 | r32 |  | {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048} |  |
| npdcch-StartSF-CSS-RA | v8 | v2 | v2 |  | {v1dot5, v2, v4, v8, v16, v32, v48, v64} |  |
| npdcch-Offset-RA | zero | zero | zero |  | {zero, oneEighth, oneFourth, threeEighth} |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  | {n8, n10, n11, n12, n20, n22, n23, n24, n32, n34, n35, n36, n40, n44, n48} |  |
| Note 1: See Clause 6.7.3.2 in TS 36.331 [2] for further information on the parameters in this table. |  |  |  |  |  |

Table A.3.18-2: NPRACH.R-2: TDD Reference NPRACH Configuration

| Field | Value |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH coverage level |  |  |  |  |  |
| rsrp-ThresholdsPrach | {rsrp1, rsrp2} |  |  | The values of NPRACH RSRP thresholds for will be set according the requirement of individual test cases |  |
| nprach-PreambleFormat | fmt-0 |  |  | See TS 36.211 [16] section 10.1.6 |  |
| Parameters per NPRACH coverage Level |  |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 |  | Valid values as defined in TS 36.331 [2] |  |
| nprach-Periodicity | ms80 | ms80 | ms80 |  | {ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ms10240} |  |
| nprach-StartTime | ms10 | ms10 | ms10 |  | {ms10, ms20, ms40, ms80, ms160, ms320, ms640, ms1280, ms 2560, ms5120} |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  | {n0, n12, n24, n36, n2, n18, n34} |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  | {n12, n24, n36, n48} |  |
| nprach-SubcarrierMSG3-RangeStart | {one} | {one} | {one} |  | {zero, oneThird, twoThird, one} |  |
| maxNumPreambleAttemptCE | n3 | n5 | n7 |  | {n3, n4, n5, n6, n7, n8, n10} |  |
| numRepetitionsPerPreambleAttempt | n1 | n8 | n32 |  | {n1, n2, n4, n8, n16, n32, n64, n128, n256, n512, n1024} |  |
| npdcch-NumRepetitions-RA | r1 | r8 | r32 |  | {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048} |  |
| npdcch-StartSF-CSS-RA | v8 | v4 | v4 |  | {v4, v8, v16, v32, v48, v64, v96, v128} |  |
| npdcch-Offset-RA | zero | zero | zero |  | {zero, oneEighth, oneFourth, threeEighth} |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  | {n8, n10, n11, n12, n20, n22, n23, n24, n32, n34, n35, n36, n40, n44, n48} |  |
| Note 1: See Clause 6.7.3.2 in TS 36.331 [2] for further information on the parameters in this table. |  |  |  |  |  |

Table A.3.18-3: NPRACH.R-3: NTN-TDD Reference NPRACH Configuration

| Field | Value |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH coverage level |  |  |  |  |  |
| rsrp-ThresholdsPrach | {rsrp1, rsrp2} |  |  | The values of NPRACH RSRP thresholds for will be set according the requirement of individual test cases |  |
| nprach-CP-Length | us66dot7 |  |  | NPRACH format 0 |  |
| Parameters per NPRACH coverage Level |  |  |  |  |  |
| CE Level | Level 0 | Level 1 | Level 2 |  | Valid values as defined in TS 36.331 [2] |  |
| nprach-Periodicity | ms320 | ms640 | ms1280 |  | {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560} |  |
| nprach-StartTime | ms8 | ms8 | ms8 |  | {ms8, ms16, ms32, ms64, ms128, ms256, ms512, ms1024} |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  | {n0, n12, n24, n36, n2, n18, n34} |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  | {n12, n24, n36, n48} |  |
| nprach-SubcarrierMSG3-RangeStart | {one} | {one} | {one} |  | {zero, oneThird, twoThird, one} |  |
| maxNumPreambleAttemptCE | n3 | n5 | n7 |  | {n3, n4, n5, n6, n7, n8, n10} |  |
| numRepetitionsPerPreambleAttempt | n1 | n4 | n8 |  | {n1, n2, n4, n8, n16, n32, n64, n128} |  |
| npdcch-NumRepetitions-RA | r1 | r4 | r8 |  | {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048} |  |
| npdcch-StartSF-CSS-RA | v64 | v32 | v4 |  | {v1dot5, v2, v4, v8, v16, v32, v48, v64} |  |
| npdcch-Offset-RA | zero | zero | zero |  | {zero, oneEighth, oneFourth, threeEighth} |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  | {n8, n10, n11, n12, n20, n22, n23, n24, n32, n34, n35, n36, n40, n44, n48} |  |
| Note 1: See Clause 6.7.3.2 in TS 36.331 [2] for further information on the parameters in this table. |  |  |  |  |  |

------------------------------<<< End of Change 5 >>>-------------------------

------------------------------<<< Start of Change 6 >>>----------------------------------

Annex B (normative):
Conditions for RRM requirements applicability for operating bands

------------------------------ Unmodified Sections (for Change 6)  ----------------------------------

## B.3.25A	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1 for satellite access

This clause defines the NB-IoT intra-frequency NRSRP applicable for a corresponding operating band. The UE category NB1 applicability of the conditions in Appendix B.3.25A is defined in Section 3.6.

The conditions for intra-frequency absolute NRSRP and NRSRQ accuracy requirements are defined in Table B.3.25A-1.

Table B.3.25A-1: NB-IoT intra-frequency absolute NRSRP and NRSRQ Accuracy Requirements

| Parameter | E-UTRA/NR operating band groups Note 1 | Minimum NRSRP |
| --- | --- | --- |
|  |  | dBm/15kHz |
| Conditions | NFDD_SAB _G | -139.8 |
|  | NTDD_SAB_O | -133 |
| NOTE 1: E-UTRA/NR operating band groups are as defined in Section 3.5. |  |  |

------------------------------<<< End of Change 6 >>>-------------------------