3GPP TSG-RAN WG4 Meeting #117  		R4-2520441

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	8.12.2.3

Source: 	CMCC

Title: 			 Discussion on demodulation specification enhancement for 6G study

Document for:	Discussion

- Introduction

RAN4 operation efficiency study for 6G started from RAN4#116bis meeting. For performance and demodulation part, the open issues are summarized as following [1]:

Issue 1-1-5: Issues on Performance or Demodulation Specifications

- Observed issues

  - Issue 1: In the 5G BS demodulation performance specifications, applicability of certain requirements is not explicitly linked to the support of the corresponding features.

  - Issue 2: The current structure of applicability rules in TS 38.101-4 is fragmented and inconsistent across device types

Issue 3-4: Perf/Demod specs improvements

- Proposals

  - Proposal 1: Replacing broad applicability statements with clear, centralized mappings of test coverage. This would enhance consistency and reduce ambiguity across device types and configurations.

  - Proposal 2: A capability-aware test applicability framework should be intended, e.g., For devices lacking legacy TN support, test applicability should be designed to avoid dependency on TN-related procedures and corresponding test cases."

  - Proposal 3: FRC table improvements considering

    - necessary configuration and avoid derived information

    - methodology to generate FRC table efficiently

In this contribution, we share our views on demodulation specification enhancement.

- Discussion

  - FRC Tables

In both UE and BS demodulation/conformance test specifications, including TS38.101-4, TS38.104, TS 38.141-1/2, RAN4 introduces FRC tables to assist the TE vendors in setting up test environments. Based on our observations, lots of efforts has been devoted to the introduction and maintenance of these FRCs.

Observation on FRC introduction

- Parameters such as bandwidth, SCS, modulation format and code rate are duplicated between minimum requirement Clauses and related FRC Clauses.

- Information Bit Payload per Slot and Transport block CRC per Slot can be derived using the definition in TS 38.214 Clause 5.1.3 and TS 38.212 Clause 5.2.2, based on the configured parameter including Number of allocated resource blocks, Number of consecutive PDSCH symbols, MCS, MIMO layers and allocated DMRS REs.

- Maximum throughout can be further derived from Information Bit Payload per Slot and Allocated slots using a fixed formula.

- Binary Channel Bits Per Slot can be derived using a fixed formula based on Number of allocated resource blocks, allocated CSI-RS REs, MCS and MIMO layers.

- The FRC numbering can not be finalized even at BigCR stage due to potential conflicts between different WIs.

- Same FRC tables are repeated across different specs, such as TS 38.104, TS 38.141-1 and TS 38.141-2

- The specification was expanded due to new tables need to be created for different SCS, different Modulation order, different CSI-RS/PDSCH/PRB scheduling, different features, and when the reserved columns are filled up.

- Observation on FRC maintenance

- Correction are sometimes needed for values due to misunderstanding of configurations and allocated reference signals, incorrect calculation method, or calculation errors.

- Correction are sometimes needed for Table/FRC numbering due to typos or overlapping between WIs

- Observation 1: The complexity and redundancy in FRC definitions consume a lot of time in spec drafting and maintenance, reflected in the following aspects:

- Redundant parameter definitions across Clauses, and redundant FRC definition across specs

- The calculation procedure of the derived values is fixed, which can be based on clear derivation method definition, yet only the results are currently captured, leading to potential calculation errors.

- The numbering of FRC tables and FRC itself can not be finalized even at the BigCR stage

- Observation 2: The specification was expanded because new table need to be created for different SCS, different Modulation order, different CSI-RS/PDSCH/PRB scheduling, different features, and when the reserved columns are filled up.

Based on above observations, we see the possibility of FRC enhancement. For example, only capture essential non-repeated configuration and common derivation method definition in the spec. Add new FRC by adding rows rather than columns to overcome the page size limitation.

- Proposal 1: Simplify the FRC table by only capture essential non-repeated configuration and common derivation method definition for the computed values. New FRCs should be added as rows to avoid page size constraints.

Regarding the issue of FRC table/FRC numbering conflicts when multiple WIs require simultaneous updates within the same specification version, we believe this issue can be addressed as part of a broader CR handling improvement, similar to how Clause numbering issues are managed.

- Proposal 2: Address the FRC table/FRC numbering issue as part of a broader CR handling improvement

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

- Observation 1: The complexity and redundancy in FRC definitions consume a lot of time in spec drafting and maintenance, reflected in the following aspects:

- Redundant parameter definitions across Clauses, and redundant FRC definition across specs

- The calculation procedure of the derived values is fixed, which can be based on clear derivation method definition, yet only the results are currently captured, leading to potential calculation errors.

- The numbering of FRC tables and FRC itself can not be finalized even at the BigCR stage

- Observation 2: The specification was expanded because new table need to be created for different SCS, different Modulation order, different CSI-RS/PDSCH/PRB scheduling, different features, and when the reserved columns are filled up.

- Proposal 1: Simplify the FRC table by only capture essential non-repeated configuration and common derivation method definition for the computed values. New FRCs should be added as rows to avoid page size constraints.

- Proposal 2: Address the FRC table/FRC numbering issue as part of a broader CR handling improvement

- Reference

- R4-2514648, Way Forward for [116bis][111] 6G operation efficiency, CATT