3GPP RAN WG4 Meeting #117																	R4-2520704

Dallas, TX, USA, 17th – 21st November 2025

Agenda item:	8.2.2

Source:	CableLabs, Charter Communications, Rogers Communications

Title:	UL 1k-QAM & DL 4k-QAM achievability in a UMa FWA scenario

WI/SI:	6G System Parameters

Release:	Rel-20

Document for:	Approval

# 1	Introduction

In Prague in Oct. 2025, RAN4#116bis achieved the following agreement on 6GR uplink (UL) 1024-QAM [1]:

“Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations”

RAN1#122bis achieved the following agreement on 6GR UL 1024-QAM and downlink (DL) 4096-QAM [2]:

“For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, need to study performance (assuming realistic channel estimation, time/freq synchronization assumption, phase noise assumption, etc), complexity/power consumption, requirements, benefit/necessity under applicable scenarios, associated restrictions, and challenges (such as EVM requirement, PAPR increase, MPR or A-MPR increase under realistic PA model).

- FFS: How to involve RAN4 early

- FFS: Shaping of higher order modulation

- System level evaluation can be done after link level evaluation.”

Observation 1: RAN4#116bis agreed to study UL 1024-QAM in Rel-20 6GR. RAN1#122bis agreed to study both DL 4096-QAM and UL 1024-QAM in Rel-20 6GR.

One of the study topics for high-order modulation schemes is the availability of UL 1024-QAM and DL 4096-QAM in field deployments. To evaluate their feasibility, a system-level simulation is performed for an urban macro (UMa) Fixed Wireless Access (FWA) scenario at around 4 GHz and 7 GHz, assuming a Customer Premises Equipment (CPE) maximum Equivalent Isotropically Radiated Power (EIRP) of 31 dBm with UL power control disabled. The simulation results indicate that, under both outdoor and indoor CPE conditions, there is a significant probability that the SINR can support UL 1024-QAM and DL 4096-QAM.

Proposal 1: Based on the RAN4 and RAN1 October 2025 meeting agreements and our system-level simulation results, which indicate that UL 1024-QAM and DL 4096-QAM are widely achievable in a UMa FWA scenario, we propose that RAN4 study the use of DL 4096-QAM, in addition to UL 1024-QAM, for 6GR FWA operation.

Proposal 2: We encourage other companies to perform similar system-level simulations to evaluate the achievability of UL 1024-QAM and 4096-QAM in 6GR FWA scenarios.

# 2	Simulation Assumptions

The simulation methodology and assumptions follow the RAN4 simulation in TR 38.858 framework [3]. Some key parameters are listed below:

- UMa scenario for FWA.

- Network topology: 19 sites topology, 3 sectors per site, see Figure 1.

- Both co-channel network and adjacent-channel network use TDD assuming they are TDD synced, with a grid shift of 100%, see Figure 1.

- Frequency: around 4 GHz and around 7 GHz.

- Inter-site distance (ISD) is 500 m around 4 GHz [3] and 450 m around 7 GHz [4].

- Channel bandwidth: 100 MHz [3] and [4].

- Max BS EIRP:

  - Around 4 GHz: 72 dBm around 4 GHz [3] (this value is conservative, the actual EIRP density limit in band n77 in the USA is 62 dBm/MHz, which is 10 dB higher than in a 100 MHz channel, so that the actual DL 4096-QAM achievability could be higher).

  - Around 7 GHz: 78.3 dBm [4] and [5].

- BS antenna:

  - Around 4 GHz: 8×8 array, 5 dBi gain per element, and 23 dBi max gain for the array [3].

  - Around 7 GHz: 8×16 sub-arrays, 3×1 elements per sub-array 6.4 dBi gain per element, and 32.2 dBi max gain for the array [5].

- BS antenna height: 25 m above ground.

- Max CPE EIRP: 31 dBm, UL power control disabled.

- CPE antenna: 2×8 array, 5 dBi gain per element, and 17 dBi max gain for the array.

- CPE location: indoor (applying outdoor-to-indoor, O2I, loss based on TR 38.901) vs. outdoor (0 dB O2I loss).

- CPE height: 4 m for outdoor CPE, 1.5 m for indoor CPE.

- CPE distribution: uniformly distributed in a cell [3].

- Network load: 50% in DL and 50% in UL.

- 30 dB SINR threshold is selected for 1024-QAM in UL, and 38 dB SINR threshold is chosen for 4096-QAM in DL.

Figure 1. Network topology. The triangle symbols indicate BS sites with three sectors per site. The black solid lines represent co-channel network and red dashed lines demonstrate another network in the adjacent-channel with 100% grid shift.

# Simulation Results

Example simulation results at around 4 GHz for the outdoor CPE case are shown in Figure 2. Figure 2(a) presents a heatmap of UL SINR for cell #1 in Figure 1, showing that coverage is available almost everywhere in the cell, and a large portion of CPE locations exhibit strong SINR, indicating 1024-QAM is achievable. In Figure 2(b), the green dash-dotted line represents the UL signal-to-noise ratio (SNR) without interference, the blue solid line shows the signal-to-interference-plus-noise ratio (SINR) considering only co-channel interference (CCI), the pink dotted line depicts the SINR including both CCI and adjacent-channel interference (ACI), and the black dashed line demonstrates the minimum SINR required for UL 1024-QAM. The SINR thresholds are assumed to be 30 dB for UL 1024-QAM and 38 dB for DL 4096-QAM, which are subject to future study. Example simulation results for DL SINR are illustrated in Figure 3.

At around 4 GHz with outdoor CPE, the UL SINR exceeds the 1024-QAM SINR threshold of 30 dB for a 43.5% probability when considering both CCI and ACI. The DL SINR exceeds the 38 dB SINR threshold with an 81.7% probability under the same interference considerations. These results indicate that UL 1024-QAM and DL 4096-QAM are widely achievable in such an outdoor CPE FWA UMa scenario. The results for both 4 and 7 GHz, and for both outdoor and indoor CPE cases, are summarized in Table 1.

Observation 2: The simulation results indicate that there is a significant probability that the SINR can support UL 1024-QAM and DL4096-QAM: (a) at around 4 GHz with outdoor CPE: the probability that UL 1024-QAM can be supported is 43.5% and DL 4096-QAM can be supported is 81.7%; (b) at around 4 GHz with indoor CPE: the probability that UL 1024-QAM can be supported is 12.8% and DL 4096-QAM can be supported is 59.6%; (c) at around 7 GHz with outdoor CPE: the probability that UL 1024-QAM can be supported is 72% and DL 4096-QAM can be supported is 98.5%; and (d) at around 7 GHz with indoor CPE: the probability that UL 1024-QAM can be supported is 18.5% and DL 4096-QAM can be supported is 53.6%.

(a)																	(b)

Figure 2. (a) UL SINR heatmap for cell #1 in Figure 1, black triangle indicates BS site, colorbar represents SINR in dB. (b) UL SINR CDFs and 1024-QAM achievability.

(a)																	(b)

Figure 3. DL SINR heatmap for cell #1 in Figure 1, black triangle indicates BS site, colorbar represents SINR in dB. (b) DL SINR CDFs and 4096-QAM achievability.

Table 1. Summary of UL 1024-QAM and DL 4096-QAM achievability results.

| Frequency | ISD (m) | CPE location | UL 1024-QAM availability p(SINR>=30 dB) |  |  | DL 4096-QAM availability p(SINR>=38 dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | No CCI or ACI | CCI Only | CCI+ACI | No CCI or ACI | CCI Only | CCI+ACI |
| Around 4 GHz | 500 | Outdoor | 47.5% | 43.5% | 43.5% | 99.87% | 82.3% | 81.7% |
| Around 4 GHz | 500 | Indoor | 12.8% | 12.8% | 12.8% | 69.0% | 60.2% | 59.6% |
| Around 7 GHz | 450 | Outdoor | 72.8% | 72.0% | 72.0% | 99.7% | 98.6% | 98.5% |
| Around 7 GHz | 450 | Indoor | 18.5% | 18.5% | 18.5% | 54.0% | 53.6% | 53.6% |

# 4	Conclusion

The key observations and proposals of this contribution include:

Observation 1: RAN4#116bis agreed to study UL 1024-QAM in Rel-20 6GR. RAN1#122bis agreed to study both DL 4096-QAM and UL 1024-QAM in Rel-20 6GR.

Proposal 1: Based on the RAN4 and RAN1 October 2025 meeting agreements and our system-level simulation results, which indicate that UL 1024-QAM and DL 4096-QAM are widely achievable in a UMa FWA scenario, we propose that RAN4 study the use of DL 4096-QAM, in addition to UL 1024-QAM, for 6GR FWA operation.

Observation 2: The simulation results indicate that there is a significant probability that the SINR can support UL 1024-QAM and DL4096-QAM: (a) at around 4 GHz with outdoor CPE: the probability that UL 1024-QAM can be supported is 43.5% and DL 4096-QAM can be supported is 81.7%; (b) at around 4 GHz with indoor CPE: the probability that UL 1024-QAM can be supported is 12.8% and DL 4096-QAM can be supported is 59.6%; (c) at around 7 GHz with outdoor CPE: the probability that UL 1024-QAM can be supported is 72% and DL 4096-QAM can be supported is 98.5%; and (d) at around 7 GHz with indoor CPE: the probability that UL 1024-QAM can be supported is 18.5% and DL 4096-QAM can be supported is 53.6%.

Proposal 2: We encourage other companies to perform similar system-level simulations to evaluate the achievability of UL 1024-QAM and 4096-QAM in 6GR FWA scenarios.

# 5	References

- R4-2514618, “Agreement for 6GR system Parameters,” Feature lead (Huawei, HiSilicon), RAN4#116bis, Prague, CZ, Oct. 2025.

- “Chair notes RAN1 #122b eom4,” RAN1#122bis, Prague, CZ, Oct. 2025.

- 3GPP TR 38.858, “Study on Evolution of NR Duplex Operation,” V18.1.0, March 2024.

- 3GPP TR 38.922, “Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz,” V19.1.0, June 2025.

- R4-2414449, “LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27,” RAN4#112, Maastricht, Netherlands, Aug. 2024.