3GPP TSG-RAN WG4 Meeting #117	R4-2520908

Dallas, USA, Nov. 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft CR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for NES TC9 Intra-frequency measurement(Scenario 3B, Case 1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN4#116, RAN4 achieved following agreement for OD-SSB basded L3 measurement when SCell is activated Sub-topic 1-2: OD-SSB based L3 measurement Issue 1-2-1: OD-SSB based L3 measurement when SCell is activated(scenario 3B) Agreement (on Thursday) - For SCell measurement, UE shall follow the OD-SSB specific SMTC when OD-SSB is activated; For neighbor cell measurement, UE follows legacy SMTC regardless of status of OD-SSB. In RAN4 #116bis, RAN4 agreed to introduce the NES Test case of OD-SSB based SA event triggered reporting tests without gap for SCell under non DRX in FR1 using 10ms OD-SSB periodicity (Scenirao 3B, Case 1) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce the SMTC configuration of SMTC period=10ms with SMTC duration 5ms - Introduce the test case9 of OD-SSB based SA event triggered reporting tests without gap for SCell under non DRX in FR1 (Scenirao 3B, Case 1) - OD-SSB periodicity: 10ms - OD-SMTC periodicity: 10ms - Legacy-SMTC periodicity 20ms |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The related test case is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------- CHANGE #1 ------------------------------

A.6.6	Measurement procedure

A.6.6.1	Intra-frequency Measurements

A.6.6.1.x	SA event triggered reporting tests without gap under non-DRX based on OD-SSB

A.6.6.1.x.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event for UE indicated by od-ssb-config TS 38.331[2] or by a MAC CE TS 38.321[7], activation of transmission for OD-SSB in a configured DL BWP of an SCell TS 38.300[10] if UE supports On-demand SSB operation. This test will partly verify the intra-frequency cell search requirements in clause 9.17.5.1 and 9.17.5.2.

A.6.6.1.x.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), a FR1 activated SCell (Cell 2) and a FR1 neighbour cell (Cell 3) on the same frequency as the SCell (Cell 2). The test parameters for PCell are given in table A.6.6.1.x.2-1, A.6.6.1.x.2-2 and A.6.6.1.x.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the SCell, and it is indicated to the UE that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 3. During the time duration, an OD-SSB transmission, which cannot be used to obtain SIB1, is activated in a configured DL BWP of the SCell (Cell 2).

Table A.6.6.1.x.2-1: Supported PCell test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. NOTE 2: The UE is only required to be tested in one of the supported test configurations with the smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. |  |

Table 6.6.1.x.2-1A: Supported SCell test configurations

| ConfigurationSCell | Description |
| --- | --- |
| 1A | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2A | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3A | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. NOTE 2: The UE is only required to be tested in one of the supported test configurations with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. NOTE 3: NR Cell 3 has the same SCS, BW and duplex mode as NR Cell 2. |  |

Table A.6.6.1.x.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for activated SCell in FR1 with non-DRX baed on OD-SSB

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Deactivated SCell |  | 1A, 2A, 3A | Cell 2 |  |
| Neighbour cell |  | 1A, 2A, 3A | Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 2: Cell 2 and 3 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| OD-SSB configuration |  | 1A | SSB.X1 FR1 |  |
|  |  | 2A | SSB.X1 FR1 |  |
|  |  | 3A | SSB.X4 FR1 |  |
| SMTC configuration |  | 1A | SMTC.2 |  |
|  |  | 2A | SMTC.1 |  |
|  |  | 3A | SMTC.1 |  |
| OD-SMTC configuration |  | 1A | SMTC.8 |  |
|  |  | 2A | SMTC.8 |  |
|  |  | 3A | SMTC.8 |  |
| A6-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 3 is 3 ms later than the timing of Cell 2. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.1.x.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for activated SCell in FR1 with non-DRX based on OD-SSB

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDD configuration |  | 1,1A | N/A |  | N/A |  | N/A |  |
|  |  | 2,2A | TDDConf.1.1 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3,3A | TDDConf.2.1 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1,1A | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2,2A | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3,3A | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1,1A | CR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2,2A | CR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3,3A | CR.2.1 TDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1,1A | CCR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2,2A | CCR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3,3A | CCR.2.1 TDD |  | N/A |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3,1A,2A,3A | OP.1 |  | OP.1 |  | OP.1 |  |
| IInitial BWP configuration |  | 1, 2, 3,1A,2A,3A | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3,1A,2A,3A | DLBWP.1.1 |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3,1A,2A,3A | ULBWP.1.1 |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3,1A,2A,3A | NCD-SSB |  | NCD-SSB |  | NCD-SSB |  |
| Note 2 | dBm/SCS | 1,1A | -98 |  |  |  |  |  |
|  |  | 2,2A | -98 |  |  |  |  |  |
|  |  | 3,3A | -95 |  |  |  |  |  |
| Note 2 | dBm/15 kHz | 1,1A | -98 |  |  |  |  |  |
|  |  | 2,2A |  |  |  |  |  |  |
|  |  | 3,3A |  |  |  |  |  |  |
|  | dB | 1,1A | 4 | 4 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2,2A |  |  |  |  |  |  |
|  |  | 3,3A |  |  |  |  |  |  |
|  | dB | 1,1A | 4 | 4 | 4 | 4 | -Infinity | 4 |
|  |  | 2,2A |  |  |  |  |  |  |
|  |  | 3,3A |  |  |  |  |  |  |
| SSB_RP Note 3 | dBm/SCS kHz | 1,1A | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 2,2A | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 3,3A | -91 | -91 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1,1A | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2,2A | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3,3A | -58.50 | -58.50 | -58.50 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1,2,3, 1A,2A,3A | AWGN |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

A.6.6.1.8.3	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH

-------------------- END OF CHANGES--------------------------