3GPP TSG-RAN WG4 Meeting #117	R4-2521036

Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.12.3

Source:	Samsung

Title:	Discussion and simulation results on Rel-19 NR MIMO Phase 5 UE performance

Document for:	Discussion

- Introduction

In the RAN plenary RAN#102 meeting the WID [1] for Rel-19 NR MIMO Phase 5 has been approved and further revised in RAN#105 meeting [2]. This is our fourth meeting on demodulation and CSI reporting performance requirements for Rel-19 NR MIMO Phase 5 WI. General impacts and test assumptions of each objective have been discussed in previous RAN4#115~ RAN4#116bis meeting, agreements are captured in [3][4][5]. For UE performance test, PMI reporting requirements for both typeI-SinglePanel-r19 and eTypeII-r19 have been agreed to be specified, while agreements also achieved not to define UE performance tests for typeII-Doppler-r19 codebook, SRS port grouping and CJT enhancement in Rel-19.

In this contribution, we provide our analysis and views on the open issues for PMI test metric and round-off operation of MIMO correlation. In addition, we submit our latest simulation results for both typeI-SinglePanel-r19 and eTypeII-r19 codebooks based on the test assumptions agreed in last meeting.

- Test assumptions for PMI reporting performance

For test assumptions of PMI reporting performance, two open issues need further discussion as below, one is for test metric, the other is for the round-off operation of MIMO correlation matrices.

Issue 1-2-2: Test metric for 64 CSI-RS ports Proposals: - Option 1: Modify the legacy test metric to following: - Option 1A: X=5dB γ=tuetrnd tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trndis the throughput measured at SNRue+[XdB] with random precoding. X could be specified to make the γ value in the reasonable range.. - Option 2: Consider possibility to define requirement only based on SNR of 90% of the maximum throughput when PMI is followed Way forward: - FFS based on simulation results provided in following meetings Issue 1-2-9: round-off operation of MIMO correlation matrices Agreement: - Companies to provide simulation results based on either option 1 or option 2, the results based on both options will be taken into account for deriving the requirements. - Further discuss and decide which option to be captured in the CR in next meeting - Option 1: Include round-off to four digits precision operation - Option 2: Not include round-off to four digits precision operation

Test metric for 64 CSI-RS ports

About the test metric for PMI reporting requirements, agreement reached in RAN4#115 meeting [3] as below to reuse legacy PMI test metric. However, since almost zero throughput for random Type I codebook case in following simulations observed by some contributions, further discussion on the test metric were triggered on RAN4#116 and RAN4#116bis meeting. On last RAN4#116bis meeting, two potential solutions were widely considered by companies, the first one is to modify the legacy test metric with additional X dB, the other one is to define requirement only based on SNR at 90% of the maximum throughput.

Issue 1-2-1: Test metric Agreement: - Reuse the test metric for Type-I PMI reporting requirements as the starting point, throughput ratio between the following PMI and random Type-I PMI, where the same Type-I codebook scheme is used for both follow PMI and random PMI throughput measurements.

From initial simulation results summary [6] in last RAN4#116bis meeting, we could observe that the SNR working points at 90% of the maximum throughput are well aligned for typeI-SinglePanel-r19, while whether the SNR working points at 90% of the maximum throughput for eTypeII-r19 are well aligned or not need further check since too limited samples. If the SNR working points could be aligned, the gamma values should also be aligned consequently. However, we see obvious difference for gamma value among companies. Therefore, it’s better to clarify the assumptions for gamma calculation, e.g., whether additional margin considered for SNR working points at 90% of the maximum throughput, whether additional margin considered for gamma values, etc. If both SNR working points and gamma values could be aligned among companies, we prefer to use the solution option 1 (modify the legacy test metric with additional X dB) which could align with existing PMI reporting performance requirements in principle.

According our simulation results in sub-sections 3.2 and 3.3, legacy test metric (i.e., X = 0dB) still works. Moreover, the difference between eTypeII-r19 and typeI-SinglePanel-r19 could be clearly captured by legacy test metric. Consequently, it seems no need to modify legacy test metric for both eTypeII-r19 and typeI-SinglePanel-r19 with 64 CSI-RS ports PMI reporting requirements.

Observation 1: Based on the initial simulation results summary in RAN4#116bis meeting, the SNR working points at 90% of the maximum throughput are well aligned for typeI-SinglePanel-r19, while whether the SNR working points at 90% of the maximum throughput for eTypeII-r19 are well aligned or not need further check since too limited samples.

Proposal 1: Clarify the assumptions for gamma calculation, e.g., whether additional margin considered for SNR working points at 90% of the maximum throughput (i.e., throughput of random precoding case used for gamma calculation is get at ideal SNR working point or with some margin SNR point), whether additional margin considered for gamma values, etc.

Observation 2: Based on our simulation results, legacy test metric still workable for both eTypeII-r19 and typeI-SinglePanel-r19 with 64 CSI-RS ports PMI reporting requirements.

Proposal 2: If legacy test metric doesn’t work for Rel-19 codebooks, prefer to use the solution option 1 (modify the legacy test metric with additional X dB) which could align with existing PMI reporting performance requirements in principle.

Round-off operation of MIMO correlation matrices

In current spec, the scaling factor a is used to obtain a positive semi-definite correlation matrix. A channel correlation matrix must be positive semi-definite because it is fundamentally a covariance matrix. Covariance matrices, by their mathematical defination and physical interpretation, are also positive semi-definite. This property guarantees that the calculated power or variance of any signal projection is non-negative, which is a physical necessity. Hence we need to promise the correlation matrix as a positive semi-definite correlation matrix. We get the scaling factor by following steps according TS38.101-4 B.2.3.1.2 and B.2.3.2.2:

1) Calculate Rspat (with full precision)

2) Loop the value of the scaling factor a from zero with step size 10^(-5)

3) Calculate Rmedium = [Rspat+a*In]/(1+a)  (with full precision)

4) Round the elements of Rmedium to 4 digits

5) Check if the the eig values of Rmedium are all no less than 0. If no, go back to step 2; if yes, select a as the final scaling factor.

Per our understanding, there are two reasons for introducing the round-off operation (defined in step 4):

- The first one is for the convenience of specification, i.e., list the values of MIMO correlation matrices in tables.

- The second one is MIMO correlation matrices implemented in TE could not be infinite digits since real device implementation.

The first reason is not needed any more for 64 CSI-RS ports since we agreed to introduce formala based MIMO correlation matrices. However, the second reason still need to be considerd because infinite digits could not be implemented by real device. Once the round-off operation exist, a scaling factor should be applied to guarantee the correlation matrix as a positive semi-definite correlation matrix.

Proposal 3: Include round-off operation in spec to keep align with real device implementation considering infinite digits could not be implemented by real device.

- Simulation for PMI reporting performance

In this section, we provide our simulation results of PMI reporting performance for both typeI-SinglePanel-r19 and eTypeII-r19, based on the simulation assumptions agreed in previous meetings.

  - Simulation assumptions

According agreements in RAN4#115, RAN4#116 and RAN4#116bis meetings [3][4][5], the simulation assumptions for PMI reporting performance test could be summarized as Table 3-1.

Table 3-1. Simulation Assumptions for typeI-SinglePanel-r19 and eTypeII-r19 performance test

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | FDD: 10 TDD: 40 |
| Subcarrier spacing |  | kHz | FDD: 15 TDD: 30 |
| Duplex Mode |  |  | FDD and TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | XP-Medium 64 x 2 and 64x4 (N1,N2) = (8,4) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| NZP CSI-RS for CSI acquisition | Number of aggregated NZP CSI-RS resources (K) |  | 2 |
|  | portMappingMethod |  | Method 1 |
|  | Number of slots for CSI-RS transmission |  | 1 |
|  | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 64 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS resource 1 (l0, l1) |  | Row 17,(2, 9) |
|  | First OFDM symbol in the PRB used for CSI-RS resource 2 (l0, l1) |  | Row 17,(5, 12) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (5,7) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband for typeI-SinglePanel-r19 Not configured for eTypeII-r19 |
| Sub-band Size |  | RB | FDD: 8 TDD: 16 |
| csi-ReportingBand |  |  | 1111111111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | FDD: 7 TDD: 11 |
| CSI request |  |  | FDD: 1 in slots i, where mod(i, 5) = 1, TDD: 1 in slots i, where mod(i, 10) = 6, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel-r19 and eTypeII-r19 |
|  | Codebook Mode |  | Scheme A |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (8,4) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | paramCombination-r19 |  | 6 (L =4, pν =1/2, β=1/2) |
|  | CodebookSubsetRestriction |  | 0x FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | FDD: 11 TDD: 9 |
| Maximum number of HARQ transmission |  |  | 4 |
| MCS value |  |  | MCS22 (Table 1) |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: For FDD, If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-7), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). For TDD, If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-12), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3 and Annex B.2.3.2.3A. |  |  |  |

  - Simulation results for FDD cases

PMI reporting performance for FDD 2Rx

For FDD with 64Tx and 2Rx antennas cases, the relative throughputs and gammas for following PMI eTypeII-r19, following PMI TypeI-SinglePanel-r19 and Random PMI with MCS22 (Table 1) are captured as Figure 3-1, and the ideal SNR working points at 90% maximum throughput are summarized in Table 3-2. Accordingly, the gamma value with legacy definition as below could be summarized in Table 3-3.

Legacy test metric gamma definition for PMI reporting requirements

γ=tuetrnd tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trndis the throughput measured at SNRue with random precoding.

Figure 3-1. Relative throughputs and gammas comparison for FDD 64T2R2Layer

Table 3-2. Ideal SNR working points at 90% maximum throughput for FDD 64T2R2Layer

| Test num. | Codebook | SNR (dB) @ 90% maximum throughput |
| --- | --- | --- |
|  |  | MCS22 |
| 1-1 | eTypeII-r19 | 2.3 |
| 1-2 | TypeI-SP-r19 | 9.8 |

Table 3-3. Gamma value for FDD 64T2R2Layer

| Test num. | Codebook | Gamma value@ SNRs |  |  |
| --- | --- | --- | --- | --- |
|  |  | Ideal SNR | Impairment margin SNR | Ideal SNR+5dB |
| 1-1 | eTypeII-r19 | 262.3 | 89.9 | 35.9 |
| 1-2 | TypeI-SP-r19 | 13.4 | 7.7 | 5.1 |

PMI reporting performance for FDD 4Rx

For FDD with 64Tx and 4Rx antennas cases, the relative throughputs and gammas for following PMI eTypeII-r19, following PMI TypeI-SinglePanel-r19 and Random PMI with MCS22 (Table 1) are captured as Figure 3-2, and the ideal SNR working points at 90% maximum throughput are summarized in Table 3-4. Accordingly, the gamma value with legacy definition could be summarized in Table 3-5.

Figure 3-2. Relative throughputs and gammas comparison for FDD 64T4R2Layer

Table 3-4. Ideal SNR working points at 90% maximum throughput for FDD 64T4R2Layer

| Test num. | Codebook | SNR (dB) @ 90% maximum throughput |
| --- | --- | --- |
|  |  | MCS22 |
| 2-1 | eTypeII-r19 | -0.6 |
| 2-2 | TypeI-SP-r19 | 2.6 |

Table 3-5. Gamma value for FDD 64T4R2Layer

| Test num. | Codebook | Gamma value@ SNRs |  |  |
| --- | --- | --- | --- | --- |
|  |  | Ideal SNR | Impairment margin SNR | Ideal SNR+5dB |
| 2-1 | eTypeII-r19 | 169.8 | 47.6 | 22.0 |
| 2-2 | TypeI-SP-r19 | 33.7 | 18.3 | 9.9 |

From above simulation results, we could observe that the 90% SNR working points for 2Rx are about 2.3/9.8dB separately for eTypeII-r19 and TypeI-SinglePannel-r19, and for 4Rx are about -0.6/2.6dB separately for eTypeII-r19 and TypeI-SinglePannel-r19. In addition, it seems the legacy test metric without additional margin XdB is still workable for both eTypeII-r19 and TypeI-SinglePannel-r19 codebooks.

  - Simulation results for TDD cases

PMI reporting performance for TDD 2Rx

For TDD with 64Tx and 2Rx antennas cases, the relative throughputs and gammas for following PMI eTypeII-r19, following PMI TypeI-SinglePanel-r19 and Random PMI with MCS22 (Table 1) are captured as Figure 3-3, and the ideal SNR working points at 90% maximum throughput are summarized in Table 3-6. Accordingly, the gamma value with legacy definition as below could be summarized in Table 3-7.

Figure 3-3. Relative throughputs and gammas comparison for TDD 64T2R2Layer

Table 3-6. Ideal SNR working points at 90% maximum throughput for TDD 64T2R2Layer

| Test num. | Codebook | SNR (dB) @ 90% maximum throughput |
| --- | --- | --- |
|  |  | MCS22 |
| 3-1 | eTypeII-r19 | 1.8 |
| 3-2 | TypeI-SP-r19 | 9.4 |

Table 3-7. Gamma value for TDD 64T2R2Layer

| Test num. | Codebook | Gamma value @ SNRs |  |  |
| --- | --- | --- | --- | --- |
|  |  | Ideal SNR | Impairment margin SNR | Ideal SNR+5dB |
| 3-1 | eTypeII-r19 | 230.2 | 88.4 | 33.0 |
| 3-2 | TypeI-SP-r19 | 13.3 | 8.2 | 5.3 |

PMI reporting performance for TDD 4Rx

For TDD with 64Tx and 4Rx antennas cases, the relative throughputs and gammas for following PMI eTypeII-r19, following PMI TypeI-SinglePanel-r19 and Random PMI with MCS22 (Table 1) are captured as Figure 3-4, and the ideal SNR working points at 90% maximum throughput are summarized in Table 3-8. Accordingly, the gamma value with legacy definition as below could be summarized in Table 3-9.

Figure 3-4. Relative throughputs and gammas comparison for TDD 64T4R2Layer

Table 3-8. Ideal SNR working points at 90% maximum throughput for TDD 64T4R2Layer

| Test num. | Codebook | SNR (dB) @ 90% maximum throughput |
| --- | --- | --- |
|  |  | MCS22 |
| 4-1 | eTypeII-r19 | -1.2 |
| 4-2 | TypeI-SP-r19 | 2.1 |

Table 3-9. Gamma value for TDD 64T4R2Layer

| Test num. | Codebook | Gamma value @ SNRs |  |  |
| --- | --- | --- | --- | --- |
|  |  | Ideal SNR | Impairment margin SNR | Ideal SNR+5dB |
| 4-1 | eTypeII-r19 | 175.0 | 55.4 | 22.9 |
| 4-2 | TypeI-SP-r19 | 33.4 | 18.4 | 9.9 |

From above simulation results, we could observe that the 90% SNR working points for 2Rx are about 1.8/9.4dB separately for eTypeII-r19 and TypeI-SinglePannel-r19, and for 4Rx are about -1.2/2.1dB separately for eTypeII-r19 and TypeI-SinglePannel-r19. In addition, it seems the legacy test metric without additional margin XdB is still workable for both eTypeII-r19 and TypeI-SinglePannel-r19 codebooks.

- Conclusion

In this contribution, we provide analysis and simulation results on UE PMI reporting performance for Rel-19 NR MIMO Phase 5 WI.

The proposals and observations could be summarized as:

Observation 1: Based on the initial simulation results summary in RAN4#116bis meeting, the SNR working points at 90% of the maximum throughput are well aligned for typeI-SinglePanel-r19, while whether the SNR working points at 90% of the maximum throughput for eTypeII-r19 are well aligned or not need further check since too limited samples.

Proposal 1: Clarify the assumptions for gamma calculation, e.g., whether additional margin considered for SNR working points at 90% of the maximum throughput (i.e., throughput of random precoding case used for gamma calculation is get at ideal SNR working point or with some margin SNR point), whether additional margin considered for gamma values, etc.

Observation 2: Based on our simulation results, legacy test metric still workable for both eTypeII-r19 and typeI-SinglePanel-r19 with 64 CSI-RS ports PMI reporting requirements.

Proposal 2: If legacy test metric doesn’t work for Rel-19 codebooks, prefer to use the solution option 1 (modify the legacy test metric with additional X dB) which could align with existing PMI reporting performance requirements in principle.

Proposal 3: Include round-off operation in spec to keep align with real device implementation considering infinite digits could not be implemented by real device.

- Reference

[1] RP-234007, New WID: NR MIMO Phase 5, RAN#102 meeting, Samsung

[2] RP-242394, WID revision: NR MIMO Phase 5, RAN#105 meeting, Samsung

[3] R4-2508627, Way Forward for [115][329] NR_MIMO_demod, RAN4#115 meeting, Samsung

[4] R4-2512527, Way Forward for [116][327] NR_MIMO_demod, RAN4#116 meeting, Samsung

[5] R4-2514806, WF on NR_MIMO_demod, RAN4#116bis meeting, Samsung

[6] R4-2514841, Simulation results summary for 64 CSI-RS ports PMI reporting requirements, RAN4#116bis meeting, Samsung