# 3GPP TSG-RAN WG4 Meeting #117	R4-2521047

# Dallas, US, 17th – 21st Oct 2025

Agenda item:	8.8.2

Source:	China Telecom

Title:	Discussion on 6G AI/ML study for RAN4 RRM driven use cases

Document for:	Discussion

- Introduction

In RAN4 #116bis meeting, RAN4 6G AI requirements were discussed and the WF [1] was approved.

In this paper, we present our views on RAN4 6G AI requirements captured in the WF [1].

- Discussion

Status in the WF [1]:

Issue 6-1c: Additional principle/method for AI-RRM use case selection Agreement: - Additional principle/method for AI-RRM use case selection - Whether the proposed AI-RRM use case should be driven by RAN4: - Existing 5G-A use cases included in RAN2 and RAN1 WIs should be precluded in RAN4 6G study. - Coordination with RAN1 and RAN2 when potential AI-RRM use cases are agreed in RAN4 to avoid overlapping scope. Issue 6-1a: Use case definition and supporting level from companies Agreement: - The scope of potential RAN4-led AI-RRM use case(s) will be further discussed in the next meeting. For information: - Below table is provided to summarize AI-RRM proposals in RAN4#116bis. - For each use case, preference/opposition from companies will be collected in future meeting for down-selection.

| Index | use cases | Model location (Training method) | Proposed by companies | Expected benefits (e.g., RS reduction, MG reduction etc.) | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- | --- |
|  | L1 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Spatial domain (inter-cell) - Time and spatial domain (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c), Nokia ((d, e), as RAN1-led study), Apple (c, d, e) |  |  |  |
|  | L3 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Time and freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) - Freq. domain prediction for N carriers single cell | NW and UE sided model (Offline training) | OPPO (a, b, c, e), Apple (c,d, e), CMCC (e), ZTE (e), Samsung (e, f), Nokia (c, d, e) |  |  |  |
|  | L3 cell measurement: - Time domain (intra-cell) - Freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c) Apple (c), Nokia (c) |  |  |  |
|  | Multi-Domain L3 level Prediction for Measurement-Gap Reduction | UE sided model (Offline training) | Apple, vivo, Samsung |  |  |  |
| 5a (5+6 combined) | Dynamic Adaptation of Measurement Procedure | UE sided model | MediaTek, Qualcomm, Apple |  |  |  |
| 7a (7+8 combined) | Spatial domain RX beam sweeping reduction (L1/L3) | UE sided model | Apple (L1 for scheduling restriction and delay reductions), vivo Qualcomm, Apple, CMCC, Xiaomi |  |  |  |
|  | AI/ML based RLM/BFD/CBD, LTM by measurement prediction | UE-sided model | OPPO, vivo (AI for RLM) |  |  |  |
|  | AI/ML based SCell related enhancement by measurement prediction | UE-sided model | OPPO |  |  |  |
|  | AI-based prediction for enhanced s-measure mechanism | UE-sided model | vivo |  |  |  |
|  | AI/ML based AGC through temporal/frequency domain prediction | UE-sided model | ZTE |  |  |  |
|  | AI/ML based DL sync through temporal/frequency domain prediction | UE-sided model | ZTE |  |  |  |
|  | AI abstraction of the wireless environment to facilitate data scheduling and network power saving | NW-sided model | CATT |  |  |  |

For L3 beam measurement, we have discussed a lot in AI Mob in Rel-19 SI. But most companies have negative attitude on non-collocated scenario prediction due to the low spatial correlation between the two frequencies, so inter-frequency prediction starts evaluation from co-located scenario.

In this case, Network must configure measurement gap even if there is only one MO which needs MG. For inter-frequency measurement, the common case is that measurement gap is needed. Only consider co-located scenario, the benefit of reducing measurement gap configuration is limited. It is necessary to study the AI/ML based frequency domain measurement prediction for non-co-located scenario.

Proposal 1: Consider the use case of AI/ML based frequency domain measurement prediction for both co-located and non-collocated cell prediction, co-located cell prediction can be as starting point.

In our understanding, the 6G new spectrum will be higher compared with main 5G spectrum FR1(410MHz - 6.425 GHz). It could be seen that beam sweeping will be more important for high frequency range.

With AI/ML based UE RX beam prediction, it is expected that RX beam sweeping factor could be reduced. With reduced RX beam sweeping factor, a lot of delays, such as cell identification, SCell activation, handover, PSCell addition etc. could be reduced, which is beneficial for system.

Proposal 2: Consider the use case of AI/ML based UE RX beam prediction, to reduce or eliminate RX beam sweeping factor.

RRM measurements on serving and neighbour cells are performed by the UE both in idle/inactive and connected modes. Measurements are almost involved in all RRM procedures and measurement gap (MG) is important. In this case, trade-off between measurement overhead and mobility performance is necessary. MGRP is large, the measurement overhead is lower, while the measurement period becomes long, the subsequent mobility action will be delayed, but short MGRP will lead to significant measurement overhead. So, the use case of MG reduction can be further studied in 6G AI-RRM.

Proposal 3: Consider the use case of MG reduction.

- Conclusion

The contribution presents our views on RAN4 6G RRM requirements, with the following proposals:

Proposal 1: Consider the use case of AI/ML based frequency domain measurement prediction for both co-located and non-collocated cell prediction, co-located cell prediction can be as starting point.

Proposal 2: Consider the use case of AI/ML based UE RX beam prediction, to reduce or eliminate RX beam sweeping factor.

Proposal 3: Consider the use case of MG reduction.

- References

- R4-2514637, WF on 6G AI, Samsung, RAN WG4 Meeting #116bis, 13 – 17 October 2025.