3GPP TSG-RAN WG4 Meeting #117	                                   R4-2521150

Dallas, US, 17th November – 21st November, 2025

Agenda item:	7.4.3.2

Source: 	ZTE Corporation

Title: 	Discussion on RRM requirements for (e)RedCap

Document for:	Approval

Introduction

In RAN#109 meeting, the WID on NR Radio Resource Management (RRM) Phase 6 has been approved [1]. Core part objectives are shown below for reference:

- Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD - For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026) [RAN4] - Note 3: No new PDSCH demodulation requirement shall be defined.

In this contribution, we will provide our views on the RRM requirements for (e)RedCap UE with Relaxed RLM/BFD and/or less than 5MHz channel bandwidth.

Discussion

From the description of WI, there are two bullets need to be discussed in R20. for the first bullet, power saving is important for (e)RedCap UE and existing Rel-17 RLM/BFD relaxation for leagcy UE can be applied to (e)RedCap UE for power saving. Meanwhile, to reduce complexity and cost, it is beneficial for (e)RedCap UE to support less than 5 MHz channel bandwidth. For the second bullet, fast SCell activation for intra-band co-located contiguous CA should be considered. According to the work plan which has been agreed in the last meeting [2], we should discuss the first issue in this stage and the second issue will be discussed on Q4, 2026.

2.1 Relaxed RLM/BFD RRM requirements

Before discussing how to apply relaxed requirements to RedCap UE, let's first review the descriptions of RedCap UE related requirements in existing specification.

For legcay UE, both SSB based and CSI-RS based RLM/BFD are considered for relax measurement. Meanwhile, for redcap UE, Requirements for SSB based radio link monitoring and CSI-RS based radio link monitoring are defined. Based on this, both SSB based and CSI-RS based RLM/BFD relax should be considered for RedCap UE.

|  | SSB based RLM/BFD | CSI-RS based RLM/BFD | SSB based RLM/BFD relaxation | CSI-RS based RLM/BFD relaxation |
| --- | --- | --- | --- | --- |
| Legcay UE | 8.1.2.2/8.5.2.2 | 8.1.3.2/8.5.3.2 | 8.1.2.4/8.5.2.4 | 8.1.3.4/8.5.3.4 |
| RedCap UE | 8.1B.2.2/8.5B.2.2 | 8.1B.3.2/8.5B.3.2 | TBD | TBD |

Proposal 1: Both SSB based and CSI-RS based RLM/BFD relaxation should be considered for (e)RedCap UE.

In the last meeting, we have reached consensus that the existing R17 RLM/BFD relaxation requirements can be reused for both FR1 and FR2, and for full-duplex FDD and TDD. Meanwhile, RAN4 only considers the relaxed RLM/BFD requirements on PCell when the DRX cycle is no longer than 80 ms.

Issue 2-2-1: Scope of relaxed RLM/BFD for (e)RedCap UE Agreement: RAN4 only considers the relaxed RLM/BFD requirements on PCell when the DRX cycle is no longer than 80 ms. Issue 2-2-2: Relaxed RLM/BFD for 2Rx (e)RedCap UE Agreement: - Reuse the existing R17 RLM/BFD relaxation requirements for both FR1 and FR2, and for full-duplex FDD and TDD.

For 1Rx (e)RedCap UE, we should further discuss whether the R17 RLM/BFD relaxation requirements can be use as baseline. According to current spec and agreements, the RLM/BFD relaxation requirements for normal UE or 2Rx (e)RedCap UE are defined as blew:

Table 8.1.2.4-1: Evaluation period TEvaluate_out_SSB_Relax for FR1

| Configuration | TEvaluate_out_SSB_Relax (ms) |
| --- | --- |
| Max(TDRX,TSSB) ≤80 ms | Max(200 K3 NOTE3, Ceil(15 K1NOTE2 P) Max(TDRX,TSSB)) |
| 80 ms < Max(TDRX,TSSB) ≤160 ms | Ceil(15 P) Max(TDRX,TSSB) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80 ms. NOTE 2: K1 = 4 for Max(TDRX,TSSB) ≤40 ms and K1 = 2 for 40 ms<Max(TDRX,TSSB) ≤80 ms. NOTE 3: K3 = K1, if K1 ≤ 2; otherwise K3 = 1. |  |

Table 8.1.3.4-1: Evaluation period TEvaluate_out_CSI-RS_Relax for FR1

| Configuration | TEvaluate_out_CSI-RS_Relax (ms) |
| --- | --- |
| Max(TDRX, TCSI-RS) ≤ 80 ms | Max(200 K3 NOTE3, Ceil(1.5 Mout P K1 NOTE2) Max(TDRX, TCSI-RS) NOTE1) |
| NOTE1: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length and no longer than 80 ms. NOTE2: K1 = 2 for 40 ms < MAX(TDRX, TCSI-RS) ≤ 80 ms, K1 = 4 for MAX(TDRX, TCSI-RS) ≤ 40 ms NOTE3: K3 = K1, if K1 ≤ 2; K3 = 1 otherwise. |  |

When Max(TDRX,TSSB) ≤40, k1=4, k3=1, and TEvaluate_out_SSB_Relax equals to Max(200, Ceil(60  P)  Max(TDRX,TSSB)). when 40 ms<Max(TDRX,TSSB) ≤80 ms, k1=2, k3=2, and TEvaluate_out_SSB_Relax equals to Max(400, Ceil(30  P)  Max(TDRX,TSSB)). Meanwhile, for CSI-RS based RLM, k1=4, k3=1, and TEvaluate_out_CSI-RS_Relax equals to Max(200, Ceil(6 Mout  P)  Max(TDRX,TSSB)) if Max(TDRX,TSSB) ≤40; k1=2, k3=2, and TEvaluate_out_CSI-RS_Relax equals to Max(400, Ceil(3 Mout  P)  Max(TDRX,TSSB)) if 40 ms<Max(TDRX,TSSB) ≤80 ms.

For 1Rx, even without measurement relaxation, the current evaluation period for DRX cycles less than 80ms is similar to the normal UE or 2Rx (e)RedCap UE that has already undergone measurement relaxation. If we relax the evaluation period again based on the current evaluation period of 1Rx, it may cause even greater delays, potentially impacting link reliability.

Table 8.1B.2.2-3: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 1Rx RedCap UE

| Configuration | TEvaluate_out_SSB,RedCap (ms) | TEvaluate_in_SSB,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(400, Ceil(20 P) TSSB) | Max(100, Ceil(5 P) TSSB) |
| DRX cycle≤320 ms | Max(400, Ceil(30 P) Max(TDRX,TSSB)) | Max(100, Ceil(7.5 P) Max(TDRX,TSSB)) |
| DRX cycle>320 ms | Ceil(20 P) TDRX | Ceil(5 P) TDRX |
| NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length. |  |  |

Table 8.1B.3.2-3: Evaluation period TEvaluate_out_CSI-RS,RedCap and TEvaluate_in_CSI-RS,RedCap for FR1 for 1Rx RedCap UE

| Configuration | TEvaluate_out_CSI-RS,RedCap (ms) | TEvaluate_in_CSI-RS,RedCap (ms) |
| --- | --- | --- |
| no DRX | Max(400, Ceil(Mout,RedCap×P)×TCSI-RS) | Max(100, Ceil(Min,RedCap×P) × TCSI-RS) |
| DRX ≤ 320 ms | Max(400, Ceil(1.5×Mout,RedCap ×P)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min,RedCap×P)× Max(TDRX, TCSI-RS)) |
| DRX > 320 ms | Ceil(Mout,RedCap ×P) × TDRX | Ceil(Min,RedCap ×P) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

Observation 1: Even without measurement relaxation for 1Rx (e)RedCap UE, the current evaluation period for DRX cycles less than 80ms is similar to the normal UE or 2Rx (e)RedCap UE that has already undergone measurement relaxation. And if the evaluation period will be relaxed based on the current evaluation period of 1Rx, it may cause even greater delays.

Proposal 2: For 1Rx (e)RedCap UEs, RAN4 should discuss whether measurement relaxation is necessary.

2.2 Support of less than 5 MHz channel bandwidth

From the WID, we should discuss RRM requirements for RedCap UE when less than 5 MHz channel bandwidth is supported. And R18 3MHz requirements for non-RedCap UE should be uesed as a baseline. In Rel-18, the impact to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz has been discussed for single carrier and relative RRM requirements were updated. For the case that CBW equals to 3MHz, PBCH DMRS detection and PBCH decoding maybe impact due to the transmission bandwidth for PBCH is smaller.

In the last meeting, we have reached consensus that there is no need to consider CSI-RS based measurement, and only define the requirements for NR FR1 PCell, which is same as less than 5MHz in Rel-18. Naturally, under the condition of bandwidth less than 5M, the requirements of legacy UE can be applied to 2Rx RedCap UE.

According to the current specification, we have summarized the impact of R18 less than 5MHz on the core requirements of legacy UE, as shown in the table below.

Impact of R18 less than 5MHz on the core requirements of legacy UE

| NR FR1 - NR FR1 Handover | T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW. |
| --- | --- |
| PDCCH transmission parameters of RLM | Number of control OFDM symbols Aggregation level (CCE) Bandwidth (PRBs) Mapping from REG to CCE |
| Time period for time index detection | In Rel-18, SSB index identification delay Unknown intra-frequency cell (Es/Iot≥-6 dB): 3+4 samples Unknown inter-frequency cell (Es/Iot≥-4 dB): 3+3 samples |

Based on these, we have the agreement blew in the last meeting:

Issue 2-1-2: LT5 for 2Rx (e)RedCap UE Agreement: - The existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with full-duplex FDD, TDD modes. - Handover - RLM/BFD - Intra-frequency/Inter-frequency measurement - Further check for HD-FDD

As to the HD-FDD, we check the current description of requirements mentioned above, i.e., handover, RLM/BFD and intra-frequency/inter-frequency measurement. Firstly, we should discuss whether the requirements for normal UE with LT5 can be reused for the 2Rx RedCap with HD-FDD. To my knowledge, HD-FDD means that only UL or DL transmission is available at this time. For certain requirements, we should add additional words to ensure that there are available RSs for DL reception.

Handover:

The handover requirments for RedCap UE are listed in clause 6.1D.1.2, applied for all modes, including FD-FDD, TDD and HD-FDD. We should pay attention that there are some side conditions for RedCap UE with HD-FDD. Unlike FD-FDD UE, which can operate on both the receiving frequency (downlink) and the transmitting frequency (uplink) at the same time, HD-FDD UE cannot receive and transmit simultaneously. It can only operate on either the receiving frequency or the transmitting frequency at a certain moment, and it needs to switch between receiving and sending. Therefore, for FD-FDD UE, it is necessary to consider the possibility of missing downlink signal reception during the uplink transmission period. Under this consideration, it is necessary to provide a reliable receiving window for HD-FDD UE.

For RedCap UE with HD-FDD, the handover requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU.

Therefore, for 2Rx (e)RedCap UE with half-duplex mode, additional clarfication is needed to gurateen the SSBs are available.

Observation 2: For (e)RedCap UE with HD-FDD, the handover requirements are met if the SSBs are available in the time duration.

Proposal 3: For 2Rx (e)RedCap UE with HD-FDD, the existing handover requirements for non-RedCap UE with LT5 can be used with the conditions blew:

-	SSB is available at the UE once every SMTC period during Tsearch

-	Three SSBs are available during T∆

-	One SSB is available during TIU.

RLM/BFD

For this part, the different between HD-FDD and FD-FDD/TDD RedCap UE is only the minimum requirement for L1 indication, which will not be impacted by LT5. therefore, for HD-FDD RedCap UEs with LT5, Minimum requirement for L1 indication in clause 8.1B.6 can be reused.

### 8.1B.6 Minimum requirement for L1 indication When the downlink radio link quality on all the configured RLM-RS resources is worse than Qout,RedCap, layer 1 of the UE shall send an out-of-sync indication for the cell to the higher layers. A layer 3 filter shall be applied to the out-of-sync indications as specified in TS 38.331 [2]. When the downlink radio link quality on at least one of the configured RLM-RS resources is better than Qin,RedCap, layer 1 of the UE shall send an in-sync indication for the cell to the higher layers. A layer 3 filter shall be applied to the in-sync indications as specified in TS 38.331 [2]. The out-of-sync and in-sync evaluations for the configured RLM-RS resources shall be performed as specified in clause 5 in TS 38.213 [3]. Two successive indications from layer 1 shall be separated by at least TIndication_interval,RedCap. When DRX is not used for RedCap UEs, TIndication_interval,RedCap is max(10 ms, TRLM-RS,M), where TRLM,M is the shortest periodicity of all configured RLM-RS resources for the monitored cell, which corresponds to TSSB specified in clause 8.1B.2 if the RLM-RS resource is SSB, or TCSI-RS specified in clause 8.1B.3 if the RLM-RS resource is CSI-RS. In case DRX is used for RedCap UEs, TIndication_interval,RedCap is Max(10ms, 1.5 × DRX_cycle_length, 1.5 × TRLM-RS,M)) if DRX_cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX_cycle_length is greater than 320ms. Upon start of T310 timer as specified in TS 38.331 [2], the UE shall monitor the configured RLM-RS resources for recovery using the evaluation period and layer 1 indication interval corresponding to the no DRX mode until the expiry or stop of T310 timer. When DRX is not used for HD-FDD RedCap UEs, TIndication_interval,RedCap is max(10 ms, TRLM-RS,M), where TRLM,M is the shortest periodicity of all configured RLM-RS resources for the monitored cell, which corresponds to TSSB specified in clause 8.1B.2 if the RLM-RS resource is SSB, or TCSI-RS specified in clause 8.1B.3 if the RLM-RS resource is CSI-RS, under the following condition - For each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period. In case DRX is used for HD-FDD RedCap UEs, TIndication_interval,RedCap is Max(10ms, 1.5 × DRX_cycle_length, 1.5 × TRLM-RS,M)) if DRX_cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX_cycle_length is greater than 320ms, under the following condition - For each RLM-RS configuration, at least one RLM-RS sample must fall with DL occasion within an indication period.

Observation 3: For Radio Link Monitoring, the different between HD-FDD and FD-FDD/TDD (e)RedCap UE is only the minimum requirement for L1 indication, which will not be impacted by LT5.

- Intra-frequency/Inter-frequency measurement

##### 9.2B.5.3.4 Scheduling availability of HD-FDD UE performing measurements on FR1 When the HD-FDD UE performs intra-frequency measurements in a FR1 band, the following restrictions apply due to SS-RSRP or SS-SINR measurement - If deriveSSB-IndexFromCell is enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1. - If deriveSSB-IndexFromCell is not enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1. When the HD-FDD UE performs intra-frequency measurements in a FR1 band, the following restrictions apply due to SS-RSRQ measurement - If deriveSSB-IndexFromCell is enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1. - If deriveSSB-IndexFromCell is not enabled, the UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

Observation 4: For intra/inter-frequency measurement requirements, the different between HD-FDD and FD-FDD/TDD RedCap UE is only the scheduling restriction, which will not be impacted by LT5.

Proposal 5: For 2Rx (e)RedCap UE with HD-FDD, the existing RLM/BFD and intra/inter-frequency measurement requirements for non-RedCap UE with LT5 can be reused.

Issue 2-1-4: RLM/BFD impact of LT5 for 1Rx (e)RedCap UE Agreement: For discussing the PDCCH hypothetical parameters, RAN4 discuss whether it is feasible to target on coverage equivalence between 1Rx and 2Rx RedCap UEs.

According to the WF, we should discuss the feasibility on coverage equivalence between 1Rx and 2Rx (e)RedCap UEs. If 1Rx and 2Rx have the same performance on coverage, the methods can be consiered like blew:

- Add BW for 1Rx

- Add OFDM symbols for 1Rx

- Reduce CCE(Aggregation level) for 1Rx

In legacy, different BWs are used for 1Rx (e)RedCap and 2Rx (e)RedCap to ensure the coverage is the same. Specifically, 24PRB are used for RedCap UE with 2Rx branches and 48 PRB are used for RedCap UE with 1Rx branch. The number of OFDM symbols is 2 for both 1Rx and 2Rx. Meanwhile, CCE is16 for 1Rx and 8 for 2Rx.

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 16; for RedCap UE with 1Rx branch. 8; for RedCap UE with 2Rx branches. |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4 dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4 dB |
| Bandwidth (PRBs) | 48; for RedCap UE with 1Rx branch. 24; for RedCap UE with 2Rx branches. |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |
| NOTE: SCS = 60kHz is not applicable for FR1. |  |

If LT5 is applied for RedCap UE, BW is fixed with 15PRB. Based on this, if the same coverage want to be achieved, OFDM symbols should be added and/or CCE should be reduced. For RedCap UE with 2Rx branches, the requirements of normal UE will be reused and it means that PDCCH hypothetical parameters are used for 2Rx RedCap UE with LT5. It can be observed that the number of control OFDM symbols for normal UE with LT5 has been added to 3. meanwhile, according to the design of other WG, the maximum value of OFDM symbols in a control resource set is three, which means it is not feasible to reach the coverage equivalence just through adding the OFDM symbols for 1Rx.

ControlResourceSet ::= SEQUENCE { controlResourceSetId ControlResourceSetId, frequencyDomainResources BIT STRING (SIZE (45)), duration INTEGER (1..maxCoReSetDuration), cce-REG-MappingType CHOICE { interleaved SEQUENCE { reg-BundleSize ENUMERATED {n2, n3, n6}, interleaverSize ENUMERATED {n2, n3, n6}, shiftIndex INTEGER(0..maxNrofPhysicalResourceBlocks-1) OPTIONAL -- Need S }, nonInterleaved NULL }, precoderGranularity ENUMERATED {sameAsREG-bundle, allContiguousRBs maxCoReSetDuration INTEGER ::= 3 -- Max number of OFDM symbols in a control resource set

Observation 5: The maximum value of OFDM symbols in a control resource set is 3, and the number of control OFDM symbols for normal UE with LT5 has been added to 3.

Based on the above analysis, if we want to get the same coverage between 1Rx (e)RedCap and 2Rx (e)RedCap, Aggregation level (CCE) should be considered to modify for 1Rx. If the number of OFDM symbols is increased from 2 to 3 while considering the reduction of CCE from 16 to 8, the same coverage between 1Rx (e)RedCap and 2Rx (e)RedCap may be achieved. However, We have some concern here. The higher the CCE, the greater the redundancy of information. Compared to 2Rx, the transmission of 1Rx is unstable. Therefore, in order to improve the reliability of transmission, a higher aggregation level is adopted. So, is it feasible to set the same aggregation level for the 1Rx (e)RedCap and 2Rx (e)RedCap?

Observation 6: If the number of OFDM symbols is increased from 2 to 3 while considering the reduction of CCE from 16 to 8, the same coverage between 1Rx (e)RedCap and 2Rx (e)RedCap may be achieved.

Proposal 6: RAN4 should discuss whether it is feasible to set the same aggregation level for 1Rx (e)RedCap and 2Rx (e)RedCap.

Conclusions

Proposal 1: Both SSB based and CSI-RS based RLM/BFD relaxation should be considered for (e)RedCap UE.

Observation 1: Even without measurement relaxation for 1Rx (e)RedCap UE, the current evaluation period for DRX cycles less than 80ms is similar to the normal UE or 2Rx (e)RedCap UE that has already undergone measurement relaxation. And if the evaluation period will be relaxed based on the current evaluation period of 1Rx, it may cause even greater delays.

Proposal 2: For 1Rx (e)RedCap UEs, RAN4 should discuss whether measurement relaxation is necessary.

Observation 2: For (e)RedCap UE with HD-FDD, the handover requirements are met if the SSBs are available in the time duration.

Proposal 3: For 2Rx (e)RedCap UE with HD-FDD, the existing handover requirements for non-RedCap UE with LT5 can be used with the conditions blew:

-	SSB is available at the UE once every SMTC period during Tsearch

-	Three SSBs are available during T∆

-	One SSB is available during TIU.

Observation 3: For Radio Link Monitoring, the different between HD-FDD and FD-FDD/TDD RedCap UE is only the minimum requirement for L1 indication, which will not be impacted by LT5.

Observation 4: For intra/inter-frequency measurement requirements, the different between HD-FDD and FD-FDD/TDD RedCap UE is only the scheduling restriction, which will not be impacted by LT5.

Proposal 5: For 2Rx (e)RedCap UE with HD-FDD, the existing RLM/BFD and intra/inter-frequency measurement requirements for non-RedCap UE with LT5 can be reused.

Observation 5: The maximum value of OFDM symbols in a control resource set is 3, and the number of control OFDM symbols for normal UE with LT5 has been added to 3.

Observation 6: If the number of OFDM symbols is increased from 2 to 3 while considering the reduction of CCE from 16 to 8, the same coverage between 1Rx (e)RedCap and 2Rx (e)RedCap may be achieved.

Proposal 6: RAN4 should discuss whether it is feasible to set the same aggregation level for 1Rx (e)RedCap and 2Rx (e)RedCap.

References

- RP-252955, WID: NR Radio Resource Management (RRM) Phase 6, Apple.

- R4-2514821, WF on NR_RRM_Ph6, Ericsson.