# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2521539

# Dallas, USA, 17th – 21th Nov, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for HD-FDD VSAT on Ku band

Agenda item:	7.6.4.2

Document for:	Approval

Introduction

At RAN plenary #108 meeting, the latest WI“Enhanced requirements for NR NTN and IoT NTN Phase 2”[1] was approved. In current WID, the goal is to specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies. The following description states RAN4’s work in future stage:

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability. - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

Discussion

RAN4 #116bis meeting has kicked off the study of HD-FDD VSAT operating in both Ka and Ku bands with FR1 numerology. As WID said, phase 1 work will commence immediately, and the study of phase 2 will commence upon completion of phase 1 and RAN1/RAN2 availability. In line with the latest WID regarding the RRM core parts, RAN4 shall present some views on Phase 1.

  - 2.1 General discussion

It is noted that phase 1 contains two sub-bullets to address distinct scenarios and pertain to different HD-FDD VSAT capabilities.

Regarding the first sub-bullet (for VSAT type 2,3,5,6), it states that RAN4 shall introduce a single set of requirements for HD-FDD for VSAT types 2,3,5,6 equipped with electronically steered antennas. Hence, there is no need to define separate core requirements for these HD-FDD VSAT with electronically steered antenna. That is, when specifying RRM requirements for HD-FDD VSAT UEs, RAN4 shall not differentiate based on whether VSAT UE is fixed or mobile, or whether it is served by GEO or LEO.

Proposal 1: For HD-FDD for VSAT types 2,3,5,6, RAN4 shall define a single set of RRM requirements, regardless of whether the VSAT UE is fixed or mobile, or served by GEO or LEO satellites.

  - 2.2 RRM core requirements

The latest WID states leveraging the RedCap NTN HD-FDD solution for NTN HD-FDD VSAT where applicable. We understand this as a directive to use the established RedCap NTN HD-FDD core requirements as a starting point for defining the RRM requirements for VSAT. Thus, the first step is to systematically review the RedCap requirements to ensure consistency when applying the same logic to the VSAT types 2,3,5,6 (Case 1). The agreements from the last meeting are listed below:

Agreement - Define the HD-FDD VSAT with FR1 numerology RRM requirements based on the Rel-19 Ku-band NTN FDD requirements, and incorporate updates to address HD-FDD impacts, using the RedCap NTN approach as the baseline. - The following RRM requirements scope for HD-FDD VSAT in Ku band with FR1 numerology - Cell reselection, including paging reception - Handover and CHO - Satellite switching - Random access - UE UL timing accuracy - RLM, including scheduling availability and L1 indication - intra/inter-frequency measurements, including scheduling availability - L1-RSRP measurement, including scheduling availability

It is observed that RedCap NTN approach shall serve as the baseline for defining the RRM requirements for HD-FDD VSAT with FR1 numerology. The following sections provides as analysis of individual requirements based on the preceding table.

2.2.1 Maximum interruption in paging reception

Among cell re-selection requirements, only the maximum interruption in paging reception has a material impact on HD-FDD UEs:

-	The 1Rx RedCap in HD-FDD shall meet all applicable requirements specified in clause 4.2C.2.5 under following conditions:

-	At least 1 SSB is available at the UE in the serving cell during the last 160ms duration.

In practice, for normal UEs, the RF chain cannot achieve complete isolation. This may introduce internal interference or others, leading to interruption in paging reception from the serving cell during cell reselection measurements. The same rationale applies to HD-FDD UEs.

However, the current specification introduces additional applicability rules specifically for HD-FDD UEs, which distinguishes them from FD-FDD UEs. A key characteristic of HD-FDD is its inability to transmit and receive transmission simultaneously. Thus, if HD-FDD UE returns to serving cell and ready to receive paging message, while UE is performing an uplink transmission and happens to miss an SSB from the serving cell. It may fail to acquire synchronization, resulting in paging reception failure. At least 1 SSB is available at the UE in the serving cell during the last 160ms duration is the worst case.

In summary, maximum paging reception interruption is influenced primarily by internal RF chain and is not strongly dependent on UE type. For HD-FDD VSAT UE, it is essential to ensure that at least one SSB is available during the last 160ms.

Proposal 2: Regarding HD-FDD VSAT UE, the legacy maximum interruption in paging reception could be reused with the following legacy applicability rule:

-	At least one SSB is available at the UE in serving cell during the last 160ms duration.

2.2.2 Handover and conditional handover requirements

For RACH-based handover, the following rules shall be added:

- For RACH-based HO, following applicability rules to adapt HD-FDD case should be added:

  - -SSB is available at the UE once every SMTC period during Tsearch

  - -One SSB is available during T∆

  - -One SSB is available during TIU

- For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

  - -One SSB is available during T∆

  - -One SSB is available during TIU

  - Actually, these applicability rules and requirements have been defined in Ka band NTN. In Rel-19 discussion, the frequency range for NTN has been extended and contained Ku band and Ka band, the following conclusion has been reached:

Agreement: - Clarify in clause 3.6.12 that “Unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT working in FR1-NTN”.

  - Following this logic, it is deemed that the aforementioned applicability rules shall be reused. These rules ensure that HD-FDD VSAT UE will always have at least one SSB for measurement. Some companies have raised concerns that this approach may cause confusion with RAN1 agreements (Case 5). However, in legacy RedCap TN discussion, collision cases were already defined and SSB is always prioritized over UL transmission, but the related applicability rules have still been captured. Following the same rational, it is considered feasible to reuse the requirements by incorporating the above applicability rules. RAN1 agreement specifies that if SSB collides with UL transmission, then SSB is prioritized. This principle is not restricted to any specific scenario; whenever such a collision occurs, SSB always has the highest priority. In contrast, these applicability rules says the SSB shall be guaranteed to be measured within each defined time period. These are two distinct aspects. Consequently, it is concluded that the legacy FR2- NTN requirements could be reused, provided that the specific applicability rules are clearly captured.

Observation 1: In legacy RedCap TN discussion, collision cases were already defined and SSB is always prioritized over UL transmission, but the related applicability rules have still been captured.

Proposal 3: Legacy FR2-NTN handover and conditional handover could be reused, provided that the following applicability rules shall be captured:

For RACH-based HO, following applicability rules to adapt HD-FDD case should be added:

  - -SSB is available at the UE once every SMTC period during Tsearch

  - -One SSB is available during T∆

  - -One SSB is available during TIU

For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

  - -One SSB is available during T∆

  - -One SSB is available during TIU

2.2.3 RRC Connection Release with redirection

During legacy RedCap NTN discussion, the following conditions were agreed upon:

The HD-FDD UE shall meet the RRC connection release with redirection requirements provided that SSB is available at the UE once every SMTC period during Tsearch.

Consequently, the above applicability rule can be directly reused for HD-FDD VSAT UE.

However, it is important to note that the legacy requirements primarily target FR1 UEs (e.g. handheld UEs) equipped with omnidirectional antennas, which do not require additional time for steering the downlink spatial reception filter to the target cell. In contrast, the HD-FDD VSAT UE employ the directional antennas, and thus the additional beam steering time shall be taken into account.

The reason to update the requirements is that HD-FDD VSAT UEs using FR1 numerology should align with FR2-NTN requirements-rather than FR1-NTN-based on the following agreement:

Agreement: Clarify in clause 3.6.12 that “Unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT working in FR1-NTN”.

Consequently, Tsat_beam defined in FR2-NTN handover shall be added and reused. The updated RRC connection release with redirection requirements could be :

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tsat_beam+Tidentify-NR + TSI-NR + TRACH

where,

-	For UE indicating ‘electronic’ via UE capability ntn-VSAT-AntennaType-r18, Tsat_beam is 3*Trs

Given that only the electronic steering mechanism is agreed for study in HD-FDD VSAT UEs, the Tsat_beam parameter is defined without considering mechanical steering. Therefore, the HD-FDD VSAT UEs shall meet the updated requirements while continuing to apply the legacy applicability rule.

Observation 2: The legacy redirection requirements primarily target FR1 UEs (e.g. handheld UEs) equipped with omnidirectional antennas.

Observation 3: Unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT working in FR1-NTN.

Proposal 4: Legacy redirection requirements shall be updated for HD-FDD VSAT UEs with adding Tsat_beam, as follows:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tsat_beam+Tidentify-NR + TSI-NR + TRACH

where,

-	For UE indicating ‘electronic’ via UE capability ntn-VSAT-AntennaType-r18, Tsat_beam is 3*Trs

Proposal 5: The legacy applicability rules shall be reused, as follows:

The HD-FDD UE shall meet the RRC connection release with redirection requirements provided that SSB is available at the UE once every SMTC period during Tsearch.

2.2.4 Random Access

For HD-FDD VSAT UE, it is proposed that both the legacy random access applicability rules and the existing random access requirements for NTN scenario shall be maintained, with the following preconditions:

HD-FDD UE with the following conditions: - The RedCap UE operating in HD-FDD mode is not expected to perform PRACH transmission on a PRACH resource of a cell if UE has not received at least one SSB associated with that PRACH resource during the last Tp period in the cell, where Tp=160 ms. - The RedCap UE operating in HD-FDD mode shall meet the PRACH requirements when performing PRACH transmission on a PRACH resource of a cell provided that the UE has received at least one SSB associated with that PRACH resource during the last Tp period before the PRACH transmission, where Tp=160 ms.

Proposal 6: The legacy random access requirements and related conditions could be maintained for HD-FDD VSAT UEs.

2.2.5 UL timing accuracy

Regarding UL timing accuracy, the existing specification has already stated that the UE shall meet the Te_NTN requirement for an initial transmission provided at least one SSB is available at the UE during the last 160ms. This means the applicability rule is already been defined, and thus there is no additional impact on UE UL timing accuracy.

Furthermore, legacy TN timing requirements do not distinguish between HD-FDD and FD-FDD UE types. Following the same logic, it is concluded that HD-FDD also has no impact on UE UL timing accuracy.

Proposal 7: HD-FDD has no impact on UE UL timing accuracy.

2.2.6 L1 RSRP measurement

For L1 RSRP measurement scheduling availability, the agreements defined in RedCap NTN could be seen as below:

When NW indicates UL overriding DL for collision handling, - RAN4 to clarify that the measurement requirements for CSI-RS and PRS apply provided that no collision between CSI-RS & PRS and UL transmission - RAN4 to clarify that measurement requirements for CSI-RS, scheduling restriction of PDSCH, and PRACH apply provided that no collision between CSI-RS/PDSCH and PRACH - Otherwise, - CSI-RS based L1 measurement and PRS based measurement is prioritized over the UL transmission. - The scheduling restriction requirements for PUCCH/PUSCH/SRS will apply.

The above agreement is aligned with RAN1 agreements. Furthermore, the reuse of the existing collision handling approach as the baseline was confirmed. Therefore, the above agreements can also serve as the baseline for L1-RSRP measurement scheduling availability.

Proposal 8: The legacy scheduling availability for L1 RSRP measurement shall be regarded as baseline.

2.2.7 Satellite switching

During R18 discussion, RAN4 defined hard satellite switch and soft satellite switch requirements for handheld UE with FR1 numerology. In R19, corresponding requirements were established for VSAT UEs operating in the Ku band. Compared to R18 discussion, the sole modification is that Tsat_beam was added into the final delay requirements. It is understood that the legacy R19 requirements could be applied under the following additional conditions:

  - -SSB is available at the UE once every SMTC period during Tsearch

  - -One SSB is available during T∆

  - -One SSB is available during TIU

For HD-FDD UEs that complies with the above applicability rules, the legacy FR2-NTN satellite switching requirements shall be reused, encompassing both hard and soft satellite switch.

Proposal 9: The legacy FR2-NTN hard and soft satellite switching requirements could be reused with the following conditions:

  - -SSB is available at the UE once every SMTC period during Tsearch

  - -One SSB is available during T∆

  - -One SSB is available during TIU

Conclusion

In this paper we provided our views on RRM requirements on R20 NR Ku band NTN enhancement.

Proposal 1: For HD-FDD for VSAT types 2,3,5,6, RAN4 shall define a single set of RRM requirements, regardless of whether the VSAT UE is fixed or mobile, or served by GEO or LEO satellites.

Proposal 2: Regarding HD-FDD VSAT UE, the legacy maximum interruption in paging reception could be reused with the following legacy applicability rule:

-	At least one SSB is available at the UE in serving cell during the last 160ms duration.

Observation 1: In legacy RedCap TN discussion, collision cases were already defined and SSB is always prioritized over UL transmission, but the related applicability rules have still been captured.

Proposal 3: Legacy FR2-NTN handover and conditional handover could be reused, provided that the following applicability rules shall be captured:

For RACH-based HO, following applicability rules to adapt HD-FDD case should be added:

  - -SSB is available at the UE once every SMTC period during Tsearch

  - -One SSB is available during T∆

  - -One SSB is available during TIU

For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

  - -One SSB is available during T∆

  - -One SSB is available during TIU

Observation 2: The legacy redirection requirements primarily target FR1 UEs (e.g. handheld UEs) equipped with omnidirectional antennas.

Observation 3: Unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT working in FR1-NTN.

Proposal 4: Legacy redirection requirements shall be updated for HD-FDD VSAT UEs with adding Tsat_beam, as follows:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tsat_beam+Tidentify-NR + TSI-NR + TRACH

where,

-	For UE indicating ‘electronic’ via UE capability ntn-VSAT-AntennaType-r18, Tsat_beam is 3*Trs

Proposal 5: The legacy applicability rules shall be reused, as follows:

The HD-FDD UE shall meet the RRC connection release with redirection requirements provided that SSB is available at the UE once every SMTC period during Tsearch.

Proposal 6: The legacy random access requirements and related conditions could be maintained for HD-FDD VSAT UEs.

Proposal 7: HD-FDD has no impact on UE UL timing accuracy.

Proposal 8: The legacy scheduling availability for L1 RSRP measurement shall be regarded as baseline.

Proposal 9: The legacy FR2-NTN hard and soft satellite switching requirements could be reused with the following conditions:

  - -SSB is available at the UE once every SMTC period during Tsearch

  - -One SSB is available during T∆

  - -One SSB is available during TIU

- References

- RP-252958, WID: New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2.

- R4-2515082, WF for [116bis][330] NR_IoT_NTN_Ph2_Part2, Media Tek.