3GPP TSG-RAN4 Meeting #117	R4-2521739

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

Source:	Nokia

Title:	Scope of test cases for the DL Coverage Enhancement feature in NTN

Agenda item:	6.19.3.2

Document for:	Approval

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_PH3). The WID contain a handful of objectives and among them, there is one topic on Downlink Coverage Enhancements. One of the main features discussed to be implemented as part of this feature is an extended SSB periodicity for the main (SA) cell in NTN, to match the periodic “active” and “inactive” periods of the cell. The switching between “active” and “inactive” is created in NTN to handle the limited power availability in the satellite.

In previous RAN4 meeting, the initial discussion on the test cases of such feature was initiated. In the present contribution we discuss the open issues in the WF from previous meeting [1] as well as add our views and proposals for the DL Coverage Enhancement feature test cases.

Discussion

In the previous meeting, RAN4 has discussed the performance requirements and potential test cases for the DL coverage enhancement feature and has reached the following way forward:

Issue 1-2-1-2: Test scope of SMTC enhancement for the UE capable of coverage enhancement Online agreement < Agreement >: - For supporting two SMTC periodicities with different offsets in one frequency layer, - RAN4 to only introduce new test case for L3 event triggered reporting. - For more than 4 SMTCs per frequency (i.e. 6) in RRC Idle/Inactive mode, - Wait for conclusion on core part discussion before discussing whether to define new test cases on cell reselection. Issue 1-2-2: Test configuration, parameters and procedures for downlink coverage enhancement Online agreement < Agreement >: - RAN4 to introduce new 160ms periodicity SSB pattern for DL enhancement NTN UE. - For supporting two SMTC periodicities with different offsets in one frequency layer, RAN4 to introduce two configurations of SMTC periodicities of 20 ms and 160 ms with different offsets . - For other unchanged test configurations, parameters and procedures, RAN4 to use the reference method to define the corresponding test cases. Issue 1-2-3: Where to capture the test cases for downlink coverage enhancement Online agreement < Agreement >: - For downlink coverage enhancement, if some new test cases are introduced, which will be added in the existing NTN test cases and they can be defined in Clause A.14.

In issue 1-2-3, it is also discussed implicitly whether to introduce new test for SSB periodicity extension, In general, we see that supporting the new extended SSB periodicity for initial access is not necessarily a fundamental change that would require all features to be retested. This do not affect any functionality at the UE. However, it is interesting to observe whether the UE is capable of adapting its measurement periods and filters to the case where SMTC periodicity is much longer. So, from one point of view, there is no significant functional difference, but on the other hand, it is interesting to check the UE capability to adapt the measurements. In order to compromise between such conflicting views, we propose to select one single test case to be tested with higher SSB periodicity.

In case the UE supports the DL Coverage Enhancement features, at least one test in RRC IDLE and one test in RRC CONNECTED mode shall be performed  also using the largest SSB periodicity configuration.

RAN 4 has agreed to introduce a new test case to test two SMTC periodicities in issue 1-2-1-2. As this is in practice a new functionality it has never been tested before. In this case, there are two possible configurations, one in which the serving cell has the smallest periodicity of SSBs (20 ms) while the neighbor cell has the largest (160 ms) and one in which the configuration is reversed. Form our point of view, it is more interesting to check if the UE handles the adaptation towards the longest periodicity, so therefore, we believe that the target cell has to be configured with largest periodicity.

In the new L3 event triggered reporting TC with two SMTC periodicities, the largest periodicity (160 ms) is to be configured in the neighbor cell while the serving cell maintain a SMTC periodicity of 20 ms.

For proposal 2, a baseline scenario is provided in Figure 1, where 3 SMTC configurations are provided, using two different SMTC periodicities.

Figure 1 – Example of test configuration with 3 SMTCs with different periodicities and offset

Conclusion

In this paper we discussed the impact of the new measurement framework proposed by RAN2 and RAN1 to handle the extended SSB periodicity in NTN under the topic of DL coverage enhancements. Based on our discussion and analysis, we made the following list of proposals:

Proposal 1: In case the UE supports the DL Coverage Enhancement features, at least one test in RRC IDLE and one test in RRC CONNECTED mode shall be performed  also using the largest SSB periodicity configuration.

Proposal 2: In the new L3 event triggered reporting TC with two SMTC periodicities, the largest periodicity (160 ms) is to be configured in the neighbor cell while the serving cell maintain a SMTC periodicity of 20 ms.

- References

- R4-2515056, Way Forward for [116bis][327] Rel-19 RRM performance_Part3, CATT, RAN WG4 Meeting #116bis, Prague, Czech, 13th – 17th October, 2025