3GPP TSG-RAN WG4 Meeting#117                                R4-2521748

Dallas, USA, 17th – 21st, Nov. 2025

Agenda item:	8.4.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on 6GR NTN BS study

Document for:	Approval

Introduction

In the RAN#108 meeting, a new WID on Study on 6G Radio [1] was approved. In the last RAN4 meeting, there were some initial discussions on 6G NR BS RF requirement and agreements were captured in WF [8].In this contribution, we would like to share some further understandings on 6GR NTN BS study.

- 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - RAN4#1116bis agreement: NTN Way Forward: Additional input from satellite companies is expected for RAN4#117: - List of requirements to be further studied (using the same format proposed in 1.1.2) - List of coexistence studies to be considered (using the format proposed in 1.3.1).

Discussion

- 2.1. Review of 5G BS/SAN evolution

In the following Figure 2.1-1, we provide the overview of 5G NR BS RF evolution and study on 7-24GHz for ITU-R WP5D reply LS which has already provided a very good foundation for 6G BS RF requirement.

Figure 2.1-1. The review of 5G NR BS evolution from Rel-15 to Rel-20

Proposal 1: for 5G NR NTN refarming bands, propose to use the RF requirement of 5G NR SAN as starting point and make further improvement if confirmed to be necessary.

Proposal 2: for 6GR NTN SAN, propose to consider the following 5G BS types at least as starting point.

6GR NTN

FR1 SAN

Ka-band SAN

Ku-band SAN

- 2.2. New BS types in addition to 5G NR

- 2.2.1. FR1 Hybrid beamforming

For NTN deployment, it is important to increase the coverage from both DL and UL, one of efficient solution is to improve the antenna array size from the limited few square meters. Considering the balance of the number of RF chains and large antenna array size, the hybrid beamforming architecture offers one solution to address this problem similar as TN BS around 7GHz supporting more than 1k or 1.5k antenna elements;

Proposal 3: propose to support the hybrid beamforming for SAN and leverage the definition method from TN BS with hybrid beamfomring.

In addition, the RF architecture for SAN is somehow a bit different from the TN BS from RDN mapping perspective. For NTN SAN, single RF chain might be able/designed to support the whole antenna array to provide the different beamprint via analog approach instead of connecting with one sub-array with digital beamforming for different users.

- 2.2.2. MSR SAN

For MRSS SAN supporting both 5G and 6G in the NR refarming bands, it is necessary to define the MSR SAN similar as TN SAN. In addition, the in-band NB-IoT within NR carrier was also introduced in Rel-18 by TEI manner. From this perspective, it should be reasonable to support the 4G(in-band IoT NTN)-5G-6G NTN MRSS in the new NTN MSR specification.

Proposal 4: for MRSS SAN, propose to consider NTN SAN with 4G(in-band IoT NTN)-5G-6G NTN MRSS in the new NTN MSR specification.

- 2.3. Improvement for the RF requirement of 6GR

- 2.3.1. EVM requirement

As shown in the following agreement, during the last RAN1#122 meeting, RAN1 has already agreed to use the 5G modulation order as basis and further discuss other enhancement and modulation orders. In general, we could apply the EVM requirement of 5G SAN as starting point..

Agreement: RAN1_122 meeting - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Proposal 5: for minimum EVM requirement of 6GR, propose to apply the 5G SAN EVM requirement as starting point for 6GR BS.

Since the EVM requirement is extremely critical for the network performance/achievable peak data rate in field. In the past, TN BS EVM requirement is mainly evaluated under the assumption of SU-MIMO with the limited MIMO layers (up to 2/4 MIMO layers ), however this kind of evaluation assumption is not quite aligned for the scenarios with high traffic load in the commercial network. For the high traffic load case, usually MU-MIMO and CA must be considered to guarantee the network capacity. For SAN deployment, it might be not necessary to consider the SU-MIMO or MU-MIMO within single beamprint, however interference between different beamprint should be also considered. These kind of co-channel interference from SAN side with multiple serving beams should be well considered to derive the appropriate EVM requirement for it. In short, in 6G phase, we propose to discuss the optimal EVM requirement in addition to the minimum EVM requirement at least to reflect the more realistic network operation including multiple beamprint case.

Figure 2.3.1-1. the illustration of SAN with multiple beam operation for different beamprint

Proposal 6: for the optimal EVM requirement, propose to have some discussion on the necessity and evaluation method to figure out the optimal/enhanced EVM requirement to enable the achievable peak data rate if possible.

- 2.3.2. ACLR requirement

ACLR requirement is a critical RF requirement to guarantee the coexistence performance between adjacent carries of the same RAT or different 3GPP RAT. Meanwhile ACLR requirement is also one of the important parameters to dimension the RF design especially from potentially different PA design and different power consumption for the overall BS, the appropriate interference antenna modelling in the adjacent channel is extremely important to quantify the interference and figure out the required ACLR requirement.

Indeed, the spatial antenna modelling in the adjacent channel was discussed in the past in TN network and could be dated back to the AAS SI phase in Rel-12 phase. The composite radiation gain pattern for pure LOS channel can be expressed in logarithmical scale as captured in TR 37.840:

AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2−1

Where ρ is the correlation level for the unwanted emission in the adjacent channel.

However after the Rel-12 LTE AAS SI, there were some contiguous discussions in both 3GPP and ITU-R WP5D on how to justify the more appropriate correlation level for the unwanted emissions for sharing study in ITU-R. During the Rel-19 SI for WP5D LS on IMT parameters of 4.4-4.8GHz, 7.125-8.4GHz and 14.8-15.35GHz, there were some further discussions on antenna modelling of unwanted emission in the adjacent channel, the array correlation factor roll-off model according to frequency offset away from the wanted signal carrier bandwidth edge is captured as following (extracted from TR 38.922).

Figure 2.3.5-1: Array correlation factor roll-off model. [TR 38.922]

Using the approach above the antenna model can be extended to capture array antenna gain outside the wanted carrier as:

AAθ,φ,f=Asubθ,φ+10log101+ρfm=1Mn=1Nwm,nvm,n2−1

However for FR1 SAN and FR2-1 SAN’s ACLR requirements as summarized in the following table, these are relatively very relaxed requirements, in other words, we might not see strong necessity to further improve this requirement.

Table 2.3.2-1. The summarized ACLR requirement for SAN in 5G phase.

| Scenarios | ACLR [dB] | NOTE |
| --- | --- | --- |
| FR1 GEO | 14 |  |
| FR1 LEO | 24 |  |
| Ka-band GEO/LEO | 12 |  |
| Ku-band GEO/LEO | 16 |  |

For sure, the potential relationship between ACLR requirement and EVM requirement should be also considered. In the past, these two requirement was discussed separately in RAN4, however in the practical implementation, there are some implicit relationship e.g. more relaxed ACLR requirement will also result in the worse EVM requirement within the wanted carrier which will cause the negative impacts on the actual network performance.

- Proposal 7: for the spatial ACLR modelling, propose to deprioritize this requirement optimization due to its relaxed value.

Conclusions

In this contribution, we shared some initial views on the 6GR NTN SAN RF study and proposals are made as following:

Proposal 1:for 5G NR NTN refarming bands, propose to use the RF requirement of 5G NR BS as starting point and make further improvement if confirmed to be necessary.

Proposal 2: for 6GR NTN SAN, propose to consider the following 5G BS types at least as starting point.

6GR NTN

FR1 SAN

Ka-band SAN

Ku-band SAN

Proposal 3: propose to support the hybrid beamforming for SAN and leverage the definition method from TN BS with hybrid beamfomring.

Proposal 4: for MRSS SAN, propose to consider NTN SAN with 4G(in-band IoT NTN)-5G-6G NTN MRSS in the new NTN MSR specification.

Proposal 5: for minimum EVM requirement of 6GR, propose to apply the 5G SAN EVM requirement as starting point for 6GR BS.

Proposal 6: for the optimal EVM requirement, propose to have some discussion on the necessity and evaluation method to figure out the optimal/enhanced EVM requirement to enable the achievable peak data rate if possible.

- Proposal 7: for the spatial ACLR modelling, propose to deprioritize this requirement optimization due to its relaxed value.

- Proposal 8: the summarized proposal for 6GR SAN requirement can be found in Annex.

References

- RP-251881,New SID: Study on 6G Radio, Approved.

- R4-2511000,Further consideration on BS RF for Rel-19 LP-WUS, Noted

- RP-252952,New WID: UE RF enhancements for NR FR1, Phase 5, Approved.

- RP-25290, New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2, Approved

- RP-252578,Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, Approved.

- TR 38.922

- R4-2316538c,Further discussion on RF requirement impacts from NES perspective, ZTE corporation

- R4-2514638, WF on 6G BS RF and co-existence, Ericsson, approved.

- TS 38.108

Annex

| 5G SAN requirements | No impact expected, might be reused as is for 6G | To be re-evaluated based on system parameters decision for 6G (SU, channel BW, …), following TR 38.863 principles/formula | Would need to be studied in the 6G SI scope |
| --- | --- | --- | --- |
| Conducted requirements |  |  |  |
| Satellite Access Node output power | X |  |  |
| Output power dynamics |  |  |  |
| RE power control dynamic range | X |  |  |
| Total power dynamic range |  | X | Due to the larger CBW and increasing PRB numbers, then this requirement should be updated accordingly. |
| Transmit ON/OFF | NA |  |  |
| Transmitted signal quality |  |  |  |
| Frequency error | X |  |  |
| Modulation quality |  | X | Please find the detailed analysis in 2.3.1. |
| Time alignment error | NA |  | This depends the CA discussion in 5G-A phase in Rel-20. |
| Unwanted emissions |  |  |  |
| Occupied bandwidth | X |  |  |
| ACLR |  | X | This is highly dependent on the 6GR coexistence study. |
| Operating Band Unwanted Emissions | X |  | This is from ITU recommendation. |
| Transmitter spurious emission |  |  |  |
| General requirement | X |  | This is from regulatory requirement and don’t see the necessity of change. |
| Protection of BS receiver of own or different BS | NA |  |  |
| Additional spurious (coex) | NA |  |  |
| Co-location | NA |  |  |
| Transmitter intermodulation | NA |  |  |
| Receiver sensitivity level |  | X | NF from 5G could be reuse for 6G. The current equation for REFSENS definition could be reused. FFS to discuss the FRC for larger channel bandwidth e.g. 200MHz or 400MHz in FR1 to simplify the conformance testing. |
| Dynamic range |  | X | The current equation for wanted signal and interfering signal could be reused. |
| In-band selectivity and blocking |  |  |  |
| ACS | X |  |  |
| In-band blocking | NA |  |  |
| Out of band blocking | X |  |  |
| Receiver spurious emissions | NA |  |  |
| Receiver intermodulation | NA |  |  |
| In channel selectivity |  | X | The current equation for wanted signal and interfering signal could be reused. |
| Radiated requirements |  |  |  |
| Radiated transmit power | X |  | EIRP related accuracy from 5G could be reused for 6G. |
| OTA Satellite Access Node output power | X |  | TRP related accuracy from 5G could be reused for 6G. |
| OTA output power dynamics |  |  |  |
| OTA RE power control dynamic range | X |  | The same as conducted part |
| OTA Total power dynamic range |  | X | The same as conducted part |
| OTA transmit ON/OFF | NA |  |  |
| OTA transmitted signal quality |  |  |  |
| OTA Frequency error | X |  |  |
| OTA Modulation quality |  | X | The same as conducted part |
| OTA Time alignment error |  | X | The same as conducted part |
| OTA Unwanted emissions |  |  |  |
| OTA Occupied bandwidth | X |  |  |
| OTA ACLR |  | X | The same as conducted part |
| OTA Operating Band Unwanted Emissions |  | X | The same as conducted part |
| OTA Transmitter spurious emission |  |  |  |
| OTA General requirement | X |  |  |
| OTA Protection of BS receiver of own or different BS | NA |  |  |
| OTA Additional spurious (coex) | NA |  |  |
| OTA Co-location | NA |  |  |
| OTA transmitter intermodulation | NA |  |  |
| OTA sensitivity | X |  |  |
| OTA receiver sensitivity level |  | X | Similar as conducted part |
| OTA dynamic range |  | X | Similar as conducted part |
| OTA in-band selectivity and blocking |  |  |  |
| OTA ACS | X |  |  |
| OTA In-band blocking | NA |  |  |
| OTA out of band blocking | X |  |  |
| OTA receiver spurious emissions | NA |  |  |
| OTA receiver intermodulation | NA |  |  |
| OTA in channel selectivity |  | X | The current equation for wanted signal and interfering signal could be reused. |