# 3GPP TSG RAN WG4 Meeting #117 														R4-2521925

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	7.8.4

Source: 	Ericsson

Title:	RRM requirements for Rel-20 low band carrier aggregation via switching

Document for:	Discussion

# Introduction

New WID [1] on Low band carrier aggregation via switching is agreed in RAN#106. In this contribution, we provide our views on the potential RRM requirements impact.

# Discussion

In Rel-19, switching occurs between the following cases (e.g., FDD = n5/n12; SDL = n29):

- Case 1: Tx/Rx on FDD carrier, no Rx on SDL carrier

- Case 2: Rx on SDL carrier, no Tx/Rx on FDD carrier

To enhance low NR band carrier aggregation, support for dual RX via switching was agreed in RAN#109. The corresponding WID is referenced below.

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

According to the WID, the key enhancement is dual RX reception, covering the following cases:

- Case 1: Tx/Rx on FDD carrier 1, no Rx on SDL carrier 2

- Case 3: Rx on both FDD carrier 1 and SDL carrier 2, no Tx on FDD carrier 1

RAN4 RRM work scope

We have started discussing the scope of RRM work for this WI, and the common understanding in RAN4 is that the RRM requirements defined for Rel-19 LB-CA switching will serve as the baseline for the Rel-20 LB CA RRM requirements. In Rel-19 following requirements are defined.

| SDL SCell related RRM requirement | Index | Requirements |
| --- | --- | --- |
|  | 1 | Deactivated SDL SCell measurement |
|  | 2 | Interruption due to deactivated SCell measurement |
|  | 3 | Activated SDL SCell L3 measurement |
|  | 4 | BFD/CBD for SDL SCell |
|  | 5 | L1-RSRP/SINR for SDL SCell |
|  | 6 | SDL SCell activation/deactivation |
|  | 7 | TCI state switch delay |
| FDD PCell related RRM requirement | 8 | PCell L3 measurement |
|  | 9 | BFD/CBD for PCell |
|  | 10 | L1-RSRP/SINR for PCell |
|  | 11 | RLM for PCell |
|  | 12 | TCI state switch delay |
|  | 13 | PL-RS switch delay for PCell |

Based on company contributions, the following differences between Rel-19 and Rel-20 have been identified for further discussion in RAN4:

- Potential requirement impact considering DL reception in FDD PCell in Rel-20 (HW, OPPO)

- Need for Tx timing requirement for PCell (ZTE)

- Need for interruption or scheduling restriction requirements for CC switching (QC)

- Need for switching gap (Nokia)

We believe the key differences mentioned above arises from the fact that, in Rel-20, the UE can either receive on FDD + SDL or perform RX/TX on FDD. According to both company views and our own, this behavior depends on the RF architecture agreed upon in RAN4 RF session. In the last meeting, the following reference architecture was agreed in the RF session. Based on that, we have further analyzed the potential impact on the requirements listed above.

Figure 1: UE reference architecture for the RF

In Rel-20, when the UE switches between Case 1 and Case 3

- FDD Tx/Rx (Case 1), or

- FDD Rx + SDL Rx with no FDD Tx (Case 3)

Based on the reference architecture agreed in RF session, the switching between these cases involves the following RF path changes.

- Case 1: FDD Duplexer + FDD DL Filter Architecture

  - Main antenna is connected to an FDD band duplexer, which enables simultaneous downlink and uplink transmission by suppressing uplink-to-downlink interference. The duplexer output is routed to the main LNA. Meanwhile, the diversity antenna is connected to an FDD band downlink filter, which connects to the diversity LNA

- Case 3: Combined SDL + FDD DL Filter Architecture

  - Main antenna connects to a combined SDL + FDD DL filter, diversity antenna connects to a separate combined SDL + FDD DL filter and the each filter output connects to its respective main or diversity LNA.

As we can observe from the RF paths involved in Case 1 and Case 3, in both configurations the main antenna ultimately connects to the main LNA, although different filtering components are used in each case. Similarly, the diversity antenna connects to the diversity LNA in both cases, with filters varying depending on the configuration. According to the RF agreement, SDL and FDD downlink signals are separated after the LNA, and there are no common components between the SDL and FDD bands. Specifically, the local oscillator (LO) and other parts of the Rx chain are separate for FDD and SDL, and operate independently.

From this perspective, we do not see a need for any timing dependency between the FDD PCell and the SDL SCell. Therefore, we do not believe the restriction of MRTD = 3 µs, which was considered in Rel-19, is applicable or necessary in this context. In this sense, we consider the architecture to be similar to an inter-band non-collocated CA architecture, and even an MRTD of 33 µs can be supported at the UE side. One disadvantage of supporting MRTD of 33 µs is that the UE may not be able to switch between Case 1 and Case 2 for an additional 30 µs. From this perspective, we are fine with keeping the 3 µs MRTD due to the collocation assumption, even though the UE supports 33 µs MRTD.

- Though UE can support larger MRTD, to reduce the interruption or switching gap between case 1 and 3, RAN4 to reuse the Rel-19 MRTD value of 3µs.

We would like to point out that the switching time capabilities remain the same as those defined for the Rel-19 band combinations: 35 µs, 70 µs, and 140 µs.

For the transition from FDD DL/UL to FDD DL + SDL DL (Dual DL), given that FDD DL/UL uses a duplexer while FDD DL shares a combined filter with SDL, it is necessary to introduce an interruption in the FDD DL reception, as the UE needs to switch from using the duplexer to the combined filter. Similar to Rel-19, the switching gap shall include the switching time, a transient time of 10 µs, and the MRTD of 3 µs.

Switching gap = Switching time + Transient time + MRTD

- If the UE supports 35 µs switching time, then the switching gap is 48 µs → equivalent to 1 symbol.

- If the UE supports 70 µs switching time, then the switching gap is 83 µs → equivalent to 2 symbols.

- If the UE supports 140 µs switching time, then the switching gap is 153 µs → equivalent to 3 symbols.

We would also like to point out that the switching gap (N1) from FDD to SDL already supports 1 to 3 symbols, and therefore, the same can be reused in this context.

Observation 1: Switching gap from FDD DL/UL to Dual DL is 1 to 3 symbols and the N1 values defined in Rel-19 can be reused.

As observed from the RF paths involved in Case 1 and Case 3, for the scenario of Dual DL to FDD DL/UL, the UE UL in FDD may start ahead of the FDD DL. Given that FDD DL/UL uses a duplexer, while FDD DL shares a combined filter with SDL, a switching gap is required to transition from Dual DLto FDD DL/UL. It is also necessary to include the UL timing advance (TA) in the switching gap calculation.

Since the FDD DL and SDL DL share the same combined filter, the additional interruption caused by UL TA applies to the FDD DL reception as well. In Rel-19, UL TA was already considered for the switch from SDL to FDD, resulting in switching gap (N2) values ranging from 1 to 30 symbols. We believe the same N2 values are applicable for the switch from Case 3 to Case 1 (i.e., Dual DL to FDD DL/UL).

Observation 2: Switching gap from Dual DL to FDD DL/UL is 1 to 30 symbols if we condier the same deployment scenarios as Rel-19. The N2 values defined in RAN1 can be reused.

Based on observation 1 and 2, we present the potential switching mechanism of Dual DL in Figure 2.

Figure 2: Switching mechanism in Rel-20 LB CA with dual DL

Based on observations 1 and 2, the key question is: how should the switching gap be specified? One approach is to define it as a network-configurable switching gap, which was the method adopted in Rel-19 and specified by RAN1. However, it could be argued that, since RAN1 is not included in the WID, RAN4 might instead specify this as an interruption.

Regardless of whether the switching gap is specified by RAN1 or RAN4, the resulting N1 and N2 values are the same. Therefore, we do not believe it is a good approach to treat this as an interruption. Instead, we recommend continuing with the network-configurable switching gap, as it provides a more predictable switching behaviour and consistent framework aligned with Rel-19.

- RAN4 to agree that the switching gap to be specified as an RRC-configurable switching gap, rather than defining it as an interruption, for the switch between Case 1 and Case 3.

- Switching gap from FDD DL/UL to FDD DL+SDL DL (N1): Configurable between 1 to 3 symbols

- Switching gap from FDD DL+SDL DL to FDD DL/UL (N2): Configurable between 1 to 30 symbols

SDL SCell requirements

Regarding RAN4 requirements for SDL SCell, when comparing the pattern design differences between Rel-19 and Rel-20 as shown section 2.4, the SDL behavior remains unchanged. Specifically, there are slots during which the UE cannot receive on SDL SCell as per the switching pattern design. In Rel-19, RAN4 already captured these missed reception occasions in the SDL SCell requirements, and we believe these can be reused for Rel-20.

One SDL SCell requirement that may need more discussion is the SCell activation delay. In Rel-19, due to the restriction that the FDD PCell and SDL SCell cannot be received simultaneously, the activation delay requirements were based on SMTC and SSB, rather than SMTC_max and SSB_max. However, when simultaneous reception of FDD PCell and SDL SCell is possible, AGC may be performed during overlapping SMTC and SSB occasions, as per legacy behavior.

Now, we have two options for the Rel-20 LB CA via switching:

- Define both legacy and Rel-19-based SDL SCell activation requirements, considering both approaches.

- Simplify UE implementation for Rel-19 UEs which also may support Rel-20 by adopting the Rel-19 method of SDL SCell activation and use SSB and SMTC.

RAN4 can further discuss whether to introduce/retain legacy-based SDL SCell activation requirements or simply reuse the Rel-19 SDL SCell activation requirements based on SSB and SMTC.

Another SDL SCell requirement that also need more discussion is interruption due to deactivated SCell measurements. In the current requirements, it was assumed that the UE cannot receive PDSCH on FDD during deactivated SDL measurements, potentially leading to missed ACKs. However, with Dual DL reception capability, the UE can maintain DL reception on FDD, and only missed UL transmission opportunities need to be considered. Therefore, the interruption requirement related to deactivated SDL SCell measurements may require further discussion.

| SDL SCell related RRM requirement | Index | Requirements | Can the Rel-19 LB CA requirement be reused | Comments |
| --- | --- | --- | --- | --- |
|  | 1 | Deactivated SDL SCell measurement | Yes. | Measurement requirements can be reused |
|  | 2 | Interruption due to deactivated SCell measurement | NO | Legacy (non LB CA) or new requirement needs to be defined |
|  | 3 | Activated SDL SCell L3 measurement | Yes | SCell measurements requirements can be resued |
|  | 4 | BFD/CBD for SDL SCell | Yes |  |
|  | 5 | L1-RSRP/SINR for SDL SCell | Yes |  |
|  | 6 | SDL SCell activation/deactivation | Yes and may need additional requirements | RAN4 to discuss if it is necessary to also support the Rel-15 design based on SSB_MAX and SMTC_MAX |
|  | 7 | TCI state switch delay | Yes |  |

- The RAN4 RRM requirements for the SDL SCell, following requirments from Rel-19 can be reused.

  - Deactivated SDL SCell measurement

  - Activated SDL SCell L3 measurement

  - BFD/CBD for SDL SCell

  - L1-RSRP/SINR for SDL SCell

  - TCI state switch delay

- The RAN4 RRM requirements for the SDL SCell, the following requirements may need further study

  - Interruption due to deactivated SCell measurement

  - SDL SCell activation/deactivation

FDD PCell requirements

In Rel-19, when there were missed RS occasions, we extended the delay requirements for the above-mentioned requirements. When there are no missed RS occasions, there is no impact to legacy requirements. The scenarios considered to deine the requirements are listed in the table below for reference.

Agreement from RAN4#116: - for gap-less based L3 measurement and L1 measurement of a serving carrier which can be either FDD PCC or activated SDL SCC: a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed. b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions. c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

One may initially assume that there is no impact on FDD reception due to Dual DL capability, and therefore no need to extend the requirements in any case. However, a closer examination suggests that switching gaps could still result in missed SSB or CSI-RS occasions, potentially leading to the reuse of the same requirements. As we observed in the above sections, N2 applies to switching from Case 3 to Case 1 (i.e., Dual DL to FDD). Since N2 can span up to 30 symbols (i.e., ~3 slots), RS occasions such as SSB and CSI-RS may be missed. We think that the Rel-19 requirements are defined generic enough and we can reuse them as it is. Based on the above analysis, our assessment of requirements impact is presented below.

| FDD PCell related RRM requirement | PCell L3 measurement | Can the Rel-19 LB CA requirements be reused |
| --- | --- | --- |
|  | BFD/CBD for PCell | Yes |
|  | L1-RSRP/SINR for PCell | Yes |
|  | RLM for PCell | Yes |
|  | TCI state switch delay | Yes |
|  | PL-RS switch delay for PCell | Yes |

- The RAN4 RRM requirements for the FDD PCell, following requirments from Rel-19 can be reused.

- PCell L3 measurement

- BFD/CBD for PCell

- L1-RSRP/SINR for PCell

- RLM for PCell

- TCI state switch delay

- PL-RS switch delay for PCell

Which WG should capture the Switching gap—RAN1 or RAN4?

RAN1 already specified the switching gap mechanism in Rel-19. To maintain consistency and simplicity, it would be appropriate to continue specifying this in RAN1. This approach avoids redefining the same behavior in RAN4 for the Rel-20 LB CA.

What needs to be captured in RAN1?

Based on the Figures 1 and 2, we think the switching behaviour or interpretation may need to be changed in the RAN1 spec for the UE supporting Rel-20 LB CA via switching. To look at the potential changes, we provide the RAN1 spec below for reference.

When the UE is provided LBCA-SwitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration

To assess the impact of potential RAN1 changes in the switching mechanism, we illustrate how these changes might be reflected in bold text. Please note that these are only for illustrative purposes and the actual changes are upto RAN1.

- The interpretation of the text highlighted in this color may need to be revised as: a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell DL carrier and  paired PCell UL carrier to the PCell DL carrier and SCell DL carrier over the first duration, and

- The interpretation of the text highlighted in this color may need to be revised as: a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the PCell DL carrier and SCell DL carrier to the PCell DL carrier and  paired PCell UL carrier over the second duration

On top of the changes described above, we also think there is additional impacts to RAN1 and we describe them below. The WID proposes reusing the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching. However, it remains unclear whether the existing pattern can be reused as it is.

In Rel-19, the semi-static switch pattern is 40 bits long, with each slot assigned to either FDD or SDL operation. In Rel-20, when the UE switches between Case 1 and Case 3, the pattern must indicate whether a slot is for:

- FDD Tx/Rx (Case 1), or

- FDD Rx + SDL Rx with no FDD Tx (Case 3)

This difference is illustrated in the tables below.

| Slot No# | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rel-19 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX |  |  |  |  |  |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |
| Rel-20 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD RX | FDD RX | FDD RX | FDD RX | FDD RX |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |

| Slot No# | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rel-19 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX |  |  |  |  |  |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |
| Rel-20 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD RX | FDD RX | FDD RX | FDD RX | FDD RX |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |

| Slot No# | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rel-19 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX |  |  |  |  |  |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |
| Rel-20 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD RX | FDD RX | FDD RX | FDD RX | FDD RX |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |

| Slot No# | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rel-19 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX |  |  |  |  |  |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |
| Rel-20 | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD TX/RX | FDD RX | FDD RX | FDD RX | FDD RX | FDD RX |
|  |  |  |  |  |  | SDL | SDL | SDL | SDL | SDL |

To check if the pattern can be used as is, we copy the RAN1 spec below for the reference.

A UE that can switch operation between a PCell, that includes a DL carrier and a paired UL carrier, and an SCell, that includes a DL carrier without a paired UL carrier, can be provided by LBCA-SwitchingPattern a bitmap of slots indicating a switching pattern between the PCell and the SCell when the SCell is activated. The switching pattern repeats continuously. The UE can either transmit/receive on the PCell in a slot or receive on the SCell in the slot. A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the SCell in the second slot. The first slot of the bitmap is same as the first slot of a system frame with SFN 0 on the PCell. The SCS for all configured DL BWPs or UL BWPs on both the PCell and the SCell is 15 kHz and the periodicity of the switching pattern is 40 slots.

As shown in the tables above, the interpretation of the pattern from Rel-19 to Rel-20 varies at the UE. Therefore, we believe RAN1/2 should clearly define how the pattern is to be interpreted at the UE and specify where the switch should occur. To assess the impact of potential RAN1 changes, we illustrate how these changes might be reflected.

- The interpretation of the text highlighted in this color may need to be revised as: The UE can either transmit/receive on the PCell in a slot or receive on the PCell and SCell in the slot,

- The interpretation of the text highlighted in this color may need to be revised as:A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the PCell and SCell in the second slot.

Collision handling rules:

RAN1 also defined different collison cases of different signal/channel colliding with the switching gap and SDL On duration or FDD ON duration, and how to handle them. With Dual DL capability, these rules may have to be revisited.

Given the limited impact to RAN1, we could identify the potential impact to RAN1 and send a LS to RAN1, allowing them to introduce the necessary changes accordingly. Alternatively, the scope of RAN1/2 could be added to the WID in the upcoming RAN meeting. As the changes from RAN1 are small, we think sending a LS may be a better option as per opur understanding.

- We prefer RAN1 to capture the switching gap. From RAN4 perspective, the following would need to be accounted for by RAN1 for completing the LBCA via switching feature with switching between Case 1 and Case 3:

- A ‘1’ in the switching pattern bitmap configured to the UE indicates that the UE is receiving on both FDD DL and SDL carriers instead of only on the SDL carriers.

- Collision handling rules

- N1 represents the switching gap from FDD to Dual DL reception

- N2 represents the switching gap from Dual DL reception to FDD

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

- Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1

- RAN4 to discuss how to involve RAN1 in the work to complete the feature

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- Though UE can support larger MRTD, to reduce the interruption or switching gap between case 1 and 3, RAN4 to reuse the Rel-19 MRTD value of 3µs.

- RAN4 to agree that the switching gap to be specified as an RRC-configurable switching gap, rather than defining it as an interruption, for the switch between Case 1 and Case 3.

- Switching gap from FDD DL/UL to FDD DL+SDL DL (N1): Configurable between 1 to 3 symbols

- Switching gap from FDD DL+SDL DL to FDD DL/UL (N2): Configurable between 1 to 30 symbols

- The RAN4 RRM requirements for the SDL SCell, following requirments from Rel-19 can be reused.

  - Deactivated SDL SCell measurement

  - Activated SDL SCell L3 measurement

  - BFD/CBD for SDL SCell

  - L1-RSRP/SINR for SDL SCell

  - TCI state switch delay

- The RAN4 RRM requirements for the SDL SCell, the following requirements may need further study

- Interruption due to deactivated SCell measurement

- SDL SCell activation/deactivation

- The RAN4 RRM requirements for the FDD PCell, following requirments from Rel-19 can be reused.

- PCell L3 measurement

- BFD/CBD for PCell

- L1-RSRP/SINR for PCell

- RLM for PCell

- TCI state switch delay

- PL-RS switch delay for PCell

- From RAN4 perspective, the following would need to be accounted for by RAN1 for completing the LBCA via switching feature with switching between Case 1 and Case 3:

- A ‘1’ in the switching pattern bitmap configured to the UE indicates that the UE is receiving on both FDD DL and SDL carriers instead of only on the SDL carriers.

- Collision handling rules

- N1 represents the switching gap from FDD to Dual DL reception

- N2 represents the switching gap from Dual DL reception to FDD

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

- Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1

- RAN4 to discuss how to involve RAN1 in the work to complete the feature

# References

[1]		RP-252953: New WID on low band CA via switching

# Switching Gap Mechanism (Rel-19)

Figure 3: Switching gap mechanism in Rel-19

The RAN1-agreed switching gap mechanism in Rel-19 is shown in Figure 1. The figure illustrates the switching location and the switching gap length, which is configurable by the NW. The configuration allows the following values for the N1 and N2:

- N1: Configurable between 1 to 3 symbols

- N2: Configurable between 1 to 30 symbols