3GPP TSG-RAN4 Meeting #117	R4-2522155

# Dallas, Texas, USA 17th – 21th November, 2025

Source: 	Qualcomm Incorporated

Title: 	Inter-vendor training collaboration and Interoperability of CSF Compression

Agenda item:	7.9.3

Document for:	Discussion

# Introduction

In this paper, we present our views regarding the channel model to design for Direction C and define performance requirement.

# Past Agreements

## System Parameters

The system parameter assumptions of 38.843 were baseline for RAN4 performance evaluation during the study item [1] [2]

Table 1: Baseline SLS assumptions for AI/ML based CSI feedback enhancement evaluations

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g., UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 2GHz as baseline, optional for 4GHz (if R16 as baseline) FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz (if R17 as baseline) |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz (if R16 as baseline) 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz (if R17 as baseline) |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU. |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g., 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| Feedback assumption |  | Realistic |

# Discussion and Results

## Channel Selection For Normative Work

RAN plenary has assigned the following tasks for RAN4 in Rel-20’s two-sided CSF compression work item [3]

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

RAN4 made the following agreement during the last meeting [4]:

Agreement Dataset for model evaluation RAN4 will discuss how to introduce the new mixed dataset for model structure evaluation purpose at the beginning of this WI, by taking into consideration of the scalability and/or various configurations. - Details are FFS including the channel model(s).

The ‘system parameters’ section shows that RAN4 used a system level channel to generate SGCS results during the Rel-19 study item of CSF compression. Simulation results of Appendix A suggest that the distribution of eigen vectors, generated from system level channel, of some companies were not aligned with other companies. This lack of calibration led to poor SGCS results when encoder and decoder were trained with misaligned datasets. Besides, system level channel has not been calibrated by RAN4 to define demodulation performance requirements, including PMI related performance requirements.

Until Rel-18, RAN4 has only calibrated TDL based channel models for demod testing. RAN4 had a work item to calibrate CDL channel models for demod testing in Rel-19. At the end of that work item, RAN4 decided to specify PDSCH demod performance requirements with CDL channel model. However, RAN4 decided to further study if performance alignment can be achieved for PMI testing with CDL channel models. This has been captured through following texts in the WID of Rel-20 WI.

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 - Using rCDL-C1 channel model detailed in TR 38.753 as starting point - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case. - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding. - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias

Performance requirements of two-sided CSF compression are similar to PMI tests. Hence, it can be concluded that CDL channels cannot yet be used for AI-ML based CSF compression tests in Rel-20.

Observation 1: RAN4 has not calibrated system level channel for PMI testing.

Observation 2: RAN4 selected system level channel to investigate two-sided CSF during the study item. Lack of calibration of system level dataset led to poor SGCS results in some scenarios where encoder and decoder were trained with different companies’ datasets.

Observation 3: RAN4 has calibrated rCDL channel models for PDSCH demodulation performance requirements. RAN4 is still studying if performance alignment in PMI reporting can be achieved with rCDL channels.

Observation 4: RAN4 has calibrated TDL-A, TDL-B and TDL-C channels to define legacy demod requirements.

The actual performance test of AI-ML based two-sided CSF compression can only happen in a link level TDL channel to avoid miscalibration issue.

Typically, RAN4 would select the same calibrated dataset for training and testing the encoder-decoder, along with defining performance requirement. However, one should investigate whether TDL based training dataset suffers from overfitting, i.e., if an encoder decoder trained purely on mixture of TDL datasets (e.g. combination of TDL-A, B and C datasets with different delay spreads) perform poorly on other datasets (e.g., system level datasets). Based on that investigation, RAN4 can decide whether to use mixture of TDL or combination of system level dataset plus mixture of TDL as training dataset to design the reference encoder and decoder.

Observation 5: The dataset used during performance tests should be included in the training dataset to design Dir C and Direction A, sub-option 3a-1.

Proposal 1: RAN4 selects TDL channel to define the performance requirements of two-sided CSI feedback mechanism of Rel-20.

Proposal 2: RAN4 discusses which of following channels to consider in the training data set to design the fully defined/specified reference model of “Direction C”.

- Option 1: Mixture of TDL (e.g., combination of TDL-A, B and C with different delay spreads) based training dataset

- Option 2: Generalized training dataset containing system level channel’s datasets plus mixture of TDL datasets.

## Performance Metric Selection for Normative Work

Different metrics can be considered to design the reference encoder/decoder of Dir C and define performance requirements. There metrics can be the following:

- SGCS

- Normalized MSE (NMSE)

- Ratio of throughput (The same metric as for the current PMI reporting requirements in 38.101-4)

RAN4 has used SGCS as the defining metric to derive the reference encoder and decoders during Rel-19. RAN4 can use reuse SGCS. Hence, RAN4 can reuse the steps that it went through during SI to align results across companies if SGCS is used as the performance metric to design the reference encoder/decoder.

RAN4 has always used the ratio of throughput as the metric to define performance requirements of PMI reporting. Although, SGCS can hint potential performance of an encoder-decoder pair, the real performance can only be observed through throughput study. Besides, SGCS based investigation does not allow us to observe the impact of practical issues like channel estimation error in performance.

Observation 6: RAN4 can reuse the steps that it went through during SI to align results across companies if SGCS is used as the performance metric to design the reference encoder/decoder of Dir C and Dir 3a-1.

Observation 7: RAN4 has always used ratio of throughput as the metric to define performance requirements of PMI reporting.

Observation 8: SGCS based investigation does not allow us to observe the impact of practical issues like channel estimation error in performance.

Proposal 3: RAN4 uses SGCS to design the reference encoder/decoder models of Dir C and Dir 3a-1.

Proposal 4: RAN4 uses the ratio of throughput (same metric as the current PMI reporting requirements in 38.101-4) to define the performance requirement of the standardized test decoder.

# Conclusions

Observation 1: RAN4 has not calibrated system level channel for PMI testing.

Observation 2: RAN4 selected system level channel to investigate two-sided CSF during the study item. Lack of calibration of system level dataset led to poor SGCS results in some scenarios where encoder and decoder were trained with different companies’ datasets.

Observation 3: RAN4 has calibrated rCDL channel models for PDSCH demodulation performance requirements. RAN4 is still studying if performance alignment in PMI reporting can be achieved with rCDL channels.

Observation 4: RAN4 has calibrated TDL-A, TDL-B and TDL-C channels to define legacy demod requirements.

Observation 5: The dataset used during performance tests should be included in the training dataset to design Dir C and Direction A, sub-option 3a-1.

Observation 6: RAN4 can reuse the steps that it went through during SI to align results across companies if SGCS is used as the performance metric to design the reference encoder/decoder of Dir C and Dir 3a-1.

Observation 7: RAN4 has always used ratio of throughput as the metric to define performance requirements of PMI reporting.

Observation 8: SGCS based investigation does not allow us to observe the impact of practical issues like channel estimation error in performance.

Proposal 1: RAN4 selects TDL channel to define the performance requirements of two-sided CSI feedback mechanism of Rel-20.

Proposal 2: RAN4 discusses which of following channels to consider in the training data set to design the fully defined/specified reference model of “Direction C”.

- Option 1: Mixture of TDL (e.g., combination of TDL-A, B and C with different delay spreads) based training dataset

- Option 2: Generalized training dataset containing system level channel’s datasets plus mixture of TDL datasets.

Proposal 3: RAN4 uses SGCS to design the reference encoder/decoder models of Dir C and Dir 3a-1.

Proposal 4: RAN4 uses the ratio of throughput (same metric as the current PMI reporting requirements in 38.101-4) to define the performance requirement of the standardized test decoder.

# References

[1] NR_AIML_air-email discussions after RAN4#111 meeting, https://list.etsi.org/scripts/wa.exe?A2=3GPP_TSG_RAN_WG4;aa5369d9.2406C&S=

[2] R4-2414447, “Alignment of model training and data set parameters for AI/ML based CSI compression”, Nokia,  3GPP TSG-RAN4 Meeting #112.

[3] RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, 3GPP TSG RAN Meeting #108.

[4] R4-2514614, WF on R20 NR_AIML_air_Ph2, 3GPP TSG-RAN WG4 #116bis.

[5] RP-252957, WID: NR demodulation performance: Phase 6, 3GPP TSG RAN Meeting #109.

# Appendix A: Simulation Results with Mismatched Dataset in Option 4

These results have been re-submitted from our contributions during the study item. The following steps were performed to evaluate the feasibility of option 4:

- Take Samsung reference encoder and mixed dataset. Use these to “label” the (encoder input, encoder output) training dataset. Train a new “own” nominal decoder using the labelled dataset. Then train an “own” encoder with the newly trained decoder

- Test “own” encoder with the nominal decoder of all companies, including own “nominal” decoder

  - Note: Nominal decoders of all companies are generated based on Samsung’s reference encoder

Table 2 shows that the SGCS obtained with all other companies’ nominal decoders are quite aligned and they are very similar to the SGCS obtained with Qualcomm’s own encoder and own decoder/

Table 2: Simulation results with Option 4 reference encoder with mixed dataset (resubmission from last meeting)

| Option 4 Table shows result of ‘own encoder’ with ‘own decoder’ and other companies’ nominal decoders (all developed based on Samsung’s reference encoder) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Dataset1=Dataset2=Dataset3=mixed |  |  |  |  |  |
|  |  | Qualcomm | MediaTek | VIVO | Nokia | Huawei, Hisilicon | Ericsson |
| RAN4 Agreed encoder, CNN | Qualcomm | 0.6972 | 0.6832 | 0.6894 | 0.6826 | 0.701 | 0.6909 |

The following simulations were performed to evaluate the importance of dataset standardization in option 4b:

- Take Samsung reference encoder and mixed dataset. Use these to “label” the (encoder input, encoder output) training dataset. Train a new “own” nominal decoder using the labelled dataset. Then train an “own” encoder with the newly trained decoder

- Test “own” encoder with own “nominal” decoder

Table 3: Simulation results with Option 4 reference encoder with different datasets (new submissions from last meeting)

| Option 4 Table shows result of ‘own encoder’ with ‘own nominal decoder’ with different datasets (Reference encoder comes from Samsung) |  |  |
| --- | --- | --- |
|  | Qualcomm “nominal” decoder Dataset1= Dataset4 = mixed Dataset2=Dataset3=own dataset | Qualcomm “nominal” decoder Dataset1 = Dataset2 = Dataset3 = mixed Dataset4 = own dataset |
| RAN4 Agreed encoder, CNN Qualcomm “own” encoder | 0.4713 | 0.4577 |

Table 3 shows that the performance of option 4b can significantly drop if a different dataset is used during training the nominal decoder or during the tests.