3GPP TSG-RAN WG4 Meeting #118	R4-2600032

Gothenburg, Sweden, 09 Feb – 13 Feb, 2026

Source:	China Telecom

Title:	Discussion on UE requirements for FR1 SU-MIMO with spatial channel model

Agenda Item:	7.5.3

Document for:	Discussion

Introduction

In RAN #109 meeting, the new WID on NR demodulation performance: Phase6 was approved [1], with the following objectives for the normative work for spatial channel model (SCM):

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 - Using rCDL-C1 channel model detailed in TR 38.753 as starting point - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case. - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding. - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

In the RAN4#117 meeting, companies discussed the test scopes, parameters and simulation assumptions and the latest status are captured in [2].

In this contribution, our views on UE requirements for FR1 SU-MIMO with spatial channel model is provided.

Discussion

According to the WID, the normative work includes mainly 3 parts: update of rCDL channel model considering the outcome in SI FS_NR_7_24GHz_Chmod (if necessary), test scope and test parameter selection, test applicability.

rCDL-C2 channel model

In the last meeting, the updated rCDL-C2 channel model deriving method has been agreed:

#### Cluster simplification and Delay scaling after cluster truncation Agreement: Use 12 clusters, choosing the 12 strongest clusters; the process for cluster selection should include sub-cluster combining, delay scaling, power normalisation, i.e. removing spatial filtering (AAV+AEP) and RMS delay spread rescaling from the 38.753 process (step 2, 6 and 7). Omit the post-truncation delay scaling step, instead having one initial RMS delay scaling, then choosing the 12 strongest clusters. #### Angle scaling for rCDL – updated rCDL channel model Way Forward Companies to update table based off the agreements at RAN4#117, applicable ‘s’ values in 38.901 may be reused, if AS used are not captured in 38.901 ‘s’ values should be derived and captured in the specification.

Therefore, it is proposed to finalize the rCDL-C2 parameter table(s) for requirement definition.

In addition, RAN4 has also agreed the baseline UE antenna modeling and frequency generalization, it is the time to start the simulation result alignment work from the next meeting.

Proposal 1: In this meeting, finalize the rCDL-C2 parameter table(s) for requirement definition. And start the simulation result alignment work from the next meeting

Other maximum doppler values in addition to X=10Hz

Agreement: Regarding maximum doppler, focus on X=10 as baseline, other values can be considered in future meetings only if: 1) consensus can be reached 2) alignment on the simulation results can be reached

In addition to the maximum Doppler 10Hz as will be used for this WI, we are supportive to consider other Doppler values to cover more practical scenarios as we usually use TDLA30-10, TDLB100-400 and TDLC300-100. But considering the progress, we recommend to deprioritize this part when the SCM requirements with maximum Doppler 10Hz are stable, or consider it further in the 6G study for RAN4 Demod.

Proposal 2: Study other Doppler values to cover more practical scenarios when the SCM requirements with maximum Doppler 10Hz are stable, or consider it further in the 6G study for RAN4 Demod.

Test scope and test parameters

Antenna Array Virtualization (AAV) assumptions

- Option 1: Only consider Ms=Ns=1 - RAN4 should further study practical method on subarray virtualization for SCM in 6GR

In the Rel-19 study, both BS AAV (8,1) and AAV (1,1) are capture in the TR38.753. However, the simulation is only performed only based on AAV (1,1).

We propose to only consider AAV (1,1) for this WI and we are supportive to further study practical method on subarray virtualization for SCM in 6GR.

Proposal 3: Only consider AAV (1,1) for this WI. RAN4 should further study practical method on subarray virtualization for SCM in 6GR.

Other test parameters for PDSCH demodulation requirements with SCM

- Proposal 1: Reuse the test parameters for the Rel-19 SI, i.e., Table 6.1-1 and Table 6.1-2 in TR38.753. - Proposal 2: Reuse configuration from [R4-2508622], including rCDL with 10 Hz Doppler (in its AS updated “C2” variant)

For the other parameters for PDSCH demodulation requirements, it is proposed to reuse the test parameters for the Rel-19 SI, i.e., Table 6.1-1 and Table 6.1-2 in TR38.753.

Proposal 4: Reuse the test parameters for the Rel-19 SI, i.e., Table 6.1-1 and Table 6.1-2 in TR38.753.

PMI bias

Agreement: RAN4 shall investigate the PMI bias for PMI tests, options could be but are not limited to: - Beam Steering - Base Station Bearing Angle Rotation - UE Azimuth Rotation - Mean Cluster Angle Rotation - RAN1 spatially consistent time evolution

We believe the existing beam steering approach is a important method to ensure the reported PMI can dynamically catch up with the latest channel condition. In addition, it can verify the UE can report the PMIs covering all directions.

The existing simple beam steering approach has been widely used for RAN4 demodulation requirement definition without testability issue. Therefore, we support the idea to firstly study the feasibility to apply the existing simple beam steering approach specified in TS 38.101-4 B.2.3.2.3 for PMI testing with SCM as a start point.

Proposal 5: Firstly, study the feasibility to apply the existing simple beam steering approach specified in TS 38.101-4 B.2.3.2.3 for PMI testing with SCM as a start point.

Test scope for eType II PMI reporting (if introduced)

- Option 1: Define PMI reporting performance requirements with 4T4R and 8T4R - Option 2: RAN4 not to consider 4TX-4RX PMI test. - During Rel-19 SCM SI, no PMI cases in 4TX-4RX setup were considered.

It is clearly captured in the WID that RAN4 will ‘specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding’, if/when performance alignment on PMI reporting is achieved. Therefore, 4T4R should be included in the scope at least in the initial study for simulation result alignment.

Proposal 6: Cover both 4T4R and 8T4R in the scope at least in the initial study for eType II PMI reporting simulation result alignment.

Rank for eType II PMI reporting (if introduced)

- Option 1: Only consider rank 2 - Option 2: Consider both rank 2 and rank 4 - Option 2a: 8T4R for rank 4 and 4T4R for rank 2 - Option 2b: Prioritize rank 4

We support to consider higher rank for PMI reporting tests. To try to reduce the case number, one possible way is to consider different rank with different antenna configuration, e.g., 8T4R for rank 4 and 4T4R for rank 2.

Proposal 7: Consider different rank with different antenna configuration, e.g., 8T4R for rank 4 and 4T4R for rank 2.

Test metric for eType II PMI reporting

As for the test metric for eType II PMI reporting, we prefer to follow the same rule for the existing eType II requirement based on TDL, i.e., following PMI (eType II)/Random PMI (Type I codebook).

Proposal 8: Reuse the same test metric for the existing eType II requirement based on TDL, i.e., Thput of following PMI (eType II)/ Thput of Random PMI (Type I codebook) with SNR @ 90 % of the maximum throughput for following PMI.

Test parameters for eType II PMI reporting and PDSCH demodulation

As for the test parameters, we propose to reuse the simulation assumptions for the Rel-19 SI as much as possible to avoid duplicate simulation workload, i.e., Table 6.1-1 and Table 6.1-2 for PDSCH requirements and Table 6.2-1 and Table 6.2-2 for PMI reporting as captured in TR38.753.

Proposal 9: Reuse the simulation assumptions/test parameters for the Rel-19 SI, i.e., Table 6.1-1 and Table 6.1-2 for PDSCH requirements and Table 6.2-1 and Table 6.2-2 for PMI reporting as captured in TR38.753.

Test applicability

It is clearly captured in the WID that all SCM new requirements are optional. Therefore, RAN4 needs to introduce new declaration for the SCM testing and UE can choose whether to pass the tests based on declaration.

It remains unclear that if RAN4 will define one single declaration on SCM requirements, or define multiple declarations for each individual test cases.

In addition, RAN4 is now defining requirements for the existing UE features, e.g., 4T4R rank4 requirement applies to Rel-15 UEs and 8T8R rank8 requirement applies to Rel-17 8Rx UEs supporting max 8 layers. Therefore, RAN4 needs to discuss whether to define release independent rules for the new SCM requirements.

Proposal 10: Introduce new declaration for the SCM testing and UE can choose whether to pass the tests based on declaration. RAN4 needs to discuss whether to define one single declaration on SCM requirements, or define multiple declarations for each individual SCM test cases.

Proposal 11: RAN4 needs to discuss whether to define release independent rules for the new SCM requirements.

Conclusion

Proposal 1: In this meeting, finalize the rCDL-C2 parameter table(s) for requirement definition. And start the simulation result alignment work from the next meeting

Proposal 2: Study other Doppler values to cover more practical scenarios when the SCM requirements with maximum Doppler 10Hz are stable, or consider it further in the 6G study for RAN4 Demod.

Proposal 3: Only consider AAV (1,1) for this WI. RAN4 should further study practical method on subarray virtualization for SCM in 6GR.

Proposal 4: Reuse the test parameters for the Rel-19 SI, i.e., Table 6.1-1 and Table 6.1-2 in TR38.753.

Proposal 5: Firstly, study the feasibility to apply the existing simple beam steering approach specified in TS 38.101-4 B.2.3.2.3 for PMI testing with SCM as a start point.

Proposal 6: Cover both 4T4R and 8T4R in the scope at least in the initial study for eType II PMI reporting simulation result alignment.

Proposal 7: Consider different rank with different antenna configuration, e.g., 8T4R for rank 4 and 4T4R for rank 2.

Proposal 8: Reuse the same test metric for the existing eType II requirement based on TDL, i.e., Thput of following PMI (eType II)/ Thput of Random PMI (Type I codebook) with SNR @ 90 % of the maximum throughput for following PMI.

Proposal 9: Reuse the simulation assumptions/test parameters for the Rel-19 SI, i.e., Table 6.1-1 and Table 6.1-2 for PDSCH requirements and Table 6.2-1 and Table 6.2-2 for PMI reporting as captured in TR38.753.

Proposal 10: Introduce new declaration for the SCM testing and UE can choose whether to pass the tests based on declaration. RAN4 needs to discuss whether to define one single declaration on SCM requirements, or define multiple declarations for each individual SCM test cases.

Proposal 11: RAN4 needs to discuss whether to define release independent rules for the new SCM requirements.

Reference

RP-252957, WID: NR demodulation performance: Phase 6, Apple, RAN#109, Sep 2025

R4-2522662, Way Forward for [117][231] NR_demod_Ph6, Nokia, China Telecom, RAN4#117, Nov 2025