3GPP TSG-RAN WG4 Meeting #118	R4-2600035

Gothenburg, Sweden, 09 Feb – 13 Feb, 2026

Source:	China Telecom

Title:	Discussion on release independent issue for 6Rx UE requirements

Agenda Item:	4.6.1.3

Document for:	Discussion

Introduction

An LS is sent to RAN2 to check the feasibility to early implement the max 6 MIMO layer signaling from Rel-18. The following feedback is given by RAN2 [1]:

RAN2 would like to thank RAN4 for the LS on release independence for 6Rx UE requirements with max 6 DL MIMO layers and max 4 DL MIMO layers. On the RAN4 question “whether UE capability signalling to allow indication of the support of maximum 6 DL MIMO layers for PDSCH as in LS R4-2505225 will be specified in Rel-19”, RAN2 would like to inform RAN4 that the following RAN2 CRs were approved in RAN#109 (See RP-252808). TS; CR; Rev; Rel; Title; Cat. 38.331; 5476; -; Rel-19; Introduction of UE capability on 6 DL MIMO layers; B. 38.306; 1350; -; Rel-19; Introduction of UE capability on 6 DL MIMO layers; B. RAN4 also asked “If Yes, can this signalling be implemented early in Rel-18?”. RAN2 would like to inform RAN4 that RAN2 had implemented a means to allow early implementation of features in the specifications. Please see the explanation below, using one of the examples in the past. - In R2-2200118, RAN4 requested "RAN4 has agreed to define these new FR2 CA FBG2 BW classes R, S, T, U as release independent from REL-15 pending confirmation from RAN2 that this is possible". This was a Rel-17 enhancement which was worked on under NR_RF_FR2_req_enh2-Core. - RAN2 agreed the Rel-17 CR to TS38.331 in R2-2313261. In the Annex C, the signaling change in this CR is stated to be early-implementable from Rel-15. Some other examples include the signalling of BCS4/5 and NS value extension, as can also be seen in the Annex C of TS38.331. So to answer the RAN4 question, it is possible to specify the early implementation of the UE capability signalling on the support of maximum 6 DL MIMO layers for PDSCH. RAN2 assumes that it is up to RAN4 to decide whether the feature should be made early implementable in the standard or not.

During the discussion in the last meeting, the following guidance is given from the RAN4 Chair:

RAN4 Chair Guidance: the non-spectrum features which have spec impact including signaling impact should not be considered as release independent and should not be incuded in 38.307 (5G;NR;Requirements on User Equipments (UEs) supporting a release-independent frequency band (3GPP TS 38.307 version 19.2.0 Release 19) - Case 1: 6Rx with up to 4 MIMO layers for handheld/FWA UE from Rel-18 - There is no signaling impact - New requirements based on 6Rx need to be introduced - Case 3: 6Rx with up to 6 MIMO layer for FWA UE only from Rel-19 - There is signaling impact - New requirements based on 6Rx need to be introduced.

In this contribution, our views on release independent issue for 6Rx UE requirements are provided.

Discussion

For 6Rx with up to 4 MIMO layers for handheld UE (Case 1), it has been confirmed that there is no new signaling impact, so we propose that the handheld 6Rx UE requirements with up to 4 layers to be release independent from Rel-18.

For 6Rx FWA UE requirements, it is worth noted that RAN4 has already introduced 8Rx FWA requirements release independent from Rel-17.

In addition, according to the RAN2 feedback in [1], RAN2 has confirmed the feasibility to specify the early implementation of the UE capability signaling, and ‘it is up to RAN4 to decide whether the feature should be made early implementable in the standard or not’.

Therefore, we think the 6Rx FWA UE requirements should be release independent from Rel-18, and we also request RAN4 to discuss the feasibility to make the 6Rx FWA UE requirements release independent from Rel-17.

Proposal 1: The handheld 6Rx UE requirements with up to 4 layers to be release independent from Rel-18.

Proposal 2: The FWA 6Rx UE requirements to be release independent from Rel-18 regardless of the supported max MIMO layers.

Proposal 3: RAN4 to discuss the feasibility to make the 6Rx FWA UE requirements release independent from Rel-17.

Conclusion

Proposal 1: The handheld 6Rx UE requirements with up to 4 layers to be release independent from Rel-18.

Proposal 2: The FWA 6Rx UE requirements to be release independent from Rel-18 regardless of the supported max MIMO layers.

Proposal 3: RAN4 to discuss the feasibility to make the 6Rx FWA UE requirements release independent from Rel-17.

Reference

R4-2520009, Reply LS on Release Independence of 6Rx, Qualcomm, RAN4#117